| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_NBR-ADV message format for IEEE 802.16m (section 16.2.3.12) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1856 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #30b |  |
| Abstract | Text proposal for 16.2.3.12 AAI_NBR-ADV section |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_NBR-ADV message format for IEEE 802.16m (section 16.2.3.12)

Inuk Jung, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to provide message format of AAI_NBR-ADV message.

- References

[1] IEEE P802.16m/D3, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

On the 80216m_D2, page 64, line 44, add current text in 16.2.3.12 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

15.2.3.12 AAI_NBR-ADV Message

Table 684—AAI_NBR-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| Number of cell types | 3 | — |
| Cell type in this message | 3 |  |
| Total Number of Segments | TBD | Number of segments for this cell type |
| Segment Index | TBD | Segment index of this message in the specific cell type |
| BS number M | TBD | Total number of BSs to be included in this segment |
| Starting Index of BS | 8 | This indicates the index offset from the last BS of the previous fragment. If this is the first fragment, the Starting Index will be 0. Hence, each fragment has one Index which corresponds to the first BS in that fragment. |
| Multi-carrier configuration change count | 4 | Refer to the change count in AAI_MC-ADV |
| Greenfield indicator | 1 | Indicates whether one or more WirelessMAN-OFDMA Reference systems are present in the vicinity of the serving ABS |
| For (i=0; i<M; i++) { | — | — |
| Index of BS | 8 | The index of the BS in the whole AAI_NBR-ADV |
| BSID | 48 | — |
| Center Frequency | TBD | The frequency raster of the center frequency of the left most carrier (i.e. lowest frequency located). |
| Multicarrier configuration mode (MCM) | 2 | 0b00: Single carrier mode 0b01: One Carrier group 0b10: Multiple Carrier groups 0b11: Reserved *Note: If 0b00, NCG and NC is implicitly set to 1. If 0b01, NCG is implicitly set to 1. |
| If (MCM ==0b10) { |  |  |
| Number of Carrier Groups (NCG) | 6 | Number of carrier groups of the ABS |
| } |  |  |
| If (MCM ==01 ¦¦ MCM ==10) { |  |  |
| Number of carriers (NC) | 6 | Number of carriers of the ABS |
| } |  |  |
| For ( j=0; j<NCG; j++) { | — | — |
| For ( j=0; j<NC; j++) { |  |  |
| SA-PREAMBLE index | TBD | — |
| Carrier Bandwidth | 3 | Bandwidth size of the carrier 0b000: 5MHz 0b001: 10MHz 0b010:20MHz 0b011: 7MHz 0b100: 8.75MHz 0b101~0b111: Reserved |
| SA-Preamble power de-boosting level | 6 | Values indicate power back-off levels in 0.1 dB steps starting from 0 dB |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| A-PREAMBLE transmit power | 8 | — |
| Physical carrier index | 6 | Refer to the change count in AAI_MC-ADV |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| for (i=0; i<P; i++) { | — | — |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| } | — | — |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| } | — | — |
| SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) 0b10: no SFH included 0b11: reserved For macrocell ABS, the bitmap shall be either 0b00 or 0b01 |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 1111 |
| If( SFH_encoding_format =00) { | — | //encoding format type-1 |
| If( Control_bitmap[0] ==1 ){ | — | — |
| SFH Subpkt 1 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| If( Control_bitmap[1] ==1 ){ | — | — |
| SFH Subpkt 2 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| If( Control_bitmap[2] ==1 ){ | — | — |
| SFH Subpkt 3 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| } | — | — |
| If( SFH_encoding_format =01) { | — | — |
| Delta information | — | Delta encoding, w.r.t. the reference BS |
| } | — | — |
| } | — | — |
| } | — | — |
| } |  |  |
| } | — | — |

--------------------------------------------------------Text End -----------------------------------------------------