| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal on Zone Switch for IEEE 802.16m (section 16.2.6.4.1.2.1) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1856 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #30b |  |
| Abstract | Text proposal for HO section 16.2.6.4.1.2.1 |  |
| Purpose | For member’s review and adoption into P802.16m_D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal on Zone Switch for IEEE 802.16m (section 16.2.6.4.1.2.1)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to further clarify Zone Switch from LZone to MZone.

- Rationale/Motivation of modifications

When performing Zone Switch, the AMS needs to first acquire full SFH information of MZone while operating in LZone. However, AMS cannot inform the BS whether it has finished receiving the SFH. Hence, an action time for Zone Switch is very hard to provide. We prefer to define a ‘Zone Switch deadline’, during which AMS shall zone switch to MZone. The ‘Zone Switch deadline’ considers the time for SFH reception.

However, data exchange in LZone does not depend on the ‘ZS deadline’. The data path of LZone finishes at the last network entry procedure in MZone.

In case of AMS fails to perform zone switch until deadline expiration, the AMS may request Zone Switch initiation via RNG-REQ message with ‘Ranging purpose indicator’ set to ‘Zone Switch’.

Fig. 1. Zone Switch signaling procedure

- References

[1] IEEE P802.16m/D3, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy #1

On the 80216m_D3 page 171, line 18, add current text in 16.2.6.4.1.2.1 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.6.4.1.2.1 Zone Switch based Handover Procedure

…

The Zone Switch Deadline conveyed in the Zone Switch TLV within the RNG-REQ message is the time allow for AMS to perform Zone Switch from LZone to MZone. In case Zone Switch Mode is set to 0, the Zone Switch deadline implies the time when BS stops all resource allocation for the AMS in the LZone. Otherwise, BS stops all resource allocation for the AMS after completion of network re-entry in MZone.

In case AMS fails to Zone Switch from LZone to MZone within the Zone Switch Deadline, the AMS shall may request the ABS to re-initiate Zone Switch by transmitting the RNG-REQ message with Ranging Purpose Indicator set to Zone Switch in the LZone.

…

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #2

On the 80216m_D3 page 172, line 49, add current text in 16.2.6.4.1.2.1 with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.6.4.1.2.1 Zone Switch based Handover Procedure

…

The Zone Switch parameters shall include the followings and are coded as TLV tuples as defined in 16.10.2.1:

- MZone A-Preamble index.

- Time offset between LZone and MZone

- Action Time Zone Switch Deadline: Action time The deadline of zone switch from LZone to MZone. AMS performs zone switch at Action Time before expiration of the deadline. If HO_Reentry_Mode=0, ABS stops all resource allocation for the AMS at LZone at the deadline.

- Zone Switch Mode: If set to 1, the AMS maintains its data communication with the ABS in LZone while performing network reentry in MZone; otherwise it breaks data communication in LZone at the Zone Switch deadline before performing network reentry in MZone.

- NONCE_ABS: It is used to derive a new PMK to be used in Mzone.

…

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #3

On the 80216m_D3 page 645, line 5, add current text in 16.10.2.1with the proposed text as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.10.2 Support for zone switch operation

16.10.2.1 RNG-RSP management message encodings

The encodings in Table 946 are specific to the RNG-RSP message sent in LZone of an ABS supporting WirelessMAN-OFDMA Reference System.

Table 946—OFDMA-specific RNG-RSP message encodings

| Name | Type (1byte) | Length | Value |
| --- | --- | --- | --- |
| MZone A-Preamble index | 41 | 1 |  |
| Time offset | 42 | 1 | Time offset |
| Action Time Deadline | 43 | 1 | Action time Deadline of zone switch from LZone to MZone. AMS performs zone switch at Action Time before expiration of the deadline. If HO_Reentry_Mode=0, ABS stops all resource allocation for the AMS at LZone at the deadline. |
| Zone Switch Mode | 44 | 1 | 0x01: AMS maintains its data communication with the ABS in LZone while performing network reentry in MZone; 0x00: AMS breaks data communication in LZone before performing network reentry in MZone. |
| NONCE_BS | 45 | 4 | For AMSID* derivation. PMK is shared between LZone and MZone during zone switch. |
| Temporary STID | 46 | 1 | Temporary STID for being used in MZone |
| Ranging initiation deadline | 47 | 1 | Valid time for Temporary STID. Shall be included if Temporary STID is included. |

…

--------------------------------------------------------Text End -----------------------------------------------------