| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for Long PDUs in 802.16m |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Aran Bergman, Muthaiah Venkatachalam Joey Chou Intel | E-mail: aran.bergman@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for Support for Long PDUs in 802.16m. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support for Long PDUs in 802.16m

Aran Bergman, Muthaiah Venkatachalam, Joey Chou

Intel

# Introduction

Limiting the PDU size to 2047 bytes (11 bits) in the AGMH will increase the number of SDU fragments in a burst. For a 14350-byte burst the number of 2047 byte PDUs is about 7. Assuming the data contains 1400- byte SDUs, all from the same connection, each PDU will most likely contain 3 SDU / SDU fragments, for a total of 21 SDU fragments per burst. If the whole burst would contain a single, unlimited, PDU, the number of SDU fragments would not be greater than 12 (or, more accurately, 10 SDUs and a maximum of 2 SDU fragments).

To reduce the required number of processing in the receiver there is a need to increase the size of the PDU length in the AGMH and the PDU Payload length in MEH, to prevent unnecessary SDU fragmentation.

# Proposed Text

[--------------------------------------------Start of Text Proposal---------------------------------------------------]

[Change 1 - Modify table 653 in subclause 15.2.2.1.1, page 18, as follows]

Table 653 – AGMH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Advanced Generic MAC Header() { |  |  |
| Flow ID | 4 |  |
| EH | 1 | Extended header presence indicator; When set to ‘1’, this field indicates that an Extended Header is present following this GMH. |
| Reserved | 2 | Shall be set to zero |
| Flag | 1 | Indicates whether the Length field is 1 or 2 bytes long. Flag = 0 means a short Length field and Flag = 1 means a long Length field |
| if (Flag == 0) { |  |  |
| Length | 118 | This field indicates the length in bytes of MAC PDU including the AGMH and extended headers if present. |
| } else { |  |  |
| Length | 16 | This field indicates the length in bytes of MAC PDU including the AGMH and extended headers if present. |
| } |  |  |
| } |  |  |

[Change 2 - Modify table 661 in subclause 15.2.2.2.3, page 23, as follows]

Table 661 MEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEH () { |  |  |
| Last | 1 | 0 = another extended header follows MEH 1 = another extended header does not follow MEH |
| Type | TBD | MEH type |
| Num Flows | 4 | Number of Flow information present in the MEH. If ‘n’ connections are multiplexed, ‘n-1’ Flow IDs and Lengths are present. Num Flows = ‘n-1’. |
| for (i = 1; i <= num flows; i++) { |  |  |
| Flow ID | 4 | Flow Identifier. The ‘i’th Flow ID indicates the Flow ID of the ‘i+1’th connection. |
| Reserved | 3 | Shall be set to zero. |
| Flag | 1 | Indicates whether the following Length field is 1 or 2 bytes long. Flag = 0 means a short Length field and Flag = 1 means a long Length field |
| if (Flag == 0) { |  |  |
| Length | 118 | Length of the payload of the ‘i’th connection. The ‘i’th length field indicates the length of the payload of the ‘i’th connection. The length of the payload of the last connection is given by “MAC PDU payload length – sum of ‘n-1’ Length fields”. |
| } else { |  |  |
| Length | 16 | Length of the payload of the ‘i’th connection. The ‘i’th length field indicates the length of the payload of the ‘i’th connection. The length of the payload of the last connection is given by “MAC PDU payload length – sum of ‘n-1’ Length fields”. |
| } |  |  |
| Reserved | variable |  |
| } |  |  |

[------------------------------------------End of Text Proposal----------------------------------------------------]