| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Simplifications to the specifications of the DL Sub-band Assignment A-MAP IE – Proposed Amendment Text |  |
| Date Submitted | 2010-01-12 |  |
| Source(s) | Sudhir Ramakrishna, Youngbo Cho, Hyunkyu Yu, Taeyoung Kim, Ji-Yun Seol, Jeongho Park, Heewon Kang, Zhouyue Pi Samsung Electronics Roshni Srinivasan Intel Corporation | sudhir.r@samsung.com |
| Re: | “Contribution in support of a comment on LB30”: Category – AWD Comments Area – Chapter 16.3.6 (DL Control) |  |
| Abstract | This contribution proposes some modifications to Sections 16.3.6.5.2.4.3 (DL Sub-band Assignment A-MAP IE) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Simplifications to the specification of the DL Sub-band Assignment A-MAP IE – Proposed Amendment Text

Sudhir Ramakrishna, Youngbo Cho, Hyunkyu Yu, Taeyoung Kim, Ji-Yun Seol, Jeongho Park, Heewon Kang, Zhouyue Pi

Samsung Electronics

Roshni Srinivasan

Intel Corporation

- Introduction

The 1st 5 text modifications proposed in this contribution refer to Section 16.3.6.5.2.4.3 (“DL Sub-band Assignment A-MAP IE”). The text modifications are in the order of occurrence in that section. The last text modification refers to Section 16.3.6.5.2.4.4 (“UL Sub-band Assignment A-MAP IE”).

In addition to editorial fixes, the text has been re-organized/re-written as compared to [1], in that the number of sub-bands in the system, rather than the system bandwidth (as in [1]), determine how the RA field is to be interpreted. This makes for easier reading and understanding, and shortens the text.

Also, in cases where one has a larger number of sub-bands (> 5), the benefits of the 2-CRU (half-sub-band) granularity are outweighed by the additional implementation complexity, in terms of the interpretation of the RA field and the number of cases that will need to be tested for. For this reason, we propose to eliminate the 2-CRU granularity for cases with >5 sub-bands, and only have a 1-sub-band granularity. This greatly simplifies the implementation of the RA interpretation rules.

In this document, text to be deleted has been struck out and marked with red font. Text to be added is in blue underlined font. Existing text to be retained is in black font.

- Text Modifications 1

- Please modify the content on Lines 21 through 24 of Page 403 as follows: The DL Sub-band Assignment A-MAP IE shall have an identical structure to the DL Basic Assignment A-MAP IE defined in Section 15.3.6.5.2.2, with the following exceptions. With the exception of the “IE Type” and the “Resource Allocation” field, all of the other fields shall be interpreted in the same manner as defined for the DL Basic Assignment A-MAP IE. The MEF field shall be 1 bit long, with 0b0/0b1 indicating HE/VE. Following the MEF field shall be a 1-bit NumIEs field, which indicates the number of IEs carrying an instance of an allocation, with 0b0/0b1 indicating that one/two IEs carry an instance of an allocation.

- Please modify the sentence on Lines 31/32 of Page 403 as follows: The structure and interpretation of the RA field for the DL Sub-band Assignment A-MAP IE shall be as defined below, for the cases of 5, 10 & 20 MHz.

- Please modify the sentence on Lines 36/37 of Page 403 as follows: The notation and terms in these steps, related to sub-channelization, are defined in Section 1516.3.5.

- In formula 208 on Line 5 of Page 404, please replace Y with YSB.

- Please modify the sentences on Lines 14 and 17 of Page 404 as follows.

Nominal Channel Bandwidth = 5 MHz YSB <= 5

For a 5 MHz system When YSB <= 5:

- Please add to the sentence on Line 18 of Page 404 (1st item in list) as follows: A single instance of a resource allocation shall be made using a single IE. The ABS shall set NumIEs to 0b0.

- Please delete the 1st sentence (2nd item in list) on Line 20 of Page 404 as follows: It may be noted that YSB <= 4 for a 5 MHz system.

- Please modify the sentence on Lines 23/24 of Page 404 as follows: Each of the j bit positions in the RA field, 0 <= j < 11, indicates the allocation or non-allocation of 2 SLRUs within a particular sub-band, using a bit-map method.

- Please modify the sentence on Lines 38/39 of Page 404 as follows: Figure 515 pictorially illustrates the above description of the interpretation of the RA field in a 5 MHz system when YSB <= 5.

- Please modify Figure 515 on Page 404 as follows.

  - Please modify the caption of Figure 515 as follows: “Figure 515-Interpretation of the RA field in a 5 MHz system when YSB <= 5”

  - There is a sentence in the figure saying “YSB <= 4 in a 5 MHz system”. Please remove this sentence.

- Text Modifications 2

Please modify Lines 59 to 65 on Page 404 as follows. Please note that this modification includes the addition of a figure, denoted below as X1, which is also referred to in the text. Please replace X1, in all places it occurs, with the appropriate figure number after editing.

------------------------------------------------------ Begin Text Modifications 2 ---------------------------------

Nominal Channel Bandwidth = 10 MHz 6 <= YSB <= 11

For a 10 MHz system a single instance of a resource allocation shall be made using a single IE. It may be noted that YSB <= 10 for a 10 MHz system. For a particular value of YSB, the AMS shall interpret the bits in the Resource Allocation field as indicated by the row corresponding to that value of YSB in Table 814.

When 6 <= YSB <= 11, a single instance of a resource allocation shall be made using a single IE, and the ABS shall set NumIEs to 0b0. The AMS shall interpret the RA field in this case as defined below.

- Each of the j bit positions in the RA field, 0 <= j < 11, indicates the allocation or non-allocation of all 4 SLRUs within a particular sub-band, using a bit-map method.

- Position j in the RA field corresponds to the 4 SLRUs with indices k such that

- Denoting RA[j] as the jth position in the RA field, 0 <= j < 11, with j = 0 being the LSB

RA[j] = 1 => The 4 SLRUs with indices k such that  have been allocated.

RA[j] = 0 => The 4 SLRUs with indices k such that have not been allocated.

Figure X1 below pictorially illustrates the above description of the interpretation of the RA field when 6 <= YSB <= 11.

Figure X1 – Interpretation of the RA field when 6 <= YSB <= 11

------------------------------------------------------ End Text Modifications 2 ---------------------------------

- Text Modifications 3

- Please remove Table 814 on Pages 405 and 406.

- Please remove all the contents of Page 407. This includes the text (on Lines 3 & 4), and Figures 516 & 517 and their captions, all on Page 407.

- Please remove all the content from the 1st line till Line 59 on Page 408. This includes Figures 518 and 519, and their associated captions.

- Text Modifications 4

Please modify the content starting from Line 62 on Page 408, and ending on Line 22 on Page 409, as follows. Please note that this modification includes the addition of a figure, denoted below as X2, which is also referred to in the text. Please replace X2, in all places it occurs, with the appropriate figure number after editing.

------------------------------------------------------ Begin proposed modification 4 ---------------------------------

Nominal Channel Bandwidth = 20 MHz 12 <= YSB <= 21

For a 20 MHz system the following procedure shall be followed: When 12 <= YSB <= 21, a single instance of a resource allocation may be made using a single or two IEs.

- It may be noted that Y <= 21 for a 20 MHz system. When YSB (total number of sub-bands over all frequency partitions) <= 11, a single instance of a resource allocation shall be made using a single IE. In this case, the AMS shall interpret the bits in the Resource Allocation field as indicated by Error: Reference source not found (the row corresponding to the particular value of YSB in that table).

- When YSB (total number of sub-bands over all frequency partitions) > 11, a single instance of a resource allocation may be made using a single or two IEs.

- When an allocation is made using 2 IEs, the RA fields of the two IEs shall be concatenated to form a 22-bit field, referred to as the Concatenated-RA field (C-RA field). The LSB of the RA field of the IE occurring last in the A-AMAP region shall be interpreted as the LSB of the C-RA field. The AMS shall interpret the bits in the C-RA field as indicated by Error: Reference source not found (the row corresponding to the particular value of YSB in that table). The ABS shall map the two IEs carrying a single instance of an allocation to MLRUs with contiguous logical indices, and shall set NumIEs to 0b1 in each of them. The AMS may shall infer that two IEs refer to the same instance of a resource allocation from the values of the NumIEs, ACID and SPID fields. The AMS shall interpret the C-RA field in this case as defined below.

- Each of the j bit positions in the C-RA field, 0 <= j < 22, indicates the allocation or non-allocation of all 4 SLRUs within a particular sub-band, using a bit-map method.

- Position j in the C-RA field corresponds to the 4 SLRUs with indices k such that

  - Denoting C-RA[j] as the jth position in the C-RA field, 0 <= j < 22, with j = 0 being the LSB

C-RA[j] = 1 => The 4 SLRUs with indices k such that  have been allocated.

C-RA[j] = 0 => The 4 SLRUs with indices k such that have not been allocated.

Figure X2 below pictorially illustrates the above description of the interpretation of the C-RA field when 12 <= YSB <= 21, and 2 IEs are used to make an allocation.

Figure X2 – Interpretation of the C-RA field when 12 <= YSB <= 21 & 2 IEs are used to make an allocation

- When an allocation is made using a single IE, the ABS shall set NumIEs to 0b0, and the RA field shall be interpreted as described below in 15.3.6.5.2.4.1. This section description defines the mapping of the RA field to all possible non-contiguous combination of indices of 2 or 3 subbands.

------------------------------------------------------ End Text Modifications 4 ---------------------------------

- Text Modifications 5

- Please remove all the content on Page 409 starting from Line 23 on that page. This content consists of the sentence on Line 24 and Figure 520 (and its caption) on that page.

- Please remove all the content on Page 410, which consists of Table 815.

- Please remove all the content on Page 411. This includes the sentence on Line 1 of that page, and Figure 521 and its caption.

- Please remove the sentence on Line 1 of Page 412, as follows: Nominal Channel Bandwidth = 20 MHz, YSB>11, Allocation Using a Single IE

- Please modify the sentence on Line 3 of Page 412, as follows: When Y YSB (total number of sub-bands over all frequency partitions) > 11, a single IE may be used to make allocations of 2 or 3 sub-bands with arbitrary non-contiguous logical indices.

- On Lines 31 and 32 of Page 412, please insert the correct figure numbers in place of “FIGURE_X8” and “FIGURE_X9”. In D3, FIGURE_X8 = 522 and FIGURE_X9 = 523, but these may change.

- Please modify the caption of Figure 522 on Page 412, as follows: Definitions of sub-band pairs and pair groups in the 20 MHz, YSB > 11, single IE case

- Please modify the caption of Figure 523 on Page 413, as follows: Partitioning and usage of the RA field in the 20 MHz, YSB > 11, single IE case

- Please remove the sentence on Line 41 of Page 413, as follows: As indicated in the table, the MSB and 6 LSB bits of the RIF are interpreted separately.

- Please modify the sentences from Lines 46 through Line 56 on Page 413 as follows: As an example, consider the case where RA = 11000100001, i.e., RIF = 1100010 and ITF = 0001. From the ITF, this is a 2 sub-band allocation. As above, let the two allocated sub-bands be denoted as SB[x] and SB[y], with SB[x]<-->SBP[u] and SB[y]<-->SBP[v]. From Table 1, since the MSB of the RIF is 1, this implies that the two allocated sub-band pairs are in different pair groups. The 1st 3 bits of the 6 LSB bits of the RIF (= 100) indicate SBP[u] relative to PG[0]; hence, SBP[u] = 4. The last 3 bits of the 6 LSB bits of the RIF (= 010) indicate SBP[v] relative to PG[1]; hence, SBP[v] = 7. Hence, the allocation is in SBP[4] = {SB[8], SB[9]} and SBP[7] = {SB[14], SB[15]}. The ITF (= 0001) indicates that the allocated SBs occupy the higher indices in the indicated SBPs. Hence, the allocation is {SB[9], SB[15]}.  In the table, the decimal values of the 7-bit RIF increase from left to right and from top to bottom (the first and last values in each row of the table are shown). The shaded values are not used to make interpretations. Each particular cell in the table, corresponding to a particular decimal value of the RIF, indicates the values of the SBPs SBP[u] and SBP[v] that have been allocated.

As an example, consider the case where RA = 00001010001, i.e., RIF = 0000101 = decimal value 5, and ITF = 0001. From the ITF, this is a 2 sub-band allocation. As above, let the two allocated sub-bands be denoted as SB[x] and SB[y], with SB[x] SBP[u] and SB[y] SBP[v]. From Table 816, ITF = 5 corresponds to the allocation SBP[u] = SBP[0] = {SB[0], SB[1]}, SBP[v] = SBP[6] = {SB[12], SB[13]}. ITF = 0001 implies that the higher SB indices in both SBP[u] and SBP[v] are being indicated; hence, the allocation is SB[x] = SB[1] and SB[y] = SB[13].

- Please REPLACE Table 816 with the following table:

Table 816 - Interpretation of the Resource Indexing Field (7 MSB bits of RA field) in the 2 sub-band allocation case

| SBP[v] → ↓ SBP[u] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  | 5 |  |  |  | 9 |
| 1 |  | 10 |  |  |  |  |  |  |  | 18 |
| 2 |  |  | 19 |  |  |  |  |  |  | 26 |
| 3 |  |  |  | 27 |  |  |  |  |  | 33 |
| 4 |  |  |  |  | 34 |  |  |  |  | 39 |
| 5 |  |  |  |  |  | 40 |  |  |  | 44 |
| 6 |  |  |  |  |  |  | 45 |  |  | 48 |
| 7 |  |  |  |  |  |  |  | 49 |  | 51 |
| 8 |  |  |  |  |  |  |  |  | 52 | 53 |
| 9 |  |  |  |  |  |  |  |  |  | 54 |

- Please remove the sentence on Line 44 of Page 414 as follows: Figure 524 pictorially illustrates the usage of the RA field to make 2-sub-band assignments as detailed in Table 816.

- Please remove all the contents of Page 415. This consists of Figure 524 and its caption.

- Please modify the caption of Figure 525 on Page 417, as follows: Allocation of 3 sub-bands, with 2 of them in the same sub-band pair, in a 20 MHz system with YSB > 11

- Please modify the caption of Figure 526 on Page 420, as follows: Allocation of 3 sub-bands, all in different sub-band pairs (except SBP[10]), in a 20 MHz system with YSB > 11

- Please modify the caption of Figure 527 on Page 423, as follows: Allocation of 3 sub-bands, all in different sub-band pairs, including SBP[10], in a 20 MHz system with YSB > 11

- Text Modifications 6

- Please modify the sentence on Line 61 of Page 423 of Section 16.3.6.5.2.4.4 as follows: With the exception of the “IE Type” and the “Resource Allocation” field, all the other fields shall be interpreted in the same manner as defined for the DL UL Basic Assignment A-MAP IE.

- Please add the following sentence following the sentence on Line 6 of Page 424 of Section 16.3.6.5.2.4.4, as follows: The structure and interpretation of the RA field for the UL Sub-band Assignment A-MAP IE shall be the same as that for the RA field for the DL Sub-band Assignment A-MAP IE, with all DL parameters/terms replaced by their UL equivalents., with the exception that a single instance of an allocation shall be made using a single IE in all cases.

- References

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems”, December 2009.