| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Segmentation rules for Assignment A-MAP IEs – Proposed Amendment Text |  |
| Date Submitted | 2010-01-10 |  |
| Source(s) | Zhouyue Pi, Sudhir Ramakrishna, Hyunkyu Yu, Kaushik Josiam, Ying Li, Heewon Kang, Samsung Electronics | z.pi@samsung.com |
| Re: | “Contribution in support of a comment on LB30”: Category – AWD Comments Area – Chapter 16.3.6 (DL Control) |  |
| Abstract | This contribution proposes some modifications to Section 16.3.6.2.2.4 (Assignment A-MAP) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Segmentation rules for Assignment A-MAP IEs – Proposed Amendment Text

Zhouyue Pi, Sudhir Ramakrishna, Hyunkyu Yu, Kaushik Josiam, Ying Li, Heewon Kang

Samsung Electronics

- Introduction

In [1], the size of an MLRU has been specified to be 56 data tones. However, several of the IEs defined are such that they may not fit into a single MLRU, and hence will have to be segmented and conveyed on multiple MLRUs; the GRA IE is an example of such an IE. Segmentation rules need to be specified, so that the bits making up a large Assignment A-MAP IE (i.e., an IE that cannot fit into a single MLRU) can be un-ambiguously mapped to multiple MLRUs.

This contribution proposes such segmentation rules and the associated processing.

In this document, text to be deleted has been struck out and marked with red font. Text to be added is in blue underlined font. Existing text to be retained is in black font. The text modifications are in Section 3.

- Rationale

When segmenting, to minimize the overhead, we would like to maintain the number of IE bits and avoid the addition of additional segmentation indicator bits. Also, the segmentation procedure should be such that it leads to a simple and un-ambiguous parsing procedure at the AMS.

We propose that an IE larger than 40 bits be divided into 40-bit segments, with the CRC-mask providing the required differentiator between the 1st and the subsequent segments.

We propose that when an IE is segmented, the CRC calculated from the 1st segment be masked with the relevant Id, referred to as First_segment_Id. For unicast IEs, the First_segment_Id is the Station Id of the intended recipient AMS, whereas for multicast IEs, it is the relevant Group Id. For all subsequent segments, the CRC is masked with an Id referred to as the NonFirstSegmentId. The NonFirstSegmentId is a unique pre-provisioned Id, known to all ABSs and AMSs.

On decoding a code-block, an AMS may then attempt the CRC-unmasking operation using both the relevant Ids  (StationID, GroupId, etc) and the NonFirstSegmentId. If an un-masking is successful with the Id itself, the AMS can infer that it is the 1st segment; on the other hand, an un-masking success with NonFirstSegmentId would imply that it is not the 1st segment. Subsequent parsing can make use of this information to re-construct the IE.

It may be noted that un-masking is a very simple operation, from the point of view of implementation. Hence, this method does not add to the implementation complexity.

This proposal has the same parsing advantages as other proposals which recommend adding extra segmentation-indicator bits, but does not require any additional overhead.

- Text Modifications

Please insert the text indicated below starting on Line 35 on Page 369 in Section 16.3.6.2.2.4.

------------------------------------------------------ Begin Text Modifications ---------------------------------

The assignment A-MAP IE shall be transmitted with one MLRU or multiple concatenated MLRUs in the A-MAP region. The number of logically contiguous MLRUs is determined based on the assignment A-MAP IE size and channel coding rate, where channel coding rate is selected based on AMS’ link condition.

Assignment A-MAP IEs with less than 40 bits are zero-padded to 40 bits. If an assignment A-MAP IE is larger than 40 bits, the ABS shall process the IE as follows.

- The IE is divided into 40-bit segments, with the 1st segment containing the IE Type as its 1st 4 bits. If the last segment has less than 40 bits, zero padding is applied to obtain 40 bits.

- For each segment, a 16-bit CRC is calculated from the 40-bits.

  - The CRC for the 1st segment shall be masked with the appropriate Id. For unicast IEs, this is the AMS Station Id; for multicast IEs, this is the appropriate Group Id.

  - For all the other segments, the CRC shall be masked with an Id referred to as the NonFirstSegmentId, which is a pre-provisioned unique Id known to all ABSs and AMSs.

- For each segment, the CRC, after masking, is appended to the 40 bits to finally yield a 56-bit segment.

- All segments from the assignment A-MAP IE are coded with the same MCS and occupy a number of logically contiguous MLRUs in sequence.

------------------------------------------------------ End Text Modifications---------------------------------

- References

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems”, December 2009.