| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Avoidance and Mitigation to Support Femto ABS (16.4.11) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Ying Li, Ji-Yun Seol, Young Bo Cho, Zhouyue Pi, Anshuman Nigam, Jung Je Son Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot#30b |  |
| Abstract | This contribution is to propose detail description on triggers for IM. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Interference Avoidance and Mitigation to Support Femto ABS (16.4.11)

Ying Li, Ji-Yun Seol, Young Bo Cho, Zhouyue Pi, Anshuman Nigam, Jung Je Son

Samsung Electronics

# Introduction

- The current draft D3 defines semi-static resource reservation for interference mitigation (IM). However, the triggers for supporting IM, such as the triggers to report strong interference of inaccessible femto, triggers for non-member MS to signal the CSG-femto about the coverage hole, triggers for reserved resource releasing, etc., are missing. This contribution proposes the solutions for the missing parts.

- Questions to be answered:

- Should there be separate triggering conditions for MS to scan ABS for handover purpose or IM purpose?

- What are the trigger conditions for MS to scan/report ABS to initiate IM?

- What are the trigger conditions for MS to scan/report ABS to terminate IM?

- What are the trigger conditions for non-member to send ranging signal to a CSG-femto to indicate it is in a coverage hole because of the inaccessible CSG femto?

# Proposed Text

Revising Two different sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Part I.

[Modify texts as suggested below: Page 603, Line 49]

16.4.11 Interference Avoidance and Interference Mitigation

An AMS may be requested by its serving macro ABS or Femto ABS to report the signal strength measurement of neighbor ABSs, including macro and/or Femto BSs. The reported information can be used by the serving ABS to coordinate with its neighbor BSs to mitigate the interference at the MSs.

Large interference from an inaccessible Femto ABS may trigger a nearby AMS to report the interference to the serving BS, and the report information should include system information of the inaccessible Femto ABS (e.g., BS_ID of the Femto ABS).

The serving ABS and/or the network can request the interfering Femtocell ABS to mitigate the interference by reducing transmission power, and/or blocking some resource region.

[Insert the following, Page 603, Line 49]:

Semi-static/dynamic interference mitigation (IM) shall have lower priority than HO. Only if HO cannot be performed, should the serving ABS consider IM.

AMS shall follow the scanning procedure for HO first. If the serving ABS signal becomes weak while the reported candidate accessible target ABSs are not good enough for HO, the serving ABS may request AMS to scan inaccessible femto ABS via AAI_SCN-RSP so that the AMS can scan and report on trigger defined in Table xxx. The AMS may scan inaccessible ABS blindly or based on the instruction. The serving ABS may request AMS to report the scanning result of inaccessible ABS via AAI_SCN-REP. Even if not requested, the AMS can report inaccessible ABS to the serving ABS via AAI_SCN-REP on trigger defined in Table xxx. The serving ABS may initiate IM with the interfering femto ABS via backhaul.

Non-member AMS may send AAI_RNG-REQ on the trigger defined in Table xxx, to the inaccessible CSG femto ABS, with an indicator to indicate the AMS is in a coverage hole generated by the femto ABS.

If the serving ABS knows that interference mitigation is coordinated and performed with the interfering femto ABS, it may request the AMS to scan the femto ABS performed coordinated IM via unsolicited AAI_SCN-RSP. The scan can be periodical. AMS should scan and report the scanning result to the serving ABS via AAI_SCN-REP, if certain trigger conditions are met, as defined in Table xxx. The report may be sent periodically. The serving ABS may terminate IM with the interfering femto ABS via backhaul.

Part II.

[Create a new section 16.4.11.1 for trigger condition and Add texts as suggested below: Page 604, Line 11]

[Insert the following: Page 604, Line 11]

16.4.11.1 Trigger condition definitions

The ABS may define trigger conditions for the following actions:

- Conditions that define when the AMS should report scanning measurement results of the inaccessible femto ABS to the serving ABS for IM initiation

- Conditions that define when the AMS should report scanning measurement results of the inaccessible femto ABS to the serving ABS for IM termination

- Conditions that define when non-member AMS should initiate signaling to CSG femto about the coverage hole due to interference of the CSG femto, by sending AAI_RNG-REQ.

- Conditions that define when the AMS should start scanning procedure for inaccessible femto ABS for IM termination

The trigger TLV (type xx) in Table xxx is encoded using the description in Table XX. The trigger TLV for another ABS type (which includes ABS type parameter: such as macro, macro hot-zone, Femto and relay, etc) is encoding using the description in Table XX

Table xxx—Trigger TLV Description

| Name | Type | Length (Bytes) | Value |
| --- | --- | --- | --- |
| Type/Function/Action | xx.1 | 1 | See Table XX—for description |
| Trigger Value | xx.2 | 1 | Trigger value is the value used in comparing measured metric for determining a trigger condition |
| Trigger averaging duration | xx.3 | 1 | Trigger averaging duration is the time measured in number of frames over which the metric measurements are averaged. |

Table XX—Trigger; Type/Function/Action Description

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Type | 2(MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric 0x2: RTD metric 0x3: Number of missed superframes metric |
| Function | 3 | Computation defining trigger condition: 0x0: Reserved 0x1: Metric of neighbor inaccessible CSG femto ABS is greater than absolute value 0x2: Metric of neighbor inaccessible CSG femto ABS is less than absolute value 0x3: Metric of neighbor inaccessible CSG femto ABS is greater than serving ABS metric by relative value 0x4: Metric of neighbor inaccessible CSG femto ABS is less than serving ABS metric by relative value 0x5: Metric of serving ABS greater than absolute value 0x6: Metric of serving ABS less than absolute value 0x7: Reserved NOTE-0x1-0x4 not applicable for RTD trigger metric NOTE-When type 0x1 is used together with function 0x3 or 0x4, the threshold value shall range from -32 dB (0x80) to +31.75 dB (0x7F). When type 0x1 is used together with function 0x1, 0x2, 0x5 or 0x6, the threshold value shall be interpreted as an unsigned byte with units of 0.25 dB, such that 0x00 is interpreted as -103.75 dBm and 0xFF is interpreted as -40 dBm NOTE-Type 0x3 can only be used together with function 0x5 or function 0x6 |
| Action | 3(LSB) | Action performed upon reaching trigger condition: 0x0: Reserved 0x1: Respond on trigger with AAI_SCN-REP to initiate IM 0x2 : Respond on trigger with AAI_SCN-REP to terminate IM 0x3: Respond on trigger with AAI_RNG-REQ with ‘Indicator of coverage hole due to inaccessible femto’ being ‘1’ to indicate that non-member AMS is in a coverage hole generated by CSG femto ABS. 0x4: Respond on trigger with AMS scanning to terminate IM 0x5, 0x6 and 0x7: Reserved NOTE -0x2, 0x4 are only applicable after IM is initiated. |

The ABS may define complex trigger conditions by including multiple Trigger TLV encodings in the same compound TLV. In this case, all included triggers shall have the same Action code (as defined in Table XX). The AMS shall perform a logical AND of all the conditions in a complex trigger condition and invoke the action of the trigger only when all trigger conditions are met.

Whenever the condition of a simple trigger or all the conditions of a complex trigger are met, the AMS shall invoke the action of the trigger. If multiple trigger conditions are met simultaneously the AMS shall invoke the action of at least one of the triggers for which the trigger condition was met.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------