| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS scanning of neighbor Femto ABSs (16.4.8.1.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Ying Li, Zhouyue Pi, Anshuman Nigam, Jung Je Son Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot#30b |  |
| Abstract | This contribution is to propose detail description on triggers for IM. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# AMS Scanning of Neighbor Femto ABSs (16.4.8.1.2)

Ying Li, Zhouyue Pi, Anshuman Nigam, Jung Je Son

Samsung Electronics

# Introduction

- In the current draft D3, it describes some methods on how AMS performs cell detection, and how AMS scans its neighbor femto ABSs. However, some of these methods are not clearly described, or some of them could be further optimized, and some corresponding messages are not complete. Therefore, this contribution proposes some clarifications of the cell detection and AMS scanning.

# Proposed Text

Revise four sections (section 16.4.8.1.2 and related messages modification) in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Part 1.

[Modify texts as suggested below: Page 602, Line 5]

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

AMS may include information detected or based on whitelist, e.g., the FA, preambles, BSID, CSGIDs of the subscribed CSG femto BSs, location information, in the AAI_NBR-REQ message.

Part 2.

[Modify texts as suggested below: Page 66, Line 36]

16.2.3.13 AAI_SCN-REQ

Table 685—Parameters for AAI_SCN-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message Scan purpose bit map (2-bit): bit # 1: being ‘1’ indicating for BS in NBR list; bit#2: being ‘1’ indicating for BS not in NBR list. |  |
| … | … | … |
| If scan purpose bit #1=’1’, list of ABSs based on the AAI_NBR-ADV |  |  |
| … | … | … |
| If scan purpose bit #2=’1’, list of ABSs not on the AAI_NBR-ADV |  |  |
| … | … | … |

Part 3.

[Modify texts as suggested below: Page 68, Line 52]

16.2.3.14 AAI_SCN-RSP

Table 686—Parameters for AAI_SCN-RSP message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message Scan purpose bit map (2-bit): bit # 1: being ‘1’ indicating for BS in NBR list; bit#2: being ‘1’ indicating for BS not in NBR list. |  |
| … | … | … |
| If scan purpose bit #1=’1’, list of ABSs based on the AAI_NBR-ADV |  |  |
| … | … | … |
| If scan purpose bit #2=’1’, list of ABSs not on the AAI_NBR-ADV |  |  |
| … | … | … |

Part 4.

[Modify texts as suggested below: Page 70, Line 31]

16.2.3.15 AAI_SCN-REP

Table 687—Parameters for AAI_SCN-REP message

| Name | Value | Usage |
| --- | --- | --- |
| Neighbor list request indication | ‘1’ requesting neighbor list ‘0’ not requesting neighbor list |  |
| … | … | … |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------