| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS scanning of neighbor Femto ABSs (16.4.8.1.2) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Ying Li, Zhouyue Pi, Anshuman Nigam, Jung Je Son Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot#30b |  |
| Abstract | This contribution is to propose detail description on triggers for IM. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# AMS Scanning of Neighbor Femto ABSs (16.4.8.1.2)

Ying Li, Zhouyue Pi, Anshuman Nigam, Jung Je Son

Samsung Electronics

# Introduction

- In the current draft D3, it describes some methods on how AMS performs cell detection, and how AMS scans its neighbor femto ABSs. However, some of these methods are not clearly described, or some of them could be further optimized, and some corresponding messages are not complete. Therefore, this contribution proposes some clarifications of the cell detection and AMS scanning.

# Proposed Text

Revise five sections (section 16.4.8.1.2 and related messages modification) in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Part 1.

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

[Modify texts as suggested below: Page 601, Line 23-35]

For neighbor Femto ABSs, an AMS performs the scanning procedure as per 16.3.6.1.2 with exceptions described in this subsection. An AMS may scan femto ABSs according to the FAs included in the broadcast AAI_NBR-ADV message. AMS may scan femto ABSs that are not included in AAI_NBR-ADV based on SA-preamble partitioning information (see 16.4.4). In addition, an AMS may scan allowed femto ABSs based on the CSG White List. Based on location information, AMS may initiate the scanning procedure (See 16.2.6.1.2). The AMS may request additional scanning opportunities by sending AAI_SCN-REQ including the detected SA-preamble index and associated FA information, and/or BSID, CSGID, and indicate whether it is for a regular scanning request or it is for scanning and neighbor request. Upon reception of the AAI_SCN-REQ with neighbor and scanning request indication being ‘0’, the ABS shall respond with an AAI_SCN-RSP regularly without optimizing a neighbor list for the AMS. Upon reception of the AAI_SCN-REQ with scanning and neighbor request indication being ‘1’, the ABS shall respond with an AAI_SCN-RSP including neighbor accessible Femto ABS list based on the SA-preamble index while the list may include information such as SA-preamble, FA, BSID, CSGID, etc.

Alternatively, an overlapped overlay Macro ABS may recommend accessible CSG Femto ABS to monitor UL signaling of accessible its member AMS which is served by the Macro ABS. The serving Macro ABS should further select a set of the accessible CSG Femto ABS for an AMS based on AMS location information, or detected base station information reported by the AMS, if such information is available, before the serving Macro ABS recommends the CSG Femto ABS to monitor UL signaling. If the set of the accessible CSG Femto ABS selected by the serving Macro ABS is empty, the serving Macro ABS shall not recommend any CSG Femto ABS to monitor UL signaling. If CSG Femto ABS is in low-duty mode, the CSG Femto ABS can try to receive A-MAP information from overlapped overlay Macro ABS and then monitor accessible its member AMS’s UL signaling in associated allocations. If CSG Femto ABS is not in low-duty mode, overlapped Macro ABS may trigger dedicated periodic ranging (see 15.4.7.4.2) for the AMS. A CSG Femto ABS can monitor the ranging preamble at the dedicated ranging slot of Macro ABS. When the received signaling quality (e.g RSSI) of an AMS is stronger than a threshold, satisfies certain conditions, CSG Femto ABS can request overlapped overlay Macro ABS to send unsolicited AAI_SCN-RSP for the AMS to scan the CSG Femto ABS.

When the AMS has to request for information of neighboring Femto ABSs belonging to a CSG then the AMS can provide the desired CSGID(s) in the AAI_SCN-REQ message to the serving ABS. The ABS can respond with a list of ABSs, addressed by BSID belonging to the requested CSGID(s) in AAI_SCN-RSP message.

ABS may send an unsolicited AAI_SCN-RSP for the AMS to scan the Femto ABS.

After scanning, the AMS may report A-preamble indices and associated physical carrier indices, or BSIDs by sending AAI_NBR-REQ or AAI_SCN-REP with neighbor request indication being ‘1’, to which the ABS may unicast an AAI_NBR-ADV that includes a list of femto ABSs which is formed based on the reported A-preamble indexices and associated physical carrier indices, or BSIDs, or the reported measurement, or any other recommended femto ABSs based on location information.

Upon identifying the existence of any Femto ABSs, the AMS may transmit an AAI_SCN-REP message including the list of the detected Femto ABSs and measurement results according to the Trigger conditions included in the AAI_SCD message.

If the AMS decides to perform HO to any Femto ABS, it may request more detailed system information of the detected neighbor Femto ABSs to the serving ABS using an AAI_NBR-REQ message.

AMS may include information detected or based on whitelist, e.g., the FA, preambles, BSID, CSGIDs of the subscribed CSG femto BSs, location information, in the AAI_NBR-REQ message.

AMS may include location information if available in AAI_SCN-REQ with neighbor request indication being ‘1’, or AAI_SCN-REP with neighbor request indication being ‘1’, or AAI_NBR-REQ. The location information may be the information from GPS at AMS. The location information, and/or the reported detected ABSs information, and/or the information of the whitelist at the AMS, can be used by the ABS to optimize the neighbor list.

When the serving BS receives AAI_NBR-REQ, AAI_SCN-REP/ AAI_SCN-REQ with the indicator of ‘NBR request” being ‘1’, it may respond with AAI_SCN-RSP with ‘response for the neighbor request’ indication bit as ‘1’.

If the indicator responding for the neighbor request is ‘1’, in AAI_SCN-RSP, it can contain both the optimized neighbor list and some scanning interval recommendations for some of the ABSs in the optimized neighbor list. If the indicator is ‘0’, it is a normal AAI_SCN-RSP.

Part 2.

[Modify texts as suggested below: Page 66, Line 36]

16.2.3.13 AAI_SCN-REQ

Table 685—Parameters for AAI_SCN-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message Scan purpose bit map (2-bit): bit # 1: being ‘1’ indicating for BS in NBR list; bit#2: being ‘1’ indicating for BS not in NBR list. | If purpose bit #1 is ‘1’, including the fields related to the scanning based on NBR list If purpose bit #2 is ‘1’, including the fields related to the scanning not based on NBR list |
| Neighbor list request indication | ‘0’ regular scanning request ‘1’ scanning and neighbor request |  |
| Interleaving Interval | The period of AMS's normal operation which is interleaved between Scan Durations. |  |
| Scan Iteration | The requested number of iterating scanning interval by an AMS. |  |
| Recommended Start Frame indication | Indicates whether Recommended start super frame number field and Recommended start frame field are included |  |
| N_Recommended_BS_Index_indication | Indicates whether N_Recommended_BS_Index field is included in this message |  |
| N_Recommended_BS_Full_indication | Indicates whether N_Recommended_BS_Full field is included in this message. |  |
| N_Recommended_Preamble_ Index_indication | Indicates whether N_Recommended_Preamble_Index field is included in this message |  |
| Recommended start super frame number | Represents the 6 least significant bits of the absolute super frame number for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. | Shall be included when Recommended Start Frame indication is set to 1 |
| Recommended start frame number | Represents recommended start frame number within the super frame number which is indicated by Recommended start super frame number field. The value indicates: 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super frame | Shall be included when Recommended Start Frame indication is set to 1 |
| If scan purpose bit #1=’1’, |  |  |
| { |  |  |
| N_Recommended_BS_Index_indication | Indicates whether N_Recommended_BS_Index field is included in this message |  |
| N_Recommended_BS_Index | This is the number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. When an AMS receives AAI_SCN-RSP message from BS in response to AAI_SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI_SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI_SCN-REQ message to the ABS. | Shall be included when N_Recommended_BS_Index_indication is set to 1 |
| Configuration Change Count for AAI_NBR-ADV | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID |  |
| Neighbor_BS_Index | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Shall be included when N_Recommended_BS_Index > 0 |
| } |  |  |
| N_Recommended_BS_Full | Number of neighboring BS to be scanned, which are using full 48-bit BSID. | Shall be included when N_Recommended_BS_Full_indication is set to 1 |
| Recommended BS ID | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_BS_Full > 0 |
| N_Recommended_ Preambe_Index | Number of preambles to be scanned | Shall be included when N_Preamble_Index_Indication is set to 1 |
| Preamble Index | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Shall be included when N_Recommended_Preamble_Index> 0 |
| N_Recommended_Carrier_Index | Number of carriers to be scanned at each neighboring ABS. | May be included when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| Recommended_Carrier_Index | Recommended physical carrier index for scan at each neighboring ABS. | Shall be included when N_Recommended_Carrier_Index > 0 |
| N_Recommended_Carrier_Index_at_Serving_ABS | Number of physical carrier index of the serving ABS to be scanned |  |
| Carrier index at serving ABS | Recommended physical carrier index to be used for scanning | Shall be included when N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| If scan purpose bit #2=’1’, |  |  |
| { |  |  |
| N_CSGID indication | ‘1’ CSG ID included ‘0’ CSG ID not included |  |
| N_CSGID | Number of the CSGIDs | Shall be included when N_CSG_indication is ‘1’ |
| CSGID(s) | One or more identifiers for one or more CSG(s) | Sent by AMS to aid ABS reselection. Shall be included when N_CSGID >0 |
| MS Location information indicator | ‘1’ MS GPS information available ‘0’ MS GPS information not available | Include when Neighbor list request indication is ‘1’ |
| MS location information | Location information: Latitude Longitude Or Other coordinate system | Include when Neighbor list request indication is ‘1’ and MS location information indicator is ‘1’ |
| } |  |  |

Part 3.

[Modify texts as suggested below: Page 68, Line 52]

16.2.3.14 AAI_SCN-RSP

Table 686—Parameters for AAI_SCN-RSP message

| Name | Value | Usage |
| --- | --- | --- |
| Scan duration | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Scan Purpose | 0b0: scan BS(s) which is in the list of the AAI_NBR-ADV message0b1: scan BS(s) which is not in the list of the AAI_NBR-ADV message Scan purpose bit map (2-bit): bit # 1: being ‘1’ indicating for BS in NBR list; bit#2: being ‘1’ indicating for BS not in NBR list. |  |
| Respond to neighbor list request indication | ‘1’ respond to neighbor request (AAI_NBR-REQ, or AAI_SCN-REP with neighbor list request indication as ‘1’, or AAI_SCN-REQ with neighbor list request indication as ‘1’) ‘0’ not a response to neighbor list request |  |
| … | … | … |
|  |  | … |
| … | … | … |

Part 4.

[Modify texts as suggested below: Page 70, Line 31]

16.2.3.15 AAI_SCN-REP

Table 687—Parameters for AAI_SCN-REP message

| Name | Value | Usage |
| --- | --- | --- |
| Neighbor list request indication | ‘1’ requesting neighbor list ‘0’ not requesting neighbor list |  |
| MS Location information indicator | ‘1’ MS GPS information available ‘0’ MS GPS information not available | Include when Neighbor list request indication is ‘1’ |
| MS location information | Location information: Latitude Longitude Or Other coordinate system MS Speed? | Include when Neighbor list request indication is ‘1’ and MS location information indicator is ‘1’ |
| Report mode | Action code for an AMS's scan report of its measurement 0: Event-triggered report 1: Periodic report according to Scan report period of AAI_SCN-RSP |  |
| … | … | … |

Part 5.

[Modify texts as suggested below: Page 96, Line 43]

16.2.3.33 AAI_NBR-REQ

Table xxx—Parameters for AAI_NBR-REQ message

| Name | Value | Usage |
| --- | --- | --- |
| CSGID in the whitelist at the MS |  |  |
| MS Location information indicator | ‘1’ MS GPS information available ‘0’ MS GPS information not available |  |
| MS location information | Location information: Latitude Longitude Or Other coordinate system MS Speed? | Include when Neighbor list request indication is ‘1’ and MS location information indicator is ‘1’ |
| Report detected ABSs |  |  |
| … | … | … |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------