| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for Femto ABS Detection (16.4.8.1.2) |  |
| Date Submitted | 2010-01-07 |  |
| Source(s) | Ying Li, Zhouyue Pi, Sudhir Ramakrishna, Baowei Ji, Anshuman Nigam, Jung Je Son Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot#30b |  |
| Abstract | This contribution is to propose detail description on triggers for IM. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support for Femto ABS Detection (16.4.8.1.2)

Ying Li, Zhouyue Pi, Sudhir Ramakrishna, Baowei Ji, Anshuman Nigam, Jung Je Son

Samsung Electronics

# Introduction

- The current draft D3 has the following text for AMS to detect Femto ABS.

- “Alternatively, an overlapped Macro ABS may recommend CSG Femto ABS to monitor UL signaling of accessible AMS which is served by the Macro ABS. If CSG Femto ABS is in low-duty mode, the CSG Femto ABS can try to receive A-MAP information from overlapped Macro ABS and then monitor accessible AMS’s UL signaling in associated allocations. If CSG Femto ABS is not in low-duty mode, overlapped Macro ABS may trigger dedicated periodic ranging (see 15.4.7.4.2) for the AMS. A CSG Femto ABS can monitor the ranging preamble at the dedicated ranging slot of Macro ABS. When the received signaling quality (e.g RSSI) of an AMS is stronger than a threshold, CSG Femto ABS can request overlapped Macro ABS to send unsolicited AAI_SCN-RSP for the AMS to scan the CSG Femto ABS.”

- However, the above approach for femto in LDM, for femto enterprise use case, many AMSs and femtos will be associated with one CSG. One femto may have to monitor many AMSs, the error rate for the blind decoding of A-MAP can be high since femto has to check A-MAP with many AMSs station IDs. The approach is not effective for enterprise case.

- The above approach for femto not in LDM, the AMS has to perform ranging periodically where such ranging transmitting may consume more battery of the AMS than AMS scanning. In addition, if there are many subscribed CSG femtos in the macro coverage for the MS, then all these femtos will monitor the ranging, where actually monitoring at some of the femtos are not necessary. Hence, the approach is not efficient.

- Also, the dedicated periodic ranging is not defined yet in D3.

- In this contribution, we propose clean-up text for the text in D3 mentioned above.

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Part I.

[Modify texts as suggested below: Page 603, Line 49]

16.4.11 Interference Avoidance and Interference Mitigation

Alternatively, an overlapped overlay Macro ABS may recommend accessible CSG Femto ABS to monitor UL signaling of accessible its member AMS which is served by the Macro ABS. The serving Macro ABS should further select a set of the accessible CSG Femto ABS for an AMS based on AMS location information, or detected base station information reported by the AMS, if such information is available, before the serving Macro ABS recommends the CSG Femto ABS to monitor UL signaling. If the set of the accessible CSG Femto ABS selected by the serving Macro ABS is empty, the serving Macro ABS shall not recommend any CSG Femto ABS to monitor UL signaling. If CSG Femto ABS is in low-duty mode, the CSG Femto ABS can try to receive A-MAP information from overlapped overlay Macro ABS and then monitor accessible its member AMS’s UL signaling in associated allocations. If CSG Femto ABS is not in low-duty mode, overlapped Macro ABS may trigger dedicated periodic ranging (see 15.4.7.4.2) for the AMS. A CSG Femto ABS can monitor the ranging preamble at the dedicated ranging slot of Macro ABS. When the received signaling quality (e.g RSSI) of an AMS is stronger than a threshold, satisfies certain conditions, CSG Femto ABS can request overlapped overlay Macro ABS to send unsolicited AAI_SCN-RSP for the AMS to scan the CSG Femto ABS.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------