| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on TEK usage after re-authentication and TEK FSM (16.2.5) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Avishay Shraga Chang Hong Shan Xiangying Yang Intel | E-mail: avishay.shraga@intel.com Phone : +972-54-5551063 |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.5.2.1.5.3” |  |
| Abstract | This contribution proposes text for TEK usage after re-authentication and TEK FSM |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on TEK usage after re-authentication (16.2.5)

Avishay Shraga, Chang Hong Shan, Xiangying Yang

Intel

- Introduction

PKMv3 introduces a new scheme for TEK update due to the fact that TEKs are derived from AK in all cases.

This FSM describes the AMS behavior on TEK management and update in the different cases

- Proposed Test

Text change 1: Modify Section 16.2.5.2.1.5.3 as follows

--------------------------------text start-----------------------------------------

16.2.5.2.1.5.3 TEK update after Re-authentication

The re-authentication is done via key-agreement three-way handshaking (With or Without Full EAP authentication) similar to that defined in Figure 400. Note that for re-authentication, i) after the key agreement, the AKold is still valid and ii) only one new TEK is derived right after key agreement. The detail procedure is as follows

•Key agreement finishes with generation of AKnew, but AKold is still valid.

•Right after Key agreement, ABS updates its TEKDLE with TEKULE and derives new TEKULE from AKNEW

•Right after the AMS realize key agreement finished successfully it starts the TEK_Reauth_Timer which once expired - the BS is already expected to derive the first TEK from new AK. Once timer expires or ABS uses its TEKULE in the DL (if AMS monitors this event), the AMS initiates TEK update procedure in order to obtain new TEKULE, this procedure is using the old AK sequence Number in key request and for CMAC calculations. (note that the PKMv3messages include AK seq number for the sake of CMAC calculation, in a given message the CMAC and TEK keys must be derived from the same AK).

•The ABS is expected to responds with the sequence number of AKnew (which is used in both CMAC and TEK) and AMS can know it needs to derive its TEKULE from AKnew

•After TEK update procedure with the AMS was completed and the ABS knows the AMS poses TEKULE derived from the new AK  , either by using it in UL or by receiving another TEK_REQ using the new AK seq number, it should derive another TEK to discard all keys dependent on AKold, and then than it can discard AKold as well.

•After obtaining the first TEK from the new AK, the AMS initiates another key update procedure using the new AK seq-number to obtain the second TEK derived from new AK.

--------------------------------text end-----------------------------------------

Text change 2: Add the proposed text to the section 16.2.5.2 as a new subsection 16.2.5.2.6 as follows.

----------------------------------------- Start of second Proposed Text --------------------------------------------

16.2.5.2.6 TEK state machine

The AMS TEK state machine of 16m consists of six states and eight events (including messages) which will trigger state transitions. The TEK FSM is presented in both a state flow diagram (Figure 16.2.5.2.y.1) and a state transition matrix (Table 16.2.5.2.y.1).

TEK FSM under shaded states in Figure 16.2.5.2.y.1 has valid SA Context.

The Authentication FSM starts an independent TEK FSM for each of its authorized SAIDs. As mentioned in [1], the AMS maintains two active TEKs for each SAID.

For the TEK update of a given SAID, the ABS includes in its Key Response with parameters such as EKS, AK Seq_Num, COUNTER_TEK. The BS encrypts DL traffic with TEKDLE and decrypts UL traffic according to the EKS bit, depending upon which of the two keys the AMS used at the time. The AMS encrypts UL traffic with TEKULE and decrypts DL traffic according to the EKS bit. See 15.2.5.2.4 [1] for details on AMS and ABS key usage requirements.

Through operation of a TEK FSM, the AMS attempts to keep the SAID related TEK Context synchronized with the ABS as defined in this spec section XXXX. TEK state machine issues TEK-REQ message to update the related TEK Context for the indicated SAID whenever required. When AMS receives a TEK-RSP message and there is a new COUNTER_TEK (not maintained by the AMS at the moment), AMS shall always update its records with the TEK Context contained in the TEK-RSP message for the associated SAID.

- Figure 16.2.5.2.y.1 TEK State Machine for PKMv3

- Table 16.2.5.2.y.1. TEK FSM state transition matrix for PKMv3

| State Event or Rcvd Message | (A) Stopped/Idle | (B) TEK in Active | (C) Rekey Wait | (D) Post key agreement | (E) Re-auth Rekey Wait | (F) Re-auth 2nd Rekey Wait |
| --- | --- | --- | --- | --- | --- | --- |
| (1) [TEK]Stop |  | Stopped/idle | Stopped/idle | Stopped/idle | Stopped/idle | Stopped/idle |
| (2) [TEK]Start | TEK in Active |  |  |  |  |  |
| (3) [TEK]Key agreement Finished |  | Post Key agreement | Post Key agreement |  |  |  |
| (4)TEK-RSP (current AK) |  |  | TEK in Active |  | Re-auth Rekey Wait |  |
| (5) TEK-RSP (new AK) |  |  | Re-auth 2nd Rekey Wait |  | Re-auth 2nd Rekey Wait |  |
| (6) TEK-RSP (newer EKS) |  |  | TEK in Active |  |  | TEK in Active |
| (7) New TEK used |  | Rekey Wait | Rekey Wait | Re-auth Rekey Wait | Re-auth 2nd Rekey wait |  |
| (8) PN Space Exhausted |  | Stopped/idle | Stopped/idle | Stopped/idle | Stopped/idle | Stopped/idle |
| (9) PN Grace space exhausted |  | Rekey Wait | Rekey Wait |  |  |  |
| (10) Rekey Counter threshold |  |  | TEK in Active |  |  |  |
| (11) Rekey Wait Timer timeout |  |  | Rekey Wait |  | RE-Authen Rekey Wait | Re-auth 2nd Rekey wait |
| (12) TEK Re-Auth timer expired |  |  |  | RE-Authen Rekey Wait |  |  |

16.2.5.2.y.1 States

Stopped/idle: This is the initial state of TEK FSM, No resources are assigned to or used by the FSM in this state - e.g., all timers are off, and no processing is scheduled.

This state is used before there are valid TEKs and in IDLE.

TEK in Active: The AMS has valid SA Context for the associated SAID and is not waiting for any TEK-RSP message and AMS is in active/HO mode under this state.

Rekey Wait: The AMS has valid SA Context for the associated SAID and is waiting for TEK-RSP message for regular TEK update for this SAID. AMS is in active mode under this state.

Post key agreement: This is a transition state after re-authentication to let the BS time to derive first TEK from new AK, in this state all TEKs are active and the AMS waits for the timer to expire so it can send TEK_REQ to the ABS.

While in this state the AMS SHALL discard all received TEK_RSP messages from ABS

Re-auth Rekey Wait: After key agreement with associated AK refresh, AMS have send TEK_RSP and stays in this state to waits for TEK-RSP message to refresh its TEKULE derived from new AK. AMS is in active mode under this state.

Re-auth 2nd Rekey Wait: When receiving the TEK-RSP message and AMS successfully refresh its first TEKULE associated with new AK, AMS shall transfer to this state to wait for the second refreshed TEK from new AK.

This state is different than normal Rekey wait in the sense that the MS SHALL not leaves this state without refreshing the second TEK from new AK.

15.2.5.3.y.2 Messages

- Note that the message formats are defined in detail in x.x.x.x.x.

TEK-REQ: refer to section x.x.x.x.x.

TEK-RSP: refer to section x.x.x.x.x.

15.2.5.3.y.3 Events

[TEK]Stop: Sent by the Authorization FSM to TEK FSM to terminate TEK FSM and remove the corresponding SAID’s TEK Context from the AMS’s key table.

[TEK]Start: Sent by the Authorization FSM to TEK FSM or a new TEK FSM to activate the TEK FSM for the associated SAID.

[TEK] New TEK used: This event MAY be triggered by the Data Path function (it is implementation specific weather to monitor this event or not) when current PMK is valid and the AMS realizes that ABS uses its TEKULE in the DL,. Once the active TEK FSM receives this event, it shall start to update the related TEK.

[TEK] Key Agreement Finished: This event is sent from the Authentication FSM to trigger the TEK update after re-auth procedure.

[Auth] TEK update finished: This event is sent to the Authentication FSM to notify that both TEK are updated with new AK and old AK context can be discarded.

PN space exhausted: This event is triggered when the PN space for the current TEK in use is exhausted.

PN grace space exhausted: This event is triggered when the PN reached its grace space and triggers TEK update procedure.

Rekey Counter threshold: This event is triggered when the number of times AMS retries TEK-REQ exceeds the maximal Rekey Counter threshold. The AMS shall continue using its current TEK.

Rekey Wait Timer timeout: This event is triggered when AMS does not receive TEK-RSP by the Rekey Wait Timer expiration.

TEK Re-Auten Timer exhausted: This event trigger the AMS to request for a new KEY after key agreement.

16.2.5.2.y.4 Parameters

All configuration parameter values take the default values from Table xxx or may be specified in Auth Reply message.

- PN Grace Space:

PN Grace Space is set to the value smaller than the maximum number of the PN Space to guarantee that AMS can update related TEK between the interval of PN Grace Space and PN Space gracefully.

It takes the default value from Table XXX..

- Rekey Wait Timer:

The timer for TEK FSM to receive TEK-RSP message after sending out TEK-REQ message.

It takes the default value from Table xxx or may be specified in a configuration setting within the Auth Reply message and is the same across all SAIDs (see 11.9.18.6).

Rekey Counter:

The counter for re-sending the TEK-REQ if AMS doesn’t receive the TEK-RSP by the Rekey Wait Timer for the SA.

The initial value is 0. It shall be increased by 1 for each re-sending. The threshold takes the value from Table XXX.

TEK Re-Authen Timer:

The timer fro the AMS to wait after key agreement before sending the key request to the ABS, this timer is used to allow the BS time to derived the new key before asking for it.

The value of this timer is taken from XXXX>

16.2.5.2.y.5 Actions

Actions taken in association with state transitions are listed by <state> (<event>) --> <state>:

1-B,C,D,E,F: Any state ([TEK]stop)->Stopped/idle

a) Remove TEK Context for the related SAID

b) Clear UL/DL PN number

2-A: Stopped ([TEK]start)->TEK in Active

a) Generate TEK context for the related SAID

b) Set TEK counter to 0

c) Set PN for each TEK to 1

3-B: TEK in Active ([TEK] Key agreement Finished)Post key agreement

a) Start TEK re-Authen Timer

3-C: Rekey wait ([TEK] Key agreement Finished)Post key agreement

a) Start TEK re-Authen Timer