| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on Authentication FSM (16.2.5) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Avishay Shraga Chang-Hong Shan Xiangying Yang Intel | E-mail: avishay.shraga@intel.com Phone : +972-54-5551063 |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.5.2.5” |  |
| Abstract | This contribution proposes updates and cleanup on the PKMv3 authentication/ state machines |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on Authentication FSM (16.2.5)

Avishay Shraga, Chang Hong Shan, Xiangying Yang

Intel

- Introduction

PKMv3 state machine was accepted and incorporated into D3 draft.

This FSM need some cleanup and updates as result from interaction with the proposed TEK FSM

- Proposed Test

Modify section 16.2.5.2.5 text and figure with the following text

--------------------------------text start-----------------------------------------

16.2.5.2.5 AMS Authentication state machine

The PKMv3 authentication state machine consists of six states and 18 events (including receipt of messages and events from other FSMs) that may trigger state transitions and send events/messages. The authentication state machine is presented in both a state flow diagram (Figure 405) and a state transition matrix (Table 725). The state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

The PKMv3 Authentication process has 2 phases: EAP phase and key agreement phase.

The EAP phase is controlled by the EAP_FSM as defined in RFC3748 and RFC4173 and it is out of scope in this standard.

The Auth_FSM is responsible for all PKMv3 phase excluding the actual EAP exchange, it is also responsible for communicates with other FSMs in the system using events.

The relationships between the security related FSMs in the system are as described in the Figure xxx.

Figure xxx—System relationships in security related FSM

Through operation of an Authentication state machine, the AMS attempts to get authenticated with the NW, maintain this authentication and support Authentication context switching for Re-authentication, PMK refresh, HO, zone switching and Idle situations. The state machine takes care of requesting the BS to renew the key hierarchy before it expires either by  initiating re-authentication  or PMK refresh only. it also supports key derivations according to definitions for optimized re-entry for HO, for location update and idle.

The optimized re-entry/Location update support is done in a special state in which the NW connection is suspended and therefore re-authentication can’t occur, the triggers for re-authentication continue to work in this state but the initiation is done only after returning to an authenticated state.

Figure 405 Authentication State Machine for PKMv3

Table 725. Authentication FSM state transition matrix for PKMv3

| State Event or receive message | (A) Stopped | (B) Not Authenticated | (C) Key Agreement MSG #3 Wait | (D) Authenticated Active | (E) Refresh Key Agreement MSG #3 Wait | (F) Authenticated Reentry Auth Wait | (G) Authenticated idle |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (1) Start Auth | Not Authenticated |  |  |  |  | Not Authenticated | Not Authenticated |
| (2) PKMv3 Key Agreement MSG #1 |  | Key Agreement MSG #3 Wait | Key Agreement MSG #3 Wait | Refresh Key Agreement MSG #3 Wait, | Refresh Key Agreement MSG #3 Wait |  |  |
| (3) PKMv3 Key Agreement MSG #3 |  |  | Authenticated Active |  | Authenticated Active |  |  |
| (4) EAP Success |  | Not Authenticated |  | Authenticated Active |  |  |  |
| (5) Key Agreement Timeout |  |  | Key Agreement MSG #3 Wait |  | Refresh Key Agreement MSG #3 Wait |  |  |
| (6) Key Agreement MSG #2 max resend elapsed |  |  | Not Authenticated |  | Authenticated Active |  |  |
| (7) Key Context Refresh needed |  |  |  | Authenticated Active |  |  |  |
| (8) Start HO Reentry |  |  |  | Authenticated Reentry Auth Wait | Authenticated Reentry Auth Wait |  |  |
| (9) refresh-PMK timeout |  |  |  | Authenticated Active |  |  |  |
| (10) HO cancelled |  |  |  |  |  | Authenticated Active |  |
| (11) TBS change |  |  |  |  |  | Authenticated Reentry Auth Wait |  |
| (12) Reentry Completed |  |  |  |  |  | Authenticated Active |  |
| (13) Auth Expired |  |  |  | Stopped | Stopped | Stopped | Stopped |
| (14) EAP Fail |  |  |  | Authenticated Active |  |  |  |
| (15) External Stop |  | Stopped | Stopped | Stopped | Stopped | Stopped |  |
| (16) Exit to Idle |  |  |  | Authenticated Idle | Authenticated Idle |  |  |
| (17) ReEntry from Idle |  |  |  |  |  |  | Authenticated Active |
| (18) Secure Location Update |  |  |  |  |  |  | Authenticated Idle |
| (19) TEK update finished |  |  |  | Authenticated Active |  |  |  |

16.2.5.2.5.1 States

Stopped: This is the initial state of the FSM. Nothing is done in this state.

Not Authenticated: The Authentication FSM is not authenticated and waiting for an MSK from the EAP FSM and start of key agreement,

Key Agreement MSG #3 Wait: The Authorization Authentication FSM holds all key hierarchy derived from MSK and is waiting to receive MSG#3 in order to validate the keys with the BS:

  - Resend MSG#2 if valid MSG#3 was not received within Key Agreement Timer.

  - Resend MSG#2 if MSG#1 with same NONCE received again (reset resend counter).

  - Discard MSG#3 received with invalid CMAC

Authenticated Active: The AMS has successfully completed EAP-based authentication and key agreement and has valid Key context derived from the MSK received from the EAP FSM. All SAs are created and TEK FSM is active for each SA:

  - PMK or derivatives (not including TEK) is about to expire 'send refresh-PMK, start refresh-PMK timer

  - refresh-PMK timer expired ' send refresh-PMK.

  - AK_Count/ CMAC_PN_* is about to be exhaust ' send refresh-PMK

  - All management messages are protected as defined in SPEC.

  - Received messages without valid encryption/CMAC are discarded.

  - Manage two key Context during transition period between 2 key agreements.

  - PMK or its derivatives (not including TEK) is about to expire -> AMS sends refresh-PMK, and refresh-PMK timer starts

  - refresh-PMK timer is expired -> AMS sends refresh-PMK.

  - AK_Count/ CMAC_PN_* is about to be exhausted -> AMS sends refresh-PMK

  - All management messages are protected as defined in Table 673.

  - Received messages without valid encryption/CMAC are discarded.

  - Manage two key Context during transition period between 2 key agreements.

Refresh Key Agreement MSG #3 Wait: The Authentication FSM holds all key hierarchy derived from newest MSK (in parallel to active context used for ongoing operation) and is waiting to receive MSG#3 in order to validate the keys with the BS:

  - Resend MSG#2 if valid MSG#3 was not received within Key Agreement Timer.

  - Resend MSG#2 if MSG#1 with valid CMAC (using active CMAC key) is received (reset resend counter).

  - Discard MSG#3 received with invalid CMAC (using newest CMAC key)

Authenticated Reentry Authentication Wait: In this state the Authorization Authentication FSM has the context of the target ABS. The AMS should have the PMK context of the target ABS in this state before it sends an AAI_RNG-REQ message with CMAC during HO or reentry from coverage loss:

  - Caches AK context of all TBSs until HO Reentry completed or HO canceled.

  - Create new context and key hierarchy for the TBS whenever TBS changes (if context is not cached).

  - Maintain AK_COUNT Lock state

Authenticated Idle: In this state the Authorization Authentication FSM caches the PMK context and derives the appropriate key hierarchy for the TBS in case of re-entry/Location update.

16.2.5.2.5.2 Messages

PKMv3 Key Agreement MSG #1: The first message of Key Agreement. It is sent from the ABS to the AMS after EAP-based authentication has finished or once the ABS decides to renew the KEY context for PMK and derived keys and it is protected by CMAC using CMAC_KEY_D of the active Key context if there is one (it is not protected for initial key agreement).

PKMv3 Key Agreement MSG #2: The second message Key Agreement. It is sent from the AMS to the ABS as a response to a valid PKMv3 Key Agreement MSG #1, it is protected by CMAC using CMAC_KEY_U of the newest EAP-based authentication (same as active if only key agreement happens or new MSK in case of full EAP re-auth)

PKMv3 Key Agreement MSG #3: The last message of Key Agreement. It is sent from the ABS to the AMS as a response to a valid PKMv3 Key Agreement MSG #2 and it is protected by CMAC-Digest using CMAC_KEY_D of the newest EAP-based authentication.

PKMv3 refresh-PMK: The message used by the AMS to request the ABS to renew all the key hierarchy (PMK and derivatives) either by initiating full EAP-based re-authentication or just new key agreement. If new key agreement is not completed within fresh key agreement timer (TBD), the AMS may re-send refresh-PMK.

PKMv3 EAP Transfer: This message is bidirectional and used for transmission of EAP packet. This message is sent unprotected in “Not Authenticated” state. In Authenticated Active state, the message SHALL be encrypted

16.2.5.2.5.3 Events

Start Authentication: After completion of basic capabilities negotiation, this event is generated to start the Authentication state machine. It is also issued when the HO Process Optimization Bit #1 of the AAI_RNG-RSP message is set to zero one (i.e. 'omit PKM authentication phase') during HO or network reentry.

EAP Success: EAP FSM generates this event to notify the Authorization Authentication FSM that it received EAP Success message from the authenticator.

Key Agreement Timeout: This event is generated when the AMS does not receive PKMv3 Key Agreement MSG #3 from the ABS within Key Agreement Timer after transmitting a PKMv3 Key Agreement MSG #2. The AMS resends the PKMv3 Key Agreement MSG #2 up to Key Agreement MSG#2 Max Resends times.

Key Agreement MSG #2 max resends elapsed: The Authorization Authentication state machine generates this event when the AMS has transmitted the PKMv3 Key Agreement MSG #2 up to Key Agreement MSG #2 Max Resends times and Key Agreement Timer expires.

Key context refresh Needed: An internal event to trigger a message to the ABS requesting for a new key agreement with/out re-authentication per ABS decision. This event can be derived from several sources such as Authorization Grace Timeout or other reason that makes authentication close to expiration.

Start HO Reentry: An event to inform the Authorization Authentication FSM that AMS is in reentry phase. The FSM should derive the new AK context for the target ABS.

refresh-PMK Timeout: A timer event that causes the AMS to resend a PKMv3 refresh-PMK message in order to ask the ABS to refresh the key hierarchy from PMK and down. This event is used in the case that key agreement is not completed successfully during refresh-PMK timer from transmitting the PKMv3 refresh-PMK message. This timer is active only after key context refresh needed event occurred.

Reentry Completed: An event to notify the Authentication FSM that reentry has finished successfully. This event is issued when the AMS receives an AAI_RNG-RSP message including HO Process Optimization Bit #1 and Bit #2 set to one and zero respectively (i.e. 'omit PKM authentication phase') during HO or network re-entry from Idle mode

HO Canceled: An event to notify the Authorization Authentication FSM that HO was canceled. The cached AK context for the serving ABS should be retrieved.

TBS (Target ABS) changed: An Event to notify the Authorization Authentication FSM that it needs to generate the AK context for the new target ABS.

Authentication Expired: This event indicates the PMKAK context became obsolete due to the expiration of PMKAK lifetime one of the derived Keys that can't be refreshed seperatelly.

EAP Failure: This event indicates EAP-failure has been received from the NW.

External Stop: The event to stop the Authorization Authentication FSM and terminate connection with ABS.

[Auth] TEK update finished: The event received from TEK FSM to notify the Auth FSM that PMK context of old PMK can be discarded

NOTE-The following events are sent by an authentication state machine to the EAP state machine:

[EAP] key agreement completed: sent to the EAP FSM once key agreement is completed which means that new authentication is valid and old keys may be discarded.

NOTE-The following events are sent by an Authorization Authentication state machine to the TEK state machine:

[TEK] Stop: Sent by the Authorization Authentication FSM to an active (non- STOP START state) TEK FSM to terminate the FSM and remove the corresponding SAID’s keying material from the AMS’s key table.

[TEK] Start: Sent by the Authorization Authentication FSM to a nonactive (STOP START state), but valid TEK FSM.

[TEK] Authorization Pending (Auth Pend): Sent by the Authorization FSM to a specific TEK FSM to place that TEK FSM in a wait state until the Authorization FSM can complete its reauthorization operation. This event shall be sent to the TEK FSM in the Operational Wait (Op Wait) or Rekey Wait states when Authorization FSM starts re-authentication.

[TEK] Authorization Complete (Auth Comp): Sent by the Authorization FSM to a TEK FSM in the Operational Reauthorize Wait (Op Reauth Wait) or Rekey Reauthorize Wait (Rekey Reauth Wait) states to clear the wait state begun by a TEK FSM Authorization Pending event. This event shall be sent to the TEK FSM in the Operational Reauthorize Wait or Rekey Reauthorize Wait states when Authorization FSM ends re-authentication.

[TEK] Key agreement finished: sent from Auth FSM after re-auth key agreement finished to trigger TEK FSM to renew both TEKs from new AK.

16.2.5.2.5.4 Parameters

Key agreement Timer: The timer which expires if the AMS does not receive a PKMv3 Key Agreement MSG #3 after sending a PKMv3 Key Agreement MSG #1.

Refresh-PMK timer EAPStart Timeout: Timeout period between sending PKMv3 Refresh-PMK EAP Start messages from Authenticated active state.

16.2.5.2.55 Actions

Actions taken in association with state transitions are listed by <Start State> (<rcvd message>) --> <End state>:

1-A: Stopped (Start Auth) −> Not Authenticated

a) Enable PKMv3 EAP-Transfer messages to be transferred.

1-F: Authenticated Reentry Authentication Wait (Start Auth) -> Not Authenticated

a) Stop TEK FSMs

b) Re-initialize the Authentication FSM

c) Enable PKMv3 EAP-Transfer messages to be transferred.

1-G: Authenticated Idle (Start Auth) -> Not Authenticated

a) Stop TEK FSMs

b) Re-initialize the Authentication FSM

c) Enable PKMv3 EAP-Transfer messages to be transferred.

2-B: Not authenticated (Key Agreement MSG#1) -> Key Agreement MSG#3 Wait

a) Obtain MSK from EAP FSM.

b) Derive all Key hierarchy (PMK, AK, CMAC key, TEK),

c) Send Key Agreement MSG#2 with CMAC

d) Start Key Agreement Timer

2-C: Key Agreement MSG#3 Wait (Key Agreement MSG#1) -> Key Agreement MSG#3 Wait

a) Send Key Agreement MSG#2

b) Start Key Agreement Timer.

2-D: Authenticated Active (Key Agreement MSG#1) -> Refresh Key Agreement MSG#3 Wait

a) Obtain MSK from EAP FSM .

b) Derive all Key hierarchy (PMK, AK, CMAC key, TEK),

c) Send Key Agreement MSG#2 with CMAC

d) Start Key Agreement Timer

2-E: Refresh Key Agreement MSG#3 Wait (Key Agreement MSG#1) -> Refresh Key Agreement MSG#3 Wait

a) Send Key Agreement MSG#2

b) Start Key Agreement Timer.

3-C: Key Agreement MSG#3 Wait (Key Agreement MSG#3) -> Authenticated Active

a) Stop Key Agreement Timer

b) Start TEK FSM per negotiated SA

c) Start Authentication Grace Timer

d) Notify EAP FSM that authentication was completed

3-E: Refresh Key Agreement MSG#3 Wait (Key Agreement MSG#3) -> Authenticated Active

a) Stop Key Agreement Timer

b) Trigger TEK FSMs to update TEK to new AK

c) Start Authentication Grace Timer

d) Notify EAP FSM about authentication completion.

e) Notify TEK FSMs about key agreement finish so they will be aboe to obtain TEKs from new AK

4-B: Not authenticated (EAP Success) -> Not authenticated

a) Obtain MSK

4-D: Authenticated Active (EAP Success) -> Authenticated Active

a) Obtain MSK

5-C: Key Agreement MSG#3 Wait (Key Agreement Timeout) -> Key Agreement MSG#3 Wait

a) Send Key Agreement MSG#2

b) Start Key Agreement Timer .

5-E: Refresh Key Agreement MSG#3 Wait (Key Agreement Timeout) -> Refresh Key Agreement MSG#3 Wait

a) Send Key Agreement MSG#2

b) Start Key Agreement Timer .

6-C: Key Agreement MSG#3 Wait (Key Agreement MSG #2 max resend elapsed) -> Not authenticated

6-E: Refresh Key Agreement MSG#3 Wait (Key Agreement MSG #2 max resend elapsed) -> Authenticated Active

.

7-D: Authenticated Active (Key context refresh needed) -> Authenticated Active

a) Send refresh-PMK Message

b) Start refresh-PMK Timer

8-D: Authenticated Active (Start HO Reentry) -> Authenticated Reentry Authentication Wait WAit

a) Generate AK Context and all derived keys for Target ABS

b) Enter AK_COUNT LOCK counter lock state

8-E: Refresh Key Agreement MSG#3 Wait (Start HO Reentry) -> Authenticated Reentry Authentication Wait WAit

a) Generate AK Context and all derived keys for Target ABS

b) Enter AK_COUNT LOCK counter lock state

c) Remove all refresh key agreement created context

9-D: Authenticated Active (refresh-PMK Timeout) -> Authenticated Active

a) Send refresh-PMK Message

b) Start refresh-PMK Timer

10-F: Authenticated Reentry Authentication Wait (HO canceled) -> Authenticated Active

a) Remove AK context of all Target ABS

b) Retrieve AK context of Serving ABS

c) Update PMK context with AK key counter value

d) Exit AK counter lock state

11-F: Authenticated Reentry Authentication Wait (HO canceled) -> Authenticated Reentry Authentication Wait

a) Cache AK context of former Target ABS

b) Retrieve or generate if not cached AK context of new Target ABS

12-F: Authenticated Reentry Authentication Wait (Reentry Completed) -> Authenticated Active

a) mark AK context of last Target ABS as Serving ABS

b) Delete AK context of all cached Target ABSs

c) Update PMK context with AK_COUNT key counter value

d) Exit AK_COUNT counter lock LOCK state

13-D,E,F: Any state with valid authentication  (Authentication expired) -> Stopped

a) Stop TEK FSMs

b) Delete all authentication context

c) Stop authentication FSM

14-D: Authenticated Active (EAP Failure) -> Authenticated Active

15-B,C,D,E,F,G: Any state (External stop) -> Stopped

a) Stop TEK FSMs if active

b) Delete all authentication context

c) Stop authentication FSM

16-D: Authenticated Active (Exit to Idle) -> Authenticated Idle

a) Stop Freeze TEK FSM

16-E: Refresh key agreement MSG#3 wait (Exit to Idle) -> Authenticated Idle

a) Stop Freeze TEK FSM

17-G: Authenticated idle (Re-entry from idle) -> Authenticated Active

a) Update AK context with AK_COUNT counter

b) Notify PMK context about AK_COUNT counter updated value

c) Derive AK context and all sub keys

d) Start Unfreeze TEK FSM

18-G: Authenticated idle (Secure location update) -> Authenticated idle

a) Update AK context with AK_COUNT counter

b) Notify PMK context about AK_COUNT counter updated value

c) Derive AK context and all sub keys

19-D: Authenticated active (TEK update finished) -> Authenticated Active

a) If all TEK FSMs reported TEK update finisheddelete old PMK context

--------------------------------text end-----------------------------------------