| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on zone switching protocol (16.2.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Xiangying Yang Muthaiah Venkatachalam Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.6” |  |
| Abstract | This contribution proposes text for enhanced zone switching procedure to provide possibility of seamless zone switching |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text  proposal on zone switching protocol (16.2.6)

Xiangying Yang, Muthaiah Venkatachalam

Intel

- Introduction

The text change propose to allow AMS and ABS to prepare zone switching in the serving zone with required target zone reentry signaling, which is carried by layer-2 transfer message. This will significantly reduce the zone switching latency, for which many reentry steps can not be omitted.

- Proposed Test

I. Modify Section 16.2.6.4.1.2.1

--------------------------------text start-----------------------------------------

The AMS shall also perform capability negotiation during network reentry in MZone through the exchange of AAI_REG-REQ/RSP message. The AMS shall also perform key agreement and security update according to the specification in section 16.2.5.2.1.5.6. AMS context mapping from LZone to MZone is performed by the ABS per section 16.2.6.4.2.3.

The capability negotiation and security update may be carried out prior to zone switching, using L2-Xfer message with transfer type 8. In the case of capability negotitation and security update completed before zone switching, ABS set HO optimization bit#1 to be one and AMS may resume data communication with ABS in MZone during reentry into MZone.

--------------------------------text end-----------------------------------------

II. Modify section 16.2.3.28 and section 16.2.5.1.5.6/7 as follows

--------------------------------text start-----------------------------------------

16.2.3.28 L2 Transfer message (AAI_L2_XFER)

The AAI provides a generic MAC control message called AAI_L2_XFER. This message acts as a generic service carrier for various services including, but not limited to:

Device provisioning bootstrap message to AMS, GPS assistance delivery to AMS, ABS(es) geo-location unicast delivery to AMS, 802.21 MIH transfer, messaging service, conveying WirelessMAN-OFDMA Advanced MAC control messages to AMS in LZone etc.

This container is also used for 16m messages that are not processed by the BS, but are rather processed by network entities beyond the BS. AAI_L2_XFER shall be transmitted only by authenticated AMSs.

The format of AAI_L2_XFER message is shown in Table 2

The enumeration of Tranfer-Type is as follows:

a)Transfer-Type = 1; GNSS assistance (DL)

b)Transfer-Type = 2; LBS measurement [Terrestrial meas. and GNSS pseudo ranges] (UL)

c)Transfer-Type = 3; Device Bootstrap (DL/UL)

d)Transfer-Type = 4; WirelessMAN-OFDMA network boundary indication (DL)

e)Transfer-Type = 5; ORAT-MSG (DL)

a)Sub-Type = 1: GERAN (GSM/GPRS/EGPRS)

b) Sub-Type = 2: UTRAN

c) Sub-Type = 3: E-UTRAN

d) Sub-Type = 4: TDSCDMA

e) Sub-Type = 5: CDMA2000

f)Transfer-Type = 6: SMS

a)Sub-Type = SMS data

b)Sub-Type = SMS confirmation

g)Transfer-Type = 7: MIH Frame

i)Sub-Type =1 : ES/CS MIH Capability Discovery

ii)Sub-Type =2 : Event Service

iii)Sub-Type = 3 : Command Service

iv)Sub-Type = 4 : Information Service

h)Transfer-Type = 8: WirelessMAN-OFDMA Advanced MAC control messages (only used in LZone)

ih)Transfer-Type = 89-127; reserved

ji)Transfer-Type = 128-255; Vendor specific types

Some of these messages have sub-types that are further defined in the type specific message payload. For example, for Transfer-Type=1, the GNSS assistance may be for GPS, Galileo or other satellite systems, which would be specified as sub-types of Transfer-Type=1.