| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed E-MBS Configuration Message (16.9.3.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Chang-Lan Tsai, Yan-Xiu Zheng, Chun-Yuan Chou, Yu-Chuan Fang, Chung-Lien Ho ITRI | E-mail: tsaichangl@itri.org.tw zhengyanxiu@itri.org.tw ccy@itri.org.tw |
| Re: | IEEE 802.16-09/0073: IEEE 802.16 Working Group Letter Ballot Recirc #30b Section 16.9.3.1 E-MBS Configuraiton Indicators |  |
| Abstract | This contribution provides detailed format of the E-MBS Configuration Message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed E-MBS Configuration Message (16.9.3.1)

Chang-Lan Tsai, Yan-Xiu Zheng, Chun-Yuan Chou, Yu-Chuan Fang, Chung-Lien Ho
ITRI

# Introduction

This contribution adds the subband and subframe indicators for the allocation of E-MBS region. E-MBS_ SUBFRAME_INDICATOR indicates the subframes carrying E-MBS traffic in each frame. For each E-MBS zone, E-MBS_SUBBAND_INDICATOR indicates the location and size allocated to the E-MBS zone. E-MBS MAP is located at the first several LRUs in the region for one E-MBS zone. E-MBS MAP Resource Index indicates the number of LRUs carrying the E-MBS MAP.

# Proposed Text

Modify Table 944 as the following.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.9.3.1 E-MBS Configuration Indicators

...

Table 944—AAI-E-MBS_CFG message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI-E-MBS_CFG_Message_Format() { |  |  |
| Num_E-MBS_Zones | 3 | Number of E-MBS Zones with which the ABS is associated |
| for(i=0; i< Num_E-MBS_Zones, i++) { |  |  |
| E-MBS_Zone_ID | 7 | The E-MBS_Zone_ID to which this EMBS MAP applies |
| MSI | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms 0b01: 4 superframes, 80 ms 0b10: 8 superframes, 160 ms 0b11: 16 superframes, 320 ms |
| E-MBS MAP Resource Index | [TBD] | Resource index includes location and allocation size. |
| E-MBS MAP ISizeOffsetSizeOffset | 5 |  |
| E-MBS_SUBBAND_INDICATOR | variable | Indicate the beginning and the number of subbands for E-MBS zone in E-MBS region. The size depends on the FFT size: 2048-FFT: 10 bits 1024-FFT: 8 bits 512-FFT: 6 bits |
| } |  |  |
| E-MBS_SUBFRAME_INDICATOR | 3 | The subframes for E-MBS zone in the E-MBS region 0b000: the last subframe in DL zone 0b001: the last 2 subframes in DL zone 0b010: the last 3 subframes in DL zone 0b011: the last 4 subframes ins DL zone 0b100: the last 5 subframes in DL zone 0b101: the last 6 subframes in DL zone 0b110: the last 7 subframes in DL zone 0b111: the last 8 subframes in DL zone |
| } |  |  |

E-MBS_SUBBAND_INDICATOR: Indicate the subbands allocated for each E-MBS zone. The size (bit) depends on the FFT size. The bit field consists on m bits, where m = 10, 8 or 6, denoted as b0b1…bm-2bm-1. The first m/2 bits represents the beginning subband index for E-MBS zone. The last m/2 bits represents the number of subbands allocated to the E-MBS zone.

E-MBS MAP Resource Index: E-MBS MSP is allocated at the first several LRUs in the region for the E-MBS zone. E-MBS MAP Resource Index indicates the allocation size of E-MBS MAP in the unit of LRUs.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------