| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Control of CSG Femto ABS by Using LDM (16.4.11) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yung-Han Chen, Zheng YanXiu, Chun-Yuan Chiu ITRI | chenyunghan@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b |  |
| Abstract | The received SFH of a non-member AMS may be severely interfered by a nearby CSG femto ABS. This contribution provides a method to mitigate the interference by using low duty mode. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Control of CSG Femto ABS by Using LDM (16.4.11)

Yung-Han Chen, Zheng Yan-Xiu, Chun-Yuan Chiu

ITRI

1. Introduction

A CSG femto ABS does not allow non-CSG-member AMS to access. If an AMS served by a Macro ABS is near to the CSG femto ABS, its received SFH may be severely interfered by the SFH transmitted by the CSG femto ABS (as mentioned in C80216m-09/2596r2).

Figure 1. Scenario of interference on SFH

The interfered non-CSG-member AMS may report interference or send an IM-purpose message to the interfering CSG femto ABS directly, through serving macro ABS or other channels. Once the CSG femto ABS receives the IM request and authenticates the requester (the interfered non-member AMS), it may start the setup and operation of low duty mode (LDM) for interference mitigation. The LDM setup includes available/unavailable interval settings, LDM start superframe number, and the LDM duration for IM. If the CSG femto ABS receives other IM-purpose message during LDM for IM, it may perform LDM setup again to adjust the LDM for IM.

2. Text Proposal

---------------------------------------------------------- Text Start ----------------------------------------------------------------

{ Insert following blue texts in line 49, page 603}

16.4.11 Interference Avoidance and Interference Mitigation

A CSG Femto ABS may operate in LDM for interference mitigation. In this case, the CSG Femto ABS shall set LDM including the available/unavailable intervals, start superframe number of LDM, and duration of LDM for IM. If the CSG Femto ABS receives any other IM-purpose message from authenticated AMS, it may adjust the setting of LDM to compromise all IM requests.

---------------------------------------------------------------Text End ------------------------------------------------------------