| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SFH Forwarding by a CSG Femto ABS for Interference Control (16.4.11) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yung-Han Chen, Zheng YanXiu, Chun-Yuan Chiu ITRI | chenyunghan@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b |  |
| Abstract | The received SFH of a non-member, macro-served AMS may be severely interfered by a nearby CSG femto ABS. This contribution provides a method that the interfering CSG femto ABS forwards the SFH of macro ABS to the AMS. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SFH Forwarding by a CSG Femto ABS for Interference Control (16.4.11)

Yung-Han Chen, Zheng Yan-Xiu, Chun-Yuan Chiu

ITRI

1. Introduction

A CSG femto ABS does not allow non-CSG-member AMS to access. If an AMS served by a Macro ABS is near to the CSG femto ABS, its received SFH may be severely interfered by the SFH transmitted by the CSG femto ABS (as mentioned in C80216m-09/2596r2).

Figure 1 – Scenario of interference on SFH

The interfered non-CSG-member AMS may report interference or send an IM-purpose message to the interfering CSG femto ABS directly, through serving macro ABS or other channels. Once the CSG femto ABS receives the IM request and authenticates the requester (the interfered non-member AMS), it may decide and prepare to forward the SFHs of the macro ABS to the interfered macro-served AMS. The CSG femto ABS may request the macro ABS to provide SHF which will be issued in the following superframe(s). The CSG femto ABS should negotiate with the macro ABS on the number of SFH to forward or the duration to support SFH forwarding. Upon obtaining the SFH from the macro ABS, the CSG femto ABS may multiplex it with its own SFH by using signal super-positioning with configurable power weighting (see Figure 2), and transmit them together. The AMSs can distinguish each SFH according to different permutation rules used by the super-positioned SFHs.

Figure 2 – SFH multiplexing by using signal super-positioning

2. Text Proposal

---------------------------------------------------------- Text Start ----------------------------------------------------------------

{ Insert following blue texts in line 50, page 603}

16.4.11 Interference Avoidance and Interference Mitigation

A CSG Femto ABS may forward the SFHs of the macro ABS to the interfered macro-served AMS. To achieve SFH forwarding, the CSG femto ABS shall request the macro ABS to provide SHF which will be issued in the following superframes. The CSG femto ABS should negotiate with the macro ABS on the number of SFH to forward. Upon obtaining the SFH from the macro ABS, the CSG femto ABS may multiplex it with its own SFH by using signal super-positioning with configurable power weighting, and transmit them together. The AMSs can distinguish SFHs in the super-positioned signal based on different permutation rules used by the SFHs..

---------------------------------------------------------------Text End ------------------------------------------------------------