| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | D3 Text Proposal for MultiBS MIMO operation |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Shuangfeng Han, David Mazzarese, Sangwoo Lee, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Wookbong Lee LG Electronics Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI | shuangf.han@samsung.com wookbong.lee@lge.com pinghengkuo@itri.org.tw |
| Re: | IEEE 802.16-09/0073: IEEE 802.16 Working Group Letter Ballot Recirc #30b announcement |  |
| Abstract | The contribution proposes some text change to multiBS MIMO section of 802.16m D3. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

D3 Text Proposal for MultiBS MIMO operation

Shuangfeng Han, David Mazzarese, Sangwoo Lee, Hokyu Choi, Heewon Kang

Samsung Electronics

Wookbong Lee

LG Electronics

Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

- Introduction

This contribution includes proposed text to provide MAC support for Multi-BS MIMO in the 802.16m D3 document.

- Motivation of modifications

To perform Downlink Multi-BS MIMO, an AMS is required to know certain parameters, such as temporary BSID of ABSs that take part in downlink multi-BS operation. In the current D3 document, Diversity set NUM, change count and BS index are broadcast in the AAI_DL_IM message. 4 bits are allocated to diversity set NUM, indicating at most 16 adjacent ABS for multi-BS MIMO operation. However, the above parameters are only for single cell processing with multiBS coordination. For multiBS joint processing, we also need corresponding parameters.

Proposed operation procedure for multi-BS joint processing:

- ABS broadcast Temp_BSID through AAI_DL_IM Message

- AMS measures NIP for each base station and see if it meets triggering mechanism

- If it meets triggering mechanism, AMS sends AAI_MULTI_BS_MIMO-REQ with Temp_BSID and NIP value for base stations which has higher NIP than threshold.

- ABS indicates which base station to coordinate in case of multi-BS joint processing (Co-MIMO and CL-MD)

- ABS allocates feedback with Feedback Polling A-MAP IE

- AMS feedback multi-BS MIMO CQI, PMI, CPMI through AAI_MultiBS_MIMO_FBK_Message

- Proposed texts:

Remedy #1: To delete Temp_BSID in Table 706 in case of Co-MIMO and CL-MD

--------------------------------------------Text Start -------------------------------------------------------

- Table 706—AAI_MultiBS_MIMO_FBK message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_MultiBS_MIMO_FBK_Messag e_Format { |  |  |
| MAC Control Message Type = xx |  |  |
| if (ICT ==0b10 or 0b11){ |  | Co-MIMO or CL-MD |
| CQI | 4 |  |
| } |  |  |
| for (i = 1; i <= N_multiBS_reports; i++) { |  |  |
| if (ICT == 0b00 or 0b01){ |  |  |
| Temp_BSID | 4 | Diversity set member ID |
| } |  |  |
| PMI | Variable | PMI from the rank-1 base codebook or base codebook subset Nt=2: 3 bits Nt=4: 4 bits Nt=8: 4 bits |
| If (ICT==0b00 or 0b01) { |  |  |

--------------------------------------------------- Text End -------------------------------------------------

Remedy #2: To modify the last paragraph on page 607

------------------------------------------------- Text Start --------------------------------------------------------

In a multi-BS joint processing mode (i.e. CL-MD mode or Co-MIMO), an ABS shall inform the corresponding AMS of the set of the ABSsparticipating in joint processing. The IDs of the joint processing ABSs should be transmitted from the serving ABS to the AMS by using the following message format shown in Table 936.

For multi-BS MIMO joint processing mode (i.e. CL-MD or Co-MIMO), an ABS shall inform the corresponding AMS of the set of ABSs participating in multi-BS MIMO using the AAI_MULTI_BS_MIMO-RSP control message.

--------------------------------------------------- Text End ----------------------------------------------------------

Remedy #3: To delete Table 936-Parameters to be transferred from serving ABSs to AMSs

Remedy #4: To add new message in section 16.2.3 AAI_MULTI_BS_MIMO-RSP

-------------------------------------------------- Text Start -------------------------------------------------------

16.2.3.xx AAI_MULTI_BS_MIMO-RSP

The AAI_MULTI_BS_MIMO-RSP message shall be transmitted by the ABS to AMS indicating which of the adjacent ABSs listed in previously broadcasted AAI_NBR-Adv message are involved in multiBS MIMO joint processing operation.

---ASN.1START

AAIMULTIBSMIMORSP Message::SEQUENCE{

Bitmap for involved adjacent ABSs in multiBS MIMO   BIT STRING (SIZE (8))

}

Table 708-AAI_MULTI_BS_MIMO-RSP

| Name | Size (bit) | Usage |
| --- | --- | --- |
| Bitmap_for_involved_adjacent_ABSs _in_multiBS_MIMO | 8 | 0: ABS is not involved in multiBS MIMO 1: ABS is involved in multiBS MIMO Each bit in this bitmap represents one ABS and the i-th bit (i=1,…,8) represents the neighboring ABS (listed in AAI_NBR-Adv message) with the i-th strongest channel to the AMS. |

--------------------------------------------------- Text End ----------------------------------------------------------

Remedy #5: To modify section 16.2.3.36

-------------------------------------------------- Text Start ---------------------------------------------------

16.2.3.36 AAI_MULTI_BS_MIMO-REQ

The AAI_MULTI_BS_MIMO-REQ message shall be transmitted by the AMS to report its preference on single BS precoding with Multi-BS MIMO coordination or multi-BS joint MIMO processing.

---ASN.1START

AAIMULTIBSMIMOREQ Message::SEQUENCE{

Multi-BS MIMO Request    Boolean,

NumBS	INTEGER (1..8)

NIPList::=SEQUENCE (SIZE (1..NumBS)) OF NIPParam -- NIPParam shall be transmitted from strongest to weakest

NIP Value   ENUMERATED {n0, n1, n2, n3}

}

NIPParam::=SEQUENCE{

Temp_BSID   INTEGER (0..15),

NIP Value   INTEGER (0..15)

}

--ASN1STOP

Table 707—AAI_MULTI_BS_MIMO-REQ message format

| Name | Value | Usage |
| --- | --- | --- |
| Multi-BS MIMO Request |  | 0: Single-BS precoding with Multi- BS Coordination 1: Multi-BS Joint MIMO Processing |
| NIP Value |  | Absolute NIP value encoded as difference to NIP_th_1 when Multi- BS MIMO Request = 0. Absolute NIP value encoded as difference to NIP_th_2 when Multi- BS MIMO Request = 1. The values are interpreted in the range [TBD]. |
| NumBS | 3 bits | Number of base station which exceeds threshold |
| Temp_BSID | 4 bits | Temp_BSID is broadcasted through AAI_DL_IM message |

------------------------------------------------- Text End---------------------------------------------------------