| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Updates to Support 4 bit HARQ Channel Indexing in the UL Assignment A-MAP IEs in IEEE P802.16m/D3 (16.3.6.5.2.4.2) |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Roshni Srinivasan, Yi Hsuan, Hujun Yin, Alexei Davydov, Huaning Niu Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | Comments on IEEE P802.16m/D3 for IEEE 802.16 WG Letter Ballot Recirc #30b |  |
| Abstract | The contribution proposes changes to the UL Basic Assignment A-MAP IE and UL Persistent Allocation A-MAP IE in IEEE P802.16m/D3 (16.3.6.5.2.4.2 and 16.3.6.5.2.4.9) |  |
| Purpose | To be discussed and adopted in TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Updates to Support 4 bit HARQ Channel Indexing in the UL Assignment A-MAP IEs in IEEE P802.16m/D3 (16.3.6.5.2.4.2)

Roshni Srinivasan, Yi Hsuan, Hujun Yin, Alexei Davydov, Huaning Niu

Intel Corporation

Introduction

The ACID field in all UL A-MAP IEs used for resource assignment identifies the HARQ channel assigned to the allocated burst. The length of the ACID field is currently set to 3 bits [1]. It has been shown in [2] that 3 bits are insufficient to identify all 16 UL HARQ channels without ambiguity in some situations.

The proposal in [2] to increase the size of the ACID field from 3 to 4 bits to explicitly address all available HARQ channels on the UL is supported in this contribution. However, instead of using the reserved bit in the IE to increase the length of the ACID field, it is proposed that all supported MIMO transmission formats are not signaled in the UL basic assignment IE. By defining a new Assignment A-MAP IE for the special case of UL collaborative spatial multiplexing with beamforming, the number of bits required for signaling MIMO transmission formats is reduced by one. The additional bit can be now assigned to the ACID and the reserved bit is still conserved for future extension.

It is also proposed that the size of the ACID field in the UL Persistent Allocation A-MAP IE be increased from 3 to 4 bits by utilizing 1 of the 2 reserved bits.

References

[1] P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Advanced Air Interface”, December 2009.

[2] C802.16m-09/2227, Ji-Yun Seol, “UL HARQ Channel Indication in IEEE 802.16m/D2”, November 2009.

Instructions to Editor

- Table 813 appears on Page 400, starting on line 26. Text to be deleted has been struck out and marked with red font. Text to be added has been underlined in blue font.

- A new subclause has been added for the UL CSM beamforming A-MAP IE and table 814 has been added under the new subclause to define this IE.

- The size of the ACID field in the UL Persistent Allocation A-MAP IE has been changed from 3 bits to 4 bits

- The number of Reserved bits in the UL Persistent Allocation A-MAP IE from 2 bits to 1 bit

____________________________Begin proposed text change #1 _________________________________

< Table 813 appears on Page 400, starting on line 26. Adopt the proposed changes as shown.>

# 16.3.6.5.2.4.2 UL basic assignment A-MAP IE

Table 813 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 813 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| CSM-PMI Indicator | 1 | Flag to indicate if both CSM and PMI are signaled 0b0: SU-MIMO with or without PMI indication , or CSM without PMI indication 0b1: CSM with PMI indication |
| if (CSM-PMI Indicator == 0b0){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled. 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b0){ |  |  |
| Reserved | 54 | Reserved bits |
| else if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 3 | Reserved bits |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| }else if (CSM-PMI Indicator == 0b1){ |  | Parameters for CSM with PMI (adaptive precoding) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 34 | HARQ channel identifier |
| Reserved | 12 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

- TNS: Total number of streams in the LRU for CSM

- SI: First pilot index for CSM.

PF: Precoding flag to indicate adaptive or non-adsptive precoding.

PMI: Precoding matrix index .

CSM-PMI Indicator: Flag to indicate if both CSM and PMI are signaled

SU-PMI Indicator: Flag to indicate if both SU-MIMO and PMI are signaled

CSM: Flag to indicate if CSM is signaled

- The Resource Index field in the UL Basic Assignment A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE, with 'DL' specific terminology replaced by 'UL' equivalents.

# 16.3.6.5.2.4.3 UL CSM beamforming A-MAP IE

Table 814 describes the fields in a UL CSM Beamforming Assignment A-MAP IE.

Table 814 UL CSM Beamforming A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_CSM_Beamforming_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL CSM_Beamforming A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 1 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL CSM Beamforming A-MAP IE. The CRC is masked by the Station ID.

________________________________End proposed text change #1 _________________________________

____________________________Begin proposed text change #2 _________________________________

< Table 824 appears on Page 432, starting on line 52. Adopt the proposed changes as shown.>

# 16.3.6.5.2.4.9 UL persistent allocation A-MAP IE

The UL persistent allocation A-MAP IE is specified in Table 824.

Table 824: UL Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent Allocation A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation |
| ACID | 34 | HARQ channel identifier N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| Reserved | 21 | Reserved bits |
| } | - | - |
| } | - | - |

________________________________End proposed text change #2 _________________________________