| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change to clean up key agreement procedure (16.2.5.2.1.4) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Samsung Electronics Avishay Shraga Xiangying Yang Intel | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.5.2.1.4” |  |
| Abstract | This contribution provides to clean up key agreement procedure |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change to clean up key agreement procedure (16.2.5.2.1.4)

Youngkyo Baek, Jicheol Lee

Samsung Electronics

Avishay Shraga, Xiangying Yang

Intel

# Introduction

This contribution suggests a clean-up text for key agreement procedure 16.2.5.2.1.4.

We suggest fixing up words keep consistency over the sections and adding some texts to cleanup the valification scheme during key agreement procedure.

# Proposed Text

Modify text in line41 page 135 as follows.

----------------------------------------------------- Start of Proposed Text#1 ---------------------------------------------------

The key agreement procedure (as shown in Figure 400) includes the following steps:

•AMS and NW complete EAP authentication completes(Authenticator got “EAP Success” from AAA and sent it to AMS).

•The ABS sends AAI_PKM_RSP (key agreement MSGmsg#1) to the AMS, after protecting it with the CMAC tuple if ABS already has an AK shared with the AMS., t The message includes a plaintext random NONCE_ABS.

•Receiving the key agreement MSG#1, t The AMS derives all security keys from the newest MSK, created by the last EAP authentication PMK, AMSID* and other parameters including AMSID*, the NONCE_ABS and a random NONCE_AMS as defined in 16.2.5.2.1.1 and sends AAI_PKM_REQ (key agreement MSGmsg #2) including the NONCE_ABS and a random NONCE_AMS to the ABS. The AAI_PKM_REQ is integrity protected (CMAC digest using the derived CMAC keys) but not encrypted.

If the AMS receives another MSG#1 before sending MSG#2 it shall ignore and discard it.

•Receiving the key agreement MSG#2, t The ABS takes the NONCE_AMS, calculates the keys and shall confirm that the supplied PMKID refers to the PMK that it has. If the PMKID is unrecognized, the ABS shall ignore the key agreement MSG#2. The ABS shall verify the CMAC.  verifies the CMAC it received based on the derived keys,iIf the CMAC is verified then the ABS knows it has the same keys which are bouind to the AMSID and ABSID, the keys are also fresh due to the 2 NONCE values in the derivation function. If the CMAC is invalid, the ABS shall ignore the key agreement MSG#2. The ABS shall verify that the NONCE_ABS in the key agreement MSG#2 matches the value provided by the ABS in the key agreement MSG#1. If the NONCE_ABS value does not match, the ABS shall ignore the key agreement MSG#2.

If the ABS does not receive the key agreement MSG#2 from the AMS within Key Agreement Timeout, it shall resend the previous key agreement MSG#1 up to KeyAgreementMSG#1MaxResends times. If the ABS reaches its maximum number of resends, it shall initiate another full authentication or drop the AMS.

•Upon successful validation of the key agreement MSG#2, tThe ABS then shall sends to the AMS AAI_PKM_RSP (key agreement MSGmsg #3) that includes the NONCE_AMS, NONCE_ABS, the supported SAIDs (0x1 or and 0x2 or both) and CMAC digest to prove the possession of the keys and their freshness.

•Receiving the key agreement MSG#3, t The AMS verifies the CMAC and derive the TEKs for the supported SAIDs. If the CMAC is invalid, the AMS shall ignore the key agreement MSG#3. The AMS shall verify that the NONCE_AMS in the key agreement MSG#3 matches the value provided by the AMS in the key agreement MSG#2. If the NONCE_AMS value does not match, the AMS shall ignore the key agreement MSG#3.

If the AMS does not receive key agreement MSG#3 from the ABS within Key Agreement Timeout, it shall resend the key agreement MSG#2. The AMS may resend the key agreement MSG#2 up to Key Agreement MSG#2 MaxResends times. If the AMS reaches its maximum number of resends, it shall initiate another full NW entry or attempt to connect to another ABS.

----------------------------------------------------- End of Proposed Text#1 ---------------------------------------------------

Modify table 713 in line 1 page 106 as follows.

----------------------------------------------------- Start of Proposed Text#2 ---------------------------------------------------

Table 713—PKMv3 Key_Agreement-MSG#2 message attributes

| Attribute | Contents |
| --- | --- |
| …… | ……. |
| NONCE_AMS | A freshly generated random number of 64 bits |
| PMKID | PMKID = Dot16KDF(PMK, 0b0000¦PMK SN¦NONCE_MS¦NONCE_BS¦ "PMKID", 64). This is used to verify sync of PMK SN(or AK SN) to PMK |
| …… | ……. |

----------------------------------------------------- End of Proposed Text#2 ---------------------------------------------------