| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification Text for Basic Frame Structure in P802.16m/D3 (16.3.3) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting ITRI | E-mail: ythsieh@itri.org.tw |
| Re: | IEEE 802.16-09/0073: IEEE 802.16 Working Group Letter Ballot Recirc #30b announcement |  |
| Abstract | This contribution proposed modification text on basic frame structure |  |
| Purpose | Discussion and adoption by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification Text for Basic Frame Structure in P802.16m/D3 (16.3.3)

Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting

ITRI

Introduction

In the current D3, the general description for the AAI subframe number for G= 1/4 does not apply in the case of 7 MHz, as shown in Table 773. We suggest enumerating the subframe numbers for each bandwidth and CP size in order not to make confusion. The detailed proposed text is as follows.

Proposed Text

Modify the first paragraph of section 16.3.3.1, on page 295, line 52 as follows,

---------------------------------------------------Text Start------------------------------------------------------------------------

The advanced air interface basic frame structure is illustrated in Figure 461. Each 20 ms superframe is divided into four equally-sized 5 ms radio frames. When using the same OFDMA parameters as in Table 766 with the channel bandwidth of 5 MHz, 10 MHz, or 20 MHz, each 5 ms radio frame further consists of eight, eight, and seven AAI subframes, respectively for G = 1/16, 1/8 and 1/41/8 and 1/16. With the channel bandwidth of 8.75 and 7 MHz, each 5 ms radio frame further consists of seven, seven and six AAI subframes, respectively for G = 1/16, 1/8 and 1/41/8 and 116. With the channel bandwidth of 7 MHz, each 5 ms radio frame further consists of six, five and five AAI subframes, respectively for G = 1/16, 1/8 and 1/4. In the case of G = 1/4, the number of AAI subframes per frame is one less than that of other CP lengths for each bandwidth case. A AAI subframe shall be assigned for either DL or UL transmission. There are four types of AAI subframes:

---------------------------------------------------Text End------------------------------------------------------------------------

References

[1] IEEE 802.16m/D3, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”