| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS identification in DCR mode (P802.16m/D3-16.2.18) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Lei Zhang Yang Liu Mary Chion Feng Xie ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB30b / Area: Chapter 16.2.18 (DCR mode) |  |
| Abstract | This contribution proposes changes to the Section 16.2.18 of IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS identification in DCR mode (P802.16m/D3-16.2.18)

Lei Zhang, Yang Liu, Mary Chion, Feng Xie

ZTE Corporation

- Introduction

In P802.16m/D3, DCR (Deregistration with content retention) mode is proposed to expedite network re-entry when AMS goes into hibernation mode or handover to other RAT. However, the description of DCR mode is quite unclear. For instance, as stated in D3, DID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs, what is the exact network entity be used to identify the AMS? On the other hand, the length of DID is limited to 10 bit in 16m, this should be not enough to identify thousands of AMSs in the network.

This contribution proposes a method to uniquely identify AMS in DCR mode.

- Proposed text

======================== Start of Proposed Text ========================

[Inset the proposed subclause at line 17 on page 20 as follows.]

- 16.2.1.2.4 Retention Identifier (RID)

The Authenticator shall assign a 16 bit RID to each AMS during DCR mode initiation from connected state or idle mode. RID together with Authenticator ID that assigns/retains the context is used to uniquely identify the DCR mode AMSs. The RID shall uniquely identify the AMS in the domain of the Authenticator.

- 16.2.18 Deregistration with content retention (DCR) mode

[Modify the paragraph from lines 31 on page 268 of P802.16m/D3 as follows.]

…

While the idle mode retention timer is valid, the network retains AMS’s information which is used to expedite AMS’s network re-entry. Retention identifier (RID) Deregistration identifier (DID) is assigned to AMS during DCR mode initiation from connected state or idle mode. with paging cycle and paging offset value of 0. RID DID together with the Authenticator ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs.

- 16.2.18.1 DCR initiation in connected state

[Modify the paragraph from lines 49 on page 268 of P802.16m/D3 as follows.]

…

In the AAI-DREG-CMD, DIDRID shall be assigned which is used for network re-entry from DCR mode to identify the AMS.

…

- 16.2.18.2 DCR mode initiation from idle mode

[Modify sentences at line 65 on page 268 as follows.]

At the time of DCR mode change, paging cycle, paging offset, paging controller values shall be updated to be 0 for the AMS and the RID and the Authenticator ID of the network entity that assigns/retains the context shall be used to uniquely identify DCR mode AMS.

- 16.2.18.4 Network reentry from DCR mode

[Modify sentences at line 13 on page 269 as follows.]

For the network reentry from DCR mode, the AMS shall initiate network reentry with the ABS by sending an AAI_RNG-REQ message with ranging purpose code of 0x06 and the RID and the Authenticator ID of the network entity that assigns/retains the context. Rest of reentry procedure shall be performed same as network reentry from idle mode described in section 16.2.17.5.

- 16.2.3.1 AAI_RNG-REQ

[Update Table-674 on page 45 of P802.16m/D3 as follows.]

Table 674—Parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform HO reentry |
| Authenticator ID | The ID of anchor Authenticator which retains the AMS’s context. | It shall be included when the AMS is attempting to perform DCR mode extension or network reentry from DCR mode. |
| Retention Identifier (RID) | The ID together with the Authenticator ID that assigns/retains the context is used to uniquely identify the DCR mode AMSs. |  |

- 16.2.3.22 AAI_DREG-RSP message

[Modify the paragraph from lines 5 on page 83 of P802.16m/D3 as follows.]

When the action code is set to 0x098, the following parameters shall be included in AAI_DREG-RSP message.

Idle Mode Retain Information

Retention identifier (RID) Deregistration identifier (DID)

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D3. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Dec 2009.