| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Acknowledgement scheme of SCH/SCEH (P802.16/D3-16.2.16.2.3.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Lei Zhang Yang Liu Yuqin Chen Nan Li Mary Chion ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB30b / Area: Chapter 16.2.16.2.3.2 (Listening Window Extension) |  |
| Abstract | This contribution proposes changes to the Section 16.2.16.2.3.2 of IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Acknowledgement scheme of SCH/SCEH (P802.16/D3-16.2.16.2.3.2)

Lei Zhang, Yang Liu, Yuqin Chen, Nan Li, Mary Chion

ZTE Corporation

- Introduction

In D3, when explicit Listening Window extension is performed through MAC signaling/extended header (i.e. SCH or SCEH), the SCH and SCEH need to be ACKed to ensure the synchronization of the sleep state machines at the AMS and ABS. However, the definition of response indication in current SCH and SCEH format seems inefficiency.

- Proposed text

======================== Start of Proposed Text ========================

- 16.2.2.1.3.4 Sleep Control header (SCH)

[Modify table 660 (from line 55 on page 25) of P802.16m/D3 as follows]

Table 660—Sleep Control header Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Sleep Control Header Format { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type = 0b0011 |
| SCH sub-type | 2 | 0b00 = Listening Window control 0b01 = Resume Sleep Cycle Indication 0b10 = Sleep cycle control 0b11 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request |
| … | … | … |

- 16.2.2.2.5 Sleep Control extended header (SCEH)

[Modify table 668 (from line 39 on page 33) of P802.16m/D3 as follows]

Table 668—SCEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| SCEH () { |  |  |
| Type | 4 | SCEH type |
| SCEH sub-type | 2 | 00 = Listening Window control 01 = Resume Sleep Cycle Indication 10 = Multi-Carrier Listening Window control 11 = Sleep cycle configuration change |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request |
| … | … | … |

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D3. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Dec 2009.