| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Acknowledgement scheme of SCH/SCEH (P802.16/D3-16.2.16.2.3.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Lei Zhang Yang Liu Yuqin Chen Nan Li Mary Chion ZTE Corporation | zhang.lei13@zte.com.cn |
| Re: | Category: LB30b / Area: Chapter 16.2.16.2.3.2 (Listening Window Extension) |  |
| Abstract | This contribution proposes changes to the Section 16.2.16.2.3.2 of IEEE P802.16m/D3 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Acknowledgement scheme of SCH/SCEH (P802.16/D3-16.2.16.2.3.2)

Lei Zhang, Yang Liu, Yuqin Chen, Nan Li, Mary Chion

ZTE Corporation

- Introduction

In D3, when explicit Listening Window extension is performed through MAC signaling/extended header (i.e. SCH or SCEH), the SCH and SCEH need to be ACKed to ensure the synchronization of the sleep state machines at the AMS and ABS. However, the definition of response indication in current SCH and SCEH format seems inefficiency. In addition, we may need a more general method to ACK MAC signaling/extended header. This contribution proposes a more general method to ACK SCH/SCEH and maybe other MAC header.

- Proposed text

======================== Start of Proposed Text ========================

- 16.2.2.1.3.4 Sleep Control header (SCH)

[Modify table 660 (from line 55 on page 25) of P802.16m/D3 as follows]

Table 660—Sleep Control header Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Sleep Control Header Format { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type = 0b0011 |
| SCH sub-type | 2 | 0b00 = Listening Window control 0b01 = Resume Sleep Cycle Indication 0b10 = Sleep cycle control 0b11 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request If Response Indication is included and set to 1, the ABS or AMS shall transmit an MAEH as acknowledgement. |
| … | … | … |

- 16.2.2.2.5 Sleep Control extended header (SCEH)

[Modify table 668 (from line 39 on page 33) of P802.16m/D3 as follows]

Table 668—SCEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| SCEH () { |  |  |
| Type | 4 | SCEH type |
| SCEH sub-type | 2 | 00 = Listening Window control 01 = Resume Sleep Cycle Indication 10 = Multi-Carrier Listening Window control 11 = Sleep cycle configuration change |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request If Response Indication is included and set to 1, the ABS or AMS shall transmit an MAEH as acknowledgement. |
| … | … | … |

- 16.2.2.2.4 MAC Control Message ACK Extended Header

[Modify the sentence (from line 41 on page 32) of P802.16m/D3 as follows]

The Message ACK extended header format is defined in Table 667. This header may be used by ABS and AMS to indicate the reception of a specific, previously received MAC control message or signaling/extended header (i.e. SCH/SCEH). When an ABS or AMS receives a MAC control message or MAC control message fragment with the Polling bit set to 1 in the MCEH, the ABS or AMS shall transmit an MAEH as an acknowledgement after receiving the complete message with the SN of the MAC control message PDU or the SN of the last received fragment if fragmented.

Table 667—MAEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MAEH() { |  |  |
| Type | 4 | MAEH Type |
| MAEH sub-type | 1 | 0 = indicate the reception of a MAC control message； 1 = indicate the reception of a MAC signaling/extended header； |
| If (MAEH sub-type == 0) { |  |  |
| ACK_SN | 8 | SN retrieved from the MCEH of the MAC PDU with the Polling bit set to 1 |
| } |  |  |
| Reserved | Variable | For byte alignment |
| } |  |  |

- 16.2.2.1.3.3 Service Specific BR without STID Header

[Modify the sentence (at line 8 on page 24) of P802.16m/D3 as follows]

Service Specific BR without STID Header is sent through dedicated UL resource assigned to the AMS, which needs to change the scheduling parameters of its service flow. BR type shall be included to indicate whether BR type is increment or aggregate. Service Specific BR without STID header format is defined in Table 659. If SCID change indicator is set to 1, the ABS shall send an acknowledgement (TBD) MAEH to confirm the change of sleep mode configuration.

======================== End of Proposed Text ========================

- References

[1] IEEE P802.16m/D3. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Dec 2009.