| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for support of dual streams per AMS in MU-MIMO (16.3.7) |  |
| Date Submitted | 2010-1-13 |  |
| Source(s) | Sangheon Kim, David Mazzarese Hokyu Choi, Heewon Kang Samsung Electronics | Sangheon.kim, d.mazzarese@samsung.com |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | This contribution proposes change for dual stream support per AMS in MU-MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications for support of dual streams per AMS in MU-MIMO (16.3.7)

Sangheon Kim, David Mazzarese, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes the modification for support of dual streams per AMS in MU-MIMO. By using sounding channels, dual-streams per AMS can be supported in MU-MIMO without any change of feedback scheme. We propose changes required for dual streams support as follows:

- Definition of MIMO Encoding Format (MEF)

- Si field in DL basic assignment A-MAP IE

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text Proposal

<Modify the section 16.3.6.5.2.4.1 on line 37, page 395 as shown >

----------------------------------------- Start of the Proposed Text #1 --------------------------------------------

# 16.3.6.5.2.4.1 DL basic assignment A-MAP IE

Table 812 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 812.

| Table 812 - DL basic assignment A-MAP IE |  |  |
| --- | --- | --- |
| Syntax | Size in bits | Description/Notes |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal Multi-layer encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 2 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for horizontal Multi-layer encoding |
| Si | 4 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a 0b00000: 2 streams with PSI=stream0 and other modulation =QPSK 0b00001: 2 streams with PSI=stream0 and other modulation =16QAM 0b00010: 2 streams with PSI=stream0 and other modulation =64QAM 0b00011: 2 streams with PSI=stream0 and other modulation information not available 0b00100: 2 streams with PSI=stream1 and other modulation =QPSK 0b00101: 2 streams with PSI=stream1 and other modulation =16QAM 0b00110: 2 streams with PSI=stream1 and other modulation =64QAM 0b00111: 2 streams with PSI=stream1 and other modulation information not available 0b01000: 3 streams with PSI=stream0 0b01001: 3 streams with PSI=stream1 0b01010: 3 streams with PSI=stream2 0b01011: 4 streams with PSI=stream0 0b01100: 4 streams with PSI=stream1 0b01101: 4 streams with PSI=stream2 0b01110: 4 streams with PSI=stream3 0b01111: 3 streams with PSI=stream0 and stream1 0b10000: 4 streams with PSI=stream0 and stream1 0b10001: 4 streams with PSI=stream2 and stream3 0b10010~0b11111: n/a |
| } else { |  |  |
| Reserved | 5 | Reserved bit |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 2 1 | Reserved bits |
| } |  |  |

----------------------------------------- End of the Proposed Text #1 --------------------------------------------

<Modify the section 16.3.7.1 on line 3, page 445 as shown >

----------------------------------------- Start of the Proposed Text #2 --------------------------------------------

# 16.3.7.1 Downlink MIMO architecture and data processing

The architecture of downlink MIMO at the transmitter side is shown in Figure 528.

The MIMO encoder block maps L MIMO layers  onto Mt MIMO streams , which are fed to the Precoder block. For the spatial multiplexing modes in SU-MIMO, "rank" is defined as the number of MIMO streams to be used for the user allocated to the Resource Unit (RU).

For SU-MIMO, only one user is scheduled in one Resource Unit (RU), and only one FEC block exists at the input of the MIMO encoder (vertical MIMO encoding at transmit side).

For MU-MIMO, multiple users can be scheduled in one RU, and multiple FEC blocks exist at the input of the MIMO encoder (horizontal MIMO encoding or combination of vertical and horizontal MIMO encoding at transmit side, which is called multi-layer encoding).

The precoder block maps MIMO stream(s) to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

The subcarrier mapping blocks map antenna-specific data to the OFDM symbol

# 16.3.7.1.1 MIMO layer to MIMO stream mapping

MIMO layer to MIMO stream mapping is performed by the MIMO encoder. The MIMO encoder is a batch processor that operates on M input symbols at a time.

The input to the MIMO encoder is represented by an M×1 vector as specified in Equation (209).

(209)

Where si is the i-th input symbol within a batch. In case of MU-MIMO transmissions, the M symbols belong to M differents AMSs. Two consecutive symbols may belong to a single MIMO layer. One AMS shall have at most one MIMO layer.

MIMO layer to MIMO stream mapping of the input symbols is done in the space dimension first. The output of the MIMO encoder is an  MIMO STC matrix as given in Equation (210), which serves as the input to the precoder.

x = S(s) 													(210)

Where

Mt is the number of MIMO streams

NF is the number of subcarriers occupied by one MIMO block

x is the output of the MIMO encoder

s is the input MIMO layer vector

S() is a function that maps an input MIMO layer vector to an STC matrix

S(s) is an STC matrix

The STC matrix x can be expressed as in Equation (211):

. 											(211)

The four MIMO encoder formats (MEF) are SFBC, vertical encoding (VE), horizontal encoding (HE) multi-layer encoding (ME),  and CDR. For SU-MIMO transmissions, the STC rate is defined as in (212)

(212)

For MU-MIMO transmissions, the STC rate per user (R) is equal to 1 or 2.

# 16.3.7.1.1.1 SFBC encoding

The input to the MIMO encoder is represented by a 2 × 1 vector.

.													 (213)

The MIMO encoder generates the SFBC matrix.

(214)

Where x is a 2x2 matrix.

The SFBC matrix, x, occupies two consecutive subcarriers.

# 16.3.7.1.1.2 Vertical encoding

The input and the output of MIMO encoder is represented by an M  1 vector.

. 												(215)

Where si is the i-th input symbol within a batch.

For vertical encoding,  belong to the same MIMO layer. The encoder is an identity operation.

# 16.3.7.1.1.3 Horizontal Multi-layer encoding

The input and output of the MIMO encoder is represented by an M  1 vector.

(216)

Where  is the i-th input symbol within a batch.

For horizontal Multi-layer encoding,  belong to different MIMO layers, where two consecutive symbols may belong to a single MIMO layer.

Horizontal Multi-layer encoding is only used for MU-MIMO mode. The encoder is an identity operation.

# 16.3.7.1.1.4 CDR encoding

The input to the MIMO encoder is represented by a 1 × 1 vector.

(217)

The MIMO encoder generates the CDR matrix.

(218)

The CDR matrix, x, occupies two consecutive subcarriers.

# 16.3.7.1.2 MIMO stream to antenna mapping

MIMO stream to antenna mapping is performed by the precoder. The output of the MIMO encoder is multiplied by an  precoder, W. The output of the precoder is denoted by an  matrix, z. The mapping can be defined in Equation (219).

(219)

Where  is the number of transmit antennas and  is the output symbol to be transmitted via the j-th physical antenna on the k-th subcarrier. Pilots within PRU are precoded in the same way as the data subcarriers.

# 16.3.7.1.2.1 Non-adaptive precoding

With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k), where Nt is the number of transmit antennas, Mt is the number of MIMO streams, and k is the physical index of the subcarrier where W(k) is applied. The matrix W is selected from the base codebook or from a subset of size Nw precoders of the base codebook for a given rank. W belongs to the base codebook or to one of the subsets of the base codebook, according to the type of allocation, MEF, Nt, MaxMt and Mt, as specified in Table 831 and Table 832. The notation CDL,OL,SU(Nt, Mt, Nw) denotes a DL OL SU-MIMO codebook subset, which consists of Nw complex matrices of dimension Nt by Mt. The base codebook and the codebook subsets are defined in 16.3.7.2.5.6.

| Table 831 - Codebook subsets used for non-adaptive precoding in DL DLRU and NLRU |  |  |
| --- | --- | --- |
| MEF | RU with Mt pilot MIMO streams outside OL region | RU in OL region with MaxMt MIMO streams |
| SFBC | Nt=2: CDL,OL,SU (2, Mt, 1), Mt=2 Nt=4: CDL,OL,SU (4, Mt, 4), Mt=2 Nt=8: CDL,OL,SU (8, Mt, 4), Mt=2 | Nt=2: CDL,OL,SU (2,MaxMt, 1), MaxMt=2 Nt=4: CDL,OL,SU (4,MaxMt, 4), MaxMt=2 Nt=8: CDL,OL,SU (8,MaxMt, 4), MaxMt=2 |
| VE | Nt=2: CDL,OL,SU (2, Mt, Nw), Mt=1,2 Nt=4: CDL,OL,SU (4, Mt, Nw), Mt=1,2,3,4 Nt=8: CDL,OL,SU (8, Mt, Nw), Mt=1,2,3,4 Nw depends on Mt | Nt=2: CDL,OL,SU (2,MaxMt, 1), MaxMt=2 Nt=4: CDL,OL,SU (4,MaxMt, 4), MaxMt=2 Nt=8: CDL,OL,SU (8,MaxMt, 4), MaxMt=2 |
| HE ME | n.a | n.a |
| CDR | n.a | Nt=2: CDL,OL,SU (2, MaxMt, 2), MaxMt=1 Nt=4: CDL,OL,SU (4, MaxMt, 4), MaxMt=1 Nt=8: CDL,OL,SU (8, MaxMt, 8), MaxMt=1 |

| Table 832 - Codebook subsets used for non-adaptive precoding in DL SLRU |  |  |
| --- | --- | --- |
| MEF | RU with Mt pilot MIMO streams outside OL region | RU in OL region with MaxMt MIMO streams |
| SFBC | n.a | n.a |
| VE | Nt=2: C(2, Mt, 3), Mt = 1, …, MaxMt Nt=4: C(4, Mt, 4), Mt = 1, …, MaxMt Nt=8: C(8, Mt, 4), Mt = 1, …, MaxMt | Nt=2: C(2, MaxMt, 3), MaxMt = 2 Nt=4: C(4, MaxMt, 4), MaxMt = 2 Nt=8: C(8, MaxMt, 4), MaxMt = 2 |
| HE ME | Nt=2: C(2, Mt, 3), Mt = 2, …, MaxMt Nt=4: C(4, Mt, 4), Mt = 2, …, MaxMt Nt=8: C(8, Mt, 3), Mt = 2, …, MaxMt | Nt=2: C(2, MaxMt, 3), MaxMt = 2 Nt=4: C(4, MaxMt, 4), MaxMt = 2 Nt=8: C(8, MaxMt, 4), MaxMt = 2 |
| CDR | n.a | Nt=2: C(2,MaxMt, 3), MaxMt = 1 Nt=4: C(4,MaxMt, 4), MaxMt = 1 Nt=8: C(8,MaxMt, 4), MaxMt = 1 |

----------------------------------------- End of the Proposed Text #2 --------------------------------------------

<Modify the section 16.3.7.1.3 on line 62, page 449 as shown >

----------------------------------------- Start of the Proposed Text #3 --------------------------------------------

# 16.3.7.1.3 Downlink MIMO modes

There are six MIMO transmission modes for unicast DL MIMO transmission as listed in Table 833.

| Table 833 - Downlink MIMO modes |  |  |  |
| --- | --- | --- | --- |
| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| Mode 0 | OL SU-MIMO (Tx diversity) | SFBC | non-adaptive |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL MU-MIMO (SM) | HE ME | non-adaptive |
| Mode 4 | CL MU-MIMO (SM) | HE ME | adaptive |
| Mode 5 | OL SU-MIMO (Tx diversity) | CDR | non-adaptive |

.

The allowed values of the parameters for each DL MIMO mode are shown in Table 834.

| Table 834 - DL MIMO parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number of transmit antennas | STC rate per MIMO layer | Number of MIMO streams | Number of subcarriers | Number of MIMO layers |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 3 | 1 | 3 |
|  | 4 | 1 | 4 | 1 | 4 |
|  | 8 | 1 | 2 | 1 | 2 |
|  | 8 | 1 | 3 | 1 | 3 |
|  | 8 | 1 | 4 | 1 | 4 |
| MIMO mode 4 | 4 | 2 and 1* | 3 | 1 | 2 |
|  | 4 | 2 and 1** | 4 | 1 | 3 |
|  | 4 | 2 | 4 | 1 | 2 |
|  | 8 | 2 and 1* | 3 | 1 | 2 |
|  | 8 | 2 and 1** | 4 | 1 | 3 |
|  | 8 | 2 | 4 | 1 | 2 |
| MIMO mode 5 | 2 | 1/2 | 1 | 2 | 1 |
|  | 4 | 1/2 | 1 | 2 | 1 |
|  | 8 | 1/2 | 1 | 2 | 1 |

* 2 streams to one AMS and 1 stream to another AMS, with 1 layer each.

** 2 streams to one AMS and 1 stream each to the other two AMSs, with 1 layer each.

Mt refers to the number of MIMO streams transmitted to one AMS with MIMO modes 0, 1, 2 and 5.

Mt refers to the total number of MIMO streams transmitted to multiple AMS on the same RU with MIMO modes 3 and 4.

----------------------------------------- End of the Proposed Text #3 --------------------------------------------

<Modify the section 16.3.7.2.2 on line 31, page 452 as shown >

----------------------------------------- Start of the Proposed Text #4 --------------------------------------------

# 16.3.7.2.2 Encoding and precoding of MU-MIMO

Multi-user MIMO schemes are used to enable a resource allocation to communicate data to two or more AMSs. Multi-user transmission with one or two MIMO streams per user is supported for MU-MIMO.

MU-MIMO includes the MIMO configuration of 2Tx antennas to support up to 2 AMSs, and 4Tx or 8Tx antennas to support up to 4 AMSs, with 1 MIMO stream per AMS.

Both OL MU-MIMO (mode 3) and CL MU-MIMO (mode 4) are supported.

# 16.3.7.2.2.1 Encoding of MU-MIMO modes

- MIMO mode 3: Horizontal Multi-layer encoding of 16.3.7.1.1.3 shall be used with MIMO mode 3.

- MIMO mode 4: Horizontal Multi-layer encoding of 16.3.7.1.1.3 shall be used with MIMO mode 4.

# 16.3.7.2.2.2 Precoding of MU-MIMO modes

- MIMO mode 3

Non-adaptive precoding of 16.3.7.1.2.1 shall be used with MIMO mode 3.

With OL MU MIMO inside the OL region, the precoder W with 2 MIMO streams is predefined and fixed over time. With OL MU MIMO outside the OL region, the precoder W is an Nt×Mt sub-matrix of a predefined Nt×MaxMt matrix.

The precoding matrix W used by the ABS is represented in Equation (223).

(223)

Where vi(k) is the precoding vector for the i-th AMS on the k-th subcarrier.

vi(k) shall be used for precoding the pilot symbols on the i-th pilot MIMO stream on the k-th subcarrier.

- MIMO mode 4

Adaptive precoding of 16.3.7.1.2.2 shall be used with MIMO mode 4.

In CL MU MIMO, the precoder W is an Nt × M matrix for each subcarrier. It is used to communicate to up to M AMSs simultaneously. The form and derivation of the precoding matrix does not need to be known at the AMS. The ABS determines the precoding matrix based on the feedback received from the AMS.

The ABS shall construct the precoding matrix W as represented in Equation (224).

(224)

Where,  is the precoding vector for the i-th AMS stream on the k-th subcarrier.

vi(k) shall be used for precoding the pilot symbols on the i-th pilot MIMO stream on the k-th subcarrier.

----------------------------------------- End of the Proposed Text #4 --------------------------------------------