| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed New ABI Contents on the Ranging Channel for Synchronized AMSs in IEEE P802.16m/D3 (Section 16.2.3.29) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Xinrong Wang, Xiangying Yang, Joey Chou, Yang-Seok Choi, Yi Hsuan Intel Corporation | {xinrong.wang; xiangying.yang; joey.chou; yang-seok.choi; yi.hsuan} @intel.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30b P802.16m/D3 : Section 16.2.3.29 |  |
| Abstract | This contribution proposes new ABI contents on the ranging channel for synchronized AMSs. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed New ABI Contents on the Ranging Channel for Synchronized AMSs in IEEE P802.16m/D3 (Section 16.2.3.29)

Xinrong Wang, Xiangying Yang, Joey Chou, Yang-Seok Choi, Yi Hsuan

Intel Corporation

# 1. Introduction

Ranging channel for synchronized AMSs signaling is broadcasted in ABI. The ABI contents on the ranging channel for synchronized AMSs are missing in the current draft IEEE P802.16m/D3 [1]. This contribution provides this information.

This contribution also cleans up many syntax errors of the ASN.1 coding of AAI_System Configuration Descriptor (SCD) Message in section 16.2.3.29.

# Reference

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” December 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 16.2.3.29 of P802.16m/D3

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

[Add the following contents into the AAI_SCD contents in Section 16.2.3.29]

-- System Configuration Descriptor Message

AAI-SCD ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

configChangeCount         INTEGER (0..15),

bsRestartCount            INTEGER (0..15),

saPreamblePartForBsType   SEQUENCE {},

triggerTlvEncoding        SEQUENCE {},

defaultHORssiCinrAvg      SEQUENCE {},

normalizedCINR            SEQUENCE {},

olMimoParameters          SEQUENCE {

olRegionOn              INTEGER (0..1) OPTIONAL,

olRank1Config           INTEGER (0..7) OPTIONAL,

sbOlRegion2Size         INTEGER (0..15) OPTIONAL},

graParameters             SEQUENCE {},

periodicRngParameters     SEQUENCE {},

gammaIotFp0               INTEGER (0..15) OPTIONAL,

gammaIotFp1               INTEGER (0..15) OPTIONAL,

gammaIotFp2               INTEGER (0..15) OPTIONAL,

gammaIotFp3               INTEGER (0..15) OPTIONAL,

alpha                     INTEGER (0.. 7) OPTIONAL,

beta                      INTEGER (0..1) OPTIONAL,

minmumSinr                INTEGER (0..15) OPTIONAL,

brChConfigMinAccessClass  INTEGER (0..4) OPTIONAL,

soundingMultiplexType     ENUMERATED {

fdm (0),

cdm (1)},

maxCsIndexPForSounding    ENUMERATED {

n4 (0),

n9 (1),

n16 (2),

n18 (3),

n36 (4)},

relaySupportParameter     SEQUENCE {

relayZoneAmsAlocIndc    INTEGER (0..1) OPTIONAL},

-- the periodicity of ranging channel for synchronized AMSs

-- allocation (Table 893)

periodicityOfRngChSync    INTEGER (0..3),

-- the parameter ks controlling the start root index of ranging

-- preamble codes for synchronized AMSs

cntlStartCodeOfRngChSync      INTEGER (0..15),

rangingPreambleCodeSync       CHOICE {

-- the number of codes for periodic ranging (Table 891)

nonFemtoBS              INTEGER (0..3),

-- the number of codes for initial, handover, and periodic

-- ranging in Femto-cell (Table 892)

femtoBS                 INTEGER (0..15)}

}

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------