| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed New Information Contents on the Ranging Channel for Synchronized AMSs in IEEE P802.16m/D3 (Section 16.2.3.29 and Section 16.3.6.5.1.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Xinrong Wang, Xiangying Yang, Joey Chou, Yang-Seok Choi, Yi Hsuan Intel Corporation HyunWoo Lee, Jin Sam Kwak, and Heejeong Cho LG Electronics Yan-Xiu Zheng, Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang ITRI Pei-Kai Liao, Yih-Shen Chen, Paul Cheng MediaTek Inc Hwasun Yoo, Chiwoo Lim Samsung Electronics | {xinrong.wang; xiangying.yang; joey.chou; yang-seok.choi; yi.hsuan} @intel.com {hyunwoolee.lee; jinsam.kwak; heejeong.cho} @lge.com zhengyanxiu@itri.org.tw pk.liao@mediatek.com hwasun.yoo@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30b P802.16m/D3 : Section 16.2.3.29 and Section 16.3.6.5.1.2 |  |
| Abstract | This contribution proposes new information contents on the ranging channel for synchronized AMSs. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed New Information Contents on the Ranging Channel for Synchronized AMSs in IEEE P802.16m/D3 (Section 16.2.3.29 and Section 16.3.6.5.1.2)

Xinrong Wang, Xiangying Yang, Joey Chou, Yang-Seok Choi, and Yi Hsuan

Intel Corporation

HyunWoo Lee, Jin Sam Kwak, and Heejeong Cho

LG Electronics

Yan-Xiu Zheng, Chang-Lan Tsai, Chung-Lien Ho, Yu-Chuan Fang

ITRI

Pei-Kai Liao, Yih-Shen Chen, Paul Cheng

MediaTek Inc

Hwasun Yoo, Chiwoo Lim

Samsung Electronics

# 1. Introduction

Signaling of ranging channel for synchronized AMSs is broadcasted, but it is missing in the current draft IEEE P802.16m/D3 [1]. This contribution provides this information. In non-Femtocell deployment scenario, signaling of ranging channel for synchronized AMSs is broadcasted in AAI_SCD in section 16.2.3.29. In Femtocell deployment scenario, signaling of ranging channel for synchronized AMSs is broadcasted in SFH SP1 in section 16.3.6.5.1.2. It is noted that ranging channel for synchronized AMSs is used for periodic ranging in non-Femtocell and is used for initial ranging, handover ranging and periodic ranging in Femtocell.

This contribution also cleans up many syntax errors of the ASN.1 coding of AAI_SCD Message in section 16.2.3.29.

# Reference

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” December 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: existing text in P802.16m/D3

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

------------------------------------------------------------------------ Remedy #1 -------------------------------------------------------------------------

[Replace the texts in Section 16.2.3.29 with the following texts]

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- System Configuration Descriptor Message

AAI-SCD ::= SEQUENCE {

messageType               OCTET STRING (SIZE(1)),

configChangeCount         INTEGER (0..15),

bsRestartCount            INTEGER (0..15),

saPreamblePartForBsType   SEQUENCE {},

triggerTlvEncoding        SEQUENCE {},

defaultHORssiCinrAvg      SEQUENCE {},

normalizedCINR            SEQUENCE {},

olMimoParameters          SEQUENCE {

olRegionOn              INTEGER (0..1) OPTIONAL,

olRank1Config           INTEGER (0..7) OPTIONAL,

sbOlRegion2Size         INTEGER (0..15) OPTIONAL},

graParameters             SEQUENCE {},

gammaIotFp0               INTEGER (0..15) OPTIONAL,

gammaIotFp1               INTEGER (0..15) OPTIONAL,

gammaIotFp2               INTEGER (0..15) OPTIONAL,

gammaIotFp3               INTEGER (0..15) OPTIONAL,

alpha                     INTEGER (0.. 7) OPTIONAL,

beta                      INTEGER (0..1) OPTIONAL,

minmumSinr                INTEGER (0..15) OPTIONAL,

brChConfigMinAccessClass  INTEGER (0..4) OPTIONAL,

-- Sounding sequence

-- D is decimation value for frequency division multiplexing

-- P is maximum cyclic shift for code division multiplexing

soundingMultiplexingType       CHOICE {

decimationValueD             ENUMERATED {

decValueD0 (4),

decValueD1 (6),

decValueD2 (8),

decValueD3 (9),

decValueD4 (12),

decValueD5 (16),

decValueD6 (18),

decValueD7 (36)},

maxCyclicShiftIndexP         ENUMERATED {

csIndexP0 (4),

csIndexP1 (6),

csIndexP2 (8),

csIndexP3 (9),

csIndexP4 (12),

csIndexP5 (16),

csIndexP6 (18),

csIndexP7 (36)}

},

shiftValueUForSoundingSymbol   INTEGER (0..255),

relaySupportParameter     SEQUENCE {

relayZoneAmsAlocIndc    INTEGER (0..1) OPTIONAL},

-- the periodicity of ranging channel for synchronized AMSs

-- allocation (Table 893)

-- This is ignored in Femtocell

periodicityOfRngChSync    INTEGER (0..3),

-- the parameter ks controlling the start root index of ranging

-- preamble codes for synchronized AMSs

-- This is ignored in Femtocell

cntlStartCodeOfRngChSync      INTEGER (0..15),

-- the number of codes for periodic ranging (Table 891)

-- this is ignored in Femtocell

rangingPreambleCodeSync       INTEGER (0..3)

}

------------------------------------------------------------------------ Remedy #2 -------------------------------------------------------------------------

[Make the following modifications in Table 801 in Section 16.3.6.5.1.2]

Table 801 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … | … | … |
| Ranging channel allocation periodicity | 2 | Indicates the periodicity of ranging channel allocation according to the Table 890. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 893890. The range of values is 0 ≤ OSF ≤ 3. |
| If ( FDM-based UL PUSC zone support ) { |  | Indicates the WirelessMAN-OFDMA support in the UL with FDM mode, when the Frame Configuration Index in S-SFH SP2 is equal to - 5 or 7 or 9 for 5/10/20MHz channel bandwidth according to Table 771 - 4 or 6 or 8 for 8.75MHz channel bandwidth according to Table 772 - 3 (with CP=1/8) for 7MHz channel bandwidth according to Table 773. |
| Allocation periodicity of ranging channel | 2 | Indicates the periodicity of ranging channel allocation according to the Table 890. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 890. The range of values is 0 ≤ OSF ≤ 3. |
| Start code information of ranging channel | 4 | Indicates the kns which is the parameter for start of code group (S). S= 16×kns+1 The range of values is 0 ≤ kns ≤ 15. |
| Ranging preamble code partition information | 4 | Indicates the number of initial, handover and periodic codes (N, O and M) according to the Table [TBD]894. |
| Ranging channel location in the frequency domainReserved | 3 | Indicate the start distributed LRU index for ranging channel. |
| } elseif ( Femtocell ) { |  | For 16m Femtocell |
| Allocation periodicity of ranging channel for synchronized AMSs | 2 | Indicates the periodicity of ranging channel allocation according to the Table 893. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 893. The range of values is 0 ≤ OSF ≤ 3. |
| Start code information of ranging channel for synchronized AMSs | 4 | Indicates the ks which is the parameter for start of code group (rs0). rs0= 6×ks+1 The range of values is 0 ≤ ks ≤ 15. |
| Ranging preamble code partition information for synchronized AMSs | 4 | Indicates the number of initial and handover ranging preamble codes (NIN, NPE and NHO) according to the Table 892. |
| Reserved | 3 |  |
| } else { |  | For 16m non-Femtocell |
| Allocation periodicity of ranging channel for non-synchronized AMSs | 2 | Indicates the periodicity of ranging channel allocation according to the Table 890. |
| Subframe offset of ranging channel | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 890. The range of values is 0 ≤ OSF ≤ 3. |
| Start code information of ranging channel for non-synchronized AMSs | 4 | Indicates the kns which is the parameter controlling the start root index of ranging preamble codes (rns0). rns0(kns)=4×kns+1 for ranging channel format 0. rns0(kns)=16×kns+1 for ranging channel format 1. The range of values is 0 ≤ kns ≤ 15. |
| Ranging preamble code partition information for non-synchronized AMSs | 4 | Indicates the number of initial and handover ranging preamble codes (NIN and NHO) according to the Table 889. |
| Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs | 2 | Indicates the number of cyclic shifted codes per root index (Mns) for ranging preamble codes according to the Table 888. |
| Ranging channel formats for non-synchronized AMSs | 1 | Indicates the ranging channel formats number of Table 883. |
| } |  |  |
| … | … | … |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------