| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Backward Compatibility in NBR-ADV Message |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Daniel Cohn, Alvarion Tehuang Liu, Mediatek Jon Labs, Wavesat | E-mail:daniel.cohn@alvarion E-mail: tehuang.liu@mediatek.com |
| Re: | P802.16Rev2/D9 |  |
| Abstract | The NBR-ADV message format in IEEE 802.16-2009 is not backward compatible with 802.16Rev2/D8 and previous versions. This contribution proposes a remedy. |  |
| Purpose | Formatting this contribution for incorporating the CR into P802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Backward Compatibility in NBR-ADV Message

Daniel Cohn, Alvarion

Tehuang Liu, Mediatek

Problem Statement

Following contribution IEEE C802.16maint-09/0008r2, in IEEE P802.16/Rev2D9 the MOB_NBR-ADV message was modified to include a new field called “Reuse factor for SBS CINR calculation for scan and handover”, as shown below.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_NBR_ADV_Message_format(){ | — | — |
| Management Message Type = 53 | — | — |
| Skip-optional-fields bitmap | 8 6 | Bit [0]: if set to 1, omit Operator ID field. Bit [1]: if set to 1, omit NBR BS ID field. Bit [2]: if set to 1, omit HO process optimization field. Bit [3]: if set to 1, omit QoS related fields. Bit [4]–[7]: Reserved. Bit [4]–[5]: Reserved. |
| Reuse factor for SBS CINR calculation for scan and handover | 2 | 00 - Physical SBS CINR for scan or handover triggers shall be calculated according to the number of subcarriers indicated in the DL Frame Prefix "Used subchannel bitmap" field. If the number of used subcarriers is lower than or equal to one third of the total number of subcarriers, then CINR shall be computed according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 3. Otherwise the CINR shall be computed according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 1 10 – Physical SBS CINR for scan or handover triggers shall be calculated according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 1 01 – Physical SBS CINR for scan or handover triggers shall be calculated according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 3 11 – reserved |

According to the rules in P802.16Rev2/D9, section 6.3.3.1, the modified fields are transmitted in the following order:

Skip-optional-fields bitmap bit #5 - reserved (MSB of this field)

Skip-optional-fields bitmap bit #4 - reserved

Skip-optional-fields bitmap bit #3 - omit QoS related fields

Skip-optional-fields bitmap bit #2 - omit HO process optimization field.

Skip-optional-fields bitmap bit #1 - omit NBR BS ID field

Skip-optional-fields bitmap bit #0 - omit Operator ID field (LSB of this field)

Reuse factor for SBS CINR calculation for scan and handover bit #1 (MSB of this field)

Reuse factor for SBS CINR calculation for scan and handover bit #0 (LSB of this field)

This modification is not backward compatible with the previous message format found in P802.16Rev2/D8, where the relevant fields were transmitted in the following order:

Skip-optional-fields bitmap bit #7 - reserved (MSB of this field)

Skip-optional-fields bitmap bit #6 - reserved

Skip-optional-fields bitmap bit #5 - reserved

Skip-optional-fields bitmap bit #4 - reserved

Skip-optional-fields bitmap bit #3 - omit QoS related fields

Skip-optional-fields bitmap bit #2 - omit HO process optimization field.

Skip-optional-fields bitmap bit #1 - omit NBR BS ID field

Skip-optional-fields bitmap bit #0 - omit Operator ID field (LSB of this field)

Proposed Solution

[Insert the following change language text at the end of page 17 of P802.16m/D3:]:

6. MAC common part sublayer

6.3 Data/Control plane

6.3.2 MAC PDU formats

6.3.2.3 MAC management messages

6.3.2.3.42 MOB_NBR-ADV (neighbor advertisement) message

[Modify the following fields in Table 144 in 6.3.2.3.42 as follows:]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_NBR_ADV_Message_format(){ | — | — |
| Management Message Type = 53 | — | — |
| Reuse factor for SBS CINR calculation for scan and handover | 2 | 00 - Physical SBS CINR for scan or handover triggers shall be calculated according to the number of subcarriers indicated in the DL Frame Prefix "Used subchannel bitmap" field. If the number of used subcarriers is lower than or equal to one third of the total number of subcarriers, then CINR shall be computed according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 3. Otherwise the CINR shall be computed according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 1 10 – Physical SBS CINR for scan or handover triggers shall be calculated according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 1 01 – Physical SBS CINR for scan or handover triggers shall be calculated according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 3 11 – reserved |
| Skip-optional-fields bitmap | 6 | Bit [0]: if set to 1, omit Operator ID field. Bit [1]: if set to 1, omit NBR BS ID field. Bit [2]: if set to 1, omit HO process optimization field. Bit [3]: if set to 1, omit QoS related fields. Bit [4]–[5]: Reserved. |
| Reuse factor for SBS CINR calculation for scan and handover | 2 | 00 - Physical SBS CINR for scan or handover triggers shall be calculated according to the number of subcarriers indicated in the DL Frame Prefix "Used subchannel bitmap" field. If the number of used subcarriers is lower than or equal to one third of the total number of subcarriers, then CINR shall be computed according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 3. Otherwise the CINR shall be computed according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 1 10 – Physical SBS CINR for scan or handover triggers shall be calculated according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 1 01 – Physical SBS CINR for scan or handover triggers shall be calculated according to the rule detailed in 8.4.12.3 for frequency reuse configuration = 3 11 – reserved |