| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Power Headroom Reporting Section for the IEEE 802.16m/D3 Amendment |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Rongzhen Yang, Shi Liu, Hujun Yin, Yang-seok Choi Intel Corp. | Rongzhen.yang@intel.com |
| Re: | Comments on P802.16m/D3 |  |
| Abstract | This contribution provides the proposed text for 16m power headroom reporting. |  |
| Purpose | To be discussed and adopted into P802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Power Headroom Reporting Section for the IEEE 802.16m/D3 Amendment

Rongzhen Yang, Shi Liu, Hujun Yin, Yang-seok Choi
Intel Corp.

- Introduction

Power headroom reporting form AMS to ABS provides very important reference information for ABS uplink resource scheduling, which includes:

- Headroom definition;

- Headroom transmission conditions;

- Headroom reporting message design;

All those parts are missed in current 802.16m D3 amendment, in this contribution, the major parts of headroom reporting text are proposed.

- References

[1] IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Oct. 2009.

- Text proposal

Instruction to editor;

Black text: existing D3 text

Blue and strike out text: for deletion

Red and underline text: for addition

[Remedy:]

[Insert one new subsection 16.3.9.4.6 “Uplink Power Headroom Reporting” after line 57 on the page 564, as follows:]

============================== Start of Proposed Text =================================

16.3.9.4.6 Uplink Power Headroom Reporting

16.3.9.4.6.1 Power Headroom Reporting Information

There are two key parameters for power headroom reporting: the base uplink transmission PSD and data SINR target value.

The base uplink transmission PSD is derived from equation (269) by setting SINRTarget=0 and Offset=0:

PSD(base)=L+NI				(273)

The parameter is reported in dBm and is quantized in 0.5 dBm steps ranging from -100 dBm (encoded 0x00) to 27.5 dBm (encoded 0xFF).

The data SINR target value is calculated by equation (270) and quantized by 8 bits as SINRTarget(Reported) with the step:

SINR=SINRMAX(dB)−SINRMIN(dB)255	(274)

After ABS received PSD(base) and SINRTarget(Reported) from AMS by power headroom reporting, all uplink channels calculated transmission power PSD status can be derived by those two key parameters.

16.3.9.4.6.2 Headroom Transmission Condition

The AMS shall send the headroom reporting message AAI_UL_PHR_Message (TBD) to ABS when the conditions listed below meet:

L(nlast)−L(n)≥Tx_Power_Report_Threshold(dB) andn−nlast≥Tx_Power_Report_Minimum_Interval	(275)

or

n−nlast≥Tx_Power_Report_Periodical_Interval		(276)

Where,

L is the pathloss defined in equation (269)

nlast is the time index when the last AMS headroom reporting message is sent, the unit is frame.

n is the current time index of AMS headroom reporting message.

Tx_Power_Report_Threshold(dB) is 4 bits unsigned integer value which shall be read in dB scale, the specific “0b1111” is read as infinite;

Tx_Power_Report_Minimum_Interval and Tx_Power_Report_Periodical_Intervalis represented by 4 bits unsigned integer value d each to represent 2dframes, the specific value d=0b1111 is read as infinite

The headroom reporting configuration parameters Tx_Power_Report_Threshold(dB), Tx_Power_Report_Minimum_Interval and Tx_Power_Report_Periodical_Interval are sent from ABS to AMS by the message AAI_UL_PHR_Config (TBD).

============================== End of Proposed Text =================================