| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Power Headroom Reporting Section for the IEEE 802.16m/D3 Amendment |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Rongzhen Yang, Shi Liu, Hujun Yin, Yang-seok Choi Intel Corp. Tim Wilson Motorola | Email: Rongzhen.yang@intel.com Email : Tim.Wilson@motorola.com |
| Re: | Comments on P802.16m/D3 |  |
| Abstract | This contribution provides the proposed text for 16m power headroom reporting. |  |
| Purpose | To be discussed and adopted into P802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Power Headroom Reporting Section for the IEEE 802.16m/D3 Amendment

Rongzhen Yang, Shi Liu, Hujun Yin, Yang-seok Choi
Intel Corp.

Tim Wilson

Motorola

- Introduction

Power headroom reporting form AMS to ABS provides very important reference information for ABS uplink resource scheduling, which includes:

- Headroom definition;

- Headroom transmission conditions;

- Headroom reporting message design;

All those parts are missed in current 802.16m D3 amendment, in this contribution, the major parts of headroom reporting text are proposed.

- References

[1] IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Oct. 2009.

- Text proposal

Instruction to editor;

Black text: existing D3 text

Blue and strike out text: for deletion

Red and underline text: for addition

[Remedy:]

[Insert one new subsection 16.3.9.4.6 “Uplink Power Headroom Reporting” after line 57 on the page 564, as follows:]

============================== Start of Proposed Text =================================

16.3.9.4.6 Uplink Power Headroom Reporting

16.3.9.4.6.1 Power Headroom Reporting Information

There are two key parameters for power headroom reporting: the base uplink transmission PSD and the data SINR target value.

The base uplink transmission PSD is derived from equation (269) by setting SINRTarget=0 and Offset=0:

PSD(base)=L+NI				(273)

The parameter is reported in dBm and is coded using 8 bits in 0.5 dBm steps ranging from -74 dBm (coded 0x00) to 53.5 dBm (coded 0xFF).

The reported data SINR target value, SINRTarget(Reported) is calculated from equation (270) by setting β=0. The value is coded using 7 bits in 0.5 dB steps ranging from -20dB (coded 0x00) to 43.5dB (coded 0x7f).

After the ABS has received PSD(base) and SINRTarget(Reported) from the AMS, the PSD for all uplink channels can be derived.

16.3.9.4.6.2 Headroom Transmission Condition

The AMS shall send the headroom reporting message AAI_UL_PHR_Message (TBD) to the ABS when the conditions listed below are met:

M(nlast)−M(n)≥Tx_Power_Report_Threshold(dB) andn−nlast≥Tx_Power_Report_Minimum_Interval	(274)

or

n−nlast≥Tx_Power_Report_Periodical_Interval		(275)

Where,

M is defined in equation 276:

M=L+SINRTarget(Reported)		(276)

Here, L is the pathloss defined in equation (269)

nlast is the frame index when the last AMS headroom reporting message was sent,.

n is the current frame index.

Tx_Power_Report_Threshold(dB) is a 4 bit unsigned integer value in 0.5 dB steps, the specific value “0b1111” shall be interpreted as “infinite”;

Tx_Power_Report_Minimum_Interval and Tx_Power_Report_Periodical_Intervalare coded by 4 bit unsigned integer values d representing 2dframes, the specific value d=0b1111 shall be interpreted as “infinite”

The headroom reporting configuration parameters Tx_Power_Report_Threshold(dB), Tx_Power_Report_Minimum_Interval and Tx_Power_Report_Periodical_Interval are sent from ABS to AMS in the message AAI_UL_PHR_Config (TBD).

============================== End of Proposed Text =================================