| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for E-MBS operation in connected state (P802.16m/D3-16.9.2.2) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Kaiying Lv, Nan Li,Bo Sun ZTE Corporation Kamran Etemad Intel Corporation | Voice: [Telephone Number (optional)]] E-mail: lv.kaiying@zte.com.cn kamran.etemad@intel.com |
| Re: | Category: LB30b / Area: Chapter 16.9.2.2.( E-MBS operation in connected state) |  |
| Abstract | This contribution proposes texts on E-MBS operation in connected state |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed text for E-MBS operation in Connected State

- Kaiying Lv，Nan Li，Bo Sun

- ZTE Corporation

Kamran Etemad

- Intel Corporation

- Proposed Text Changes

-------------------------------------------------------Start of the Text --------------------------------------------------------

[Make the following changes and add proposed text to Section16.9.2.2 in page643 beginning Line 3 in P802.16m/D3. ]

16.9.2.2 E-MBS Operation in Connected State

….

Once an AMS has received the EMBS allocation information in the EMBS-MAPs, it may not listen to the downlink channels till the next transmission of desired EMBS flow or the next E-MBS-MAP. The AMS does monitor down link channels and follow the entire idle mode and connected mode procedures as required by other services. While receiving E-MBS data, the AMS may temporary temporarily interrupt the reception of E-MBS packets for any reason, e.g. if time critical scanning is needed, without notifying the ABS.

….

16.9.2.4 E-MBS operation in Alternative Carrier

When EMBS data is transmitted on an alternative carrier, i.e. other than the AMS’s primary carrier where service flows are configured the AMS is redirected to relevant carrier through DSA as described in section 16.9.2.1.

In this case the AMS is expected to switch to that alternative carrier to receive the next EMBS configuration message and EMBS data. During the transmission of EMBS configuration messages and the EMBS data to which AMS is subscribed, the AMS with only one transceiver may not be available for signaling exchange with ABS on the primary carrier.

The AMS with multiple transceivers may be able to receive EMBS data while communicating with ABS on primary carrier.

Other EMBS operations in idle and connection states as described in section 16.9.2.2 and 16.9.2.3 also applicable to this scenario.

----------------------------------------------------End of the Text---------------------------------------------------------

Reference

[1] IEEE P802.16m/D3