| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text to Assignment A-MAP IE (Section 16.3.6.5) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Hyunkyu Yu, Seho Kim, Youngbo Cho, Heewon Kang Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot#30b” Target topic: “IEEE P802.16m/D3 section 16.3.6.5”. |  |
| Abstract | The contribution proposes an additional method to reduce A-MAP false detection. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text to Assignment A-MAP IE (Section 16.3.6.5)

Hyunkyu Yu, Seho Kim, Youngbo Cho, Heewon Kang

Samsung Electronics

- Introduction

This contribution proposes a method to reduce A-MAP false detection using reserved bits in assignment A-MAP IE.

The AMS attempts to decode all assignment A-MAP IEs using its station ID or other IDs (group ID, etc). Let’s suppose that decoding errors happen for a certain assignment A-MAP IE which is not intended to that AMS. But that error pattern and masking ID pattern can cause to pass the CRC check even if there’re decoding errors. To prevent this false detection, AMS can additionally check the error using reserved bits in assignment A-MAP IE.

If fixed bit pattern is used for reserved bits, AMS can check errors using position and value of those reserved bits. But when error pattern doesn’t change position and value of reserved bits, AMS cannot check the error additionally.

Thus, we propose to fill reserved bits using masking sequence (station ID, etc) [1]. This method enables to check errors even when error pattern doesn’t change position and value of reserved bits.

- Proposed text with markups

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

- References

[1] IEEE C802.16m-09/2988, “Proposed change to RAID and CDMA allocation AMAP IE (16.2.11.2)”

- Proposed Text Changes

[Remedy#1: Add the following text, at line 36 in page 395, section 16.3.6.5.2.4]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.5.2.4. Assignment A-MAP IE

Table 811 describes Assignment A-MAP IE types.

Table 811 – Assignment A-MAP IE types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | DL Subband Assignment A-MAP IE |
| 0b0011 | UL Subband Assignment A-MAP IE |
| 0b0100 | Feedback Allocation A-MAP IE |
| 0b0101 | UL Sounding Command A-MAP IE |
| 0b0110 | CDMA Allocation A-MAP IE |
| 0b0111 | DL Persistent Allocation A-MAP IE |
| 0b1000 | UL Persistent Allocation A-MAP IE |
| 0b1001 | DL Group Resource Allocation A-MAP IE |
| 0b1010 | UL Group Resource Allocation A-MAP IE |
| 0b1011 | Feedback Polling A-MAP IE |
| 0b1100 | BR-ACK A-MAP IE |
| 0b1101 | Broadcast Assignment A-MAP IE |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

“Reserved” Field in Assignment A-MAP IE

In each assignment A-MAP IE, one or multiple reserved fields are present according to types and contents of assignment A-MAP IEs. Those reserved fields are processed as follows.

- Suppose there are p reserved bits, r0 r1 … rp-1, in an assignment A-MAP IE, where r0 is the MSB bit in the first reserved field and rp-1 is the LSB bit in the last reserved field.

- Put the masking sequence bits, m0 m1 … m15, into reserved bits: rk=mmod(k, 16), where m0 is the LSB bit of the masking sequence.

-------------------------------------------------End proposed text----------------------------------------------------------------