| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification to DL/UL Persistent Allocation (Section 16.3.6.5) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Hyunkyu Yu, Seho Kim, Mihyun Lee, Ji-Yun Seol Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot#30b” Target topic: “IEEE P802.16m/D3 section 16.3.6.5”. |  |
| Abstract | The contribution provides clarification on DL/UL persistent allocation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification to DL/UL Persistent Allocation (Section 16.3.6.5)

Hyunkyu Yu, Seho Kim, Mihyun Lee, Ji-Yun Seol

Samsung Electronics

- Introduction

This contribution provides clarification on the operation of DL/UL persistent allocation.

(1) N_ACID: Number of ACIDs for implicit cycling of HARQ channel identifier

- In D3 [1], N_ACID is calculated based on the maximum HARQ retransmission delay and allocation period for both DL and UL. However, DL is not proper to use that equation because HARQ process is performed in an asynchronous manner. The time interval between consecutive retransmissions is not defined in DL. Thus, we recommend to use the current equation only for UL and to introduce explicit N_ACID field in DL PA A-MAP IE.

- To include N_ACID field in DL PA A-MAP IE, 6 bits HFA field (allocation case) is reduced to 3 bits for both DL and UL. Reserved bits may help to decrease the A-MAP false detection (UL PA is more critical).

(2) Simple clarification for HARQ cycling mechanism

- Proposed text with markups

Blue/Underline: Text Added

Red/Strikeout: Text Deleted

- References

[1] IEEE P802.16m/D3, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2009-12-08

- Proposed Text Changes

[Remedy#1: Modify the text, line 1 in page 430, section 16.3.6.5.2.4.8]

--------------------------------------------------Start Proposed Text-------------------------------------------------------------

16.3.6.5.2.4.8 DL PA A-MAP IE

The DL persistent allocation A-MAP IE is specified in Table 823.

Table 823 - DL Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent Allocation A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | DL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| If (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation!= 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Mt <= Nt Nt: Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| Si | 4 | Index to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 63 | Explicit Index for HARQ Feedback Allocation |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. 0b001: 2 0b010: 3 0b011: 4 0b111: 8 |
| Reserved | 1 | Reserved bit |
| Initial_ACID | 4 | HARQ channel identifier. The ACID field shall be set to the iInitial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 |
| Reserved | 2 | Reserved bit |
| } | - | - |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the DL Persistent Allocation A-MAP IE and the CRC is masked by Station ID.

The Resource Index field in the DL Persistent Allocation A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE.

The maximum HARQ retransmission delay for persistent allocation, PA_Max_ReTx_Delay can be computed from N_Max_ReTx, the maximum number of retransmission and PA_ReTx_Interval, the allowable delay between consecutive retransmission of persistent allocation as follows.

PA_Max_ReTx_Delay = N_Max_ReTx * PA_ReTx_Interval

where PA_ReTx_Interval is determined from Long TTI Indicator, Tproc, the data burst processing time, and Frame_length, the frame length as follows.

If Tproc3 and Long_TTI_Indicator = 0,

PA_ReTx_Interval = Frame_length,

Otherwise, PA_ReTx_Interval = 2*Frame_length.

- 16.3.6.5.2.4.9 UL PA A-MAP IE

The UL persistent allocation A-MAP IE is specified in Table 812.

Table 824 - UL Persistent Allocation A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent Allocation A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation Period != 0b00){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 1 | Number of streams in transmission (Mt <= Nt ), up to 2 streams per AMS supported 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ |  | Parameters for CSM |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4 streams: 0b00, 0b01, 0b10, 0b11 |
| } else if (TNS == Mt) { |  | Parameters without CSM |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding |
| Reserved | 1 | Reserved bits |
| } |  |  |
| PF | 1 | Precoding Flag 0b0: non adaptive precoding 0b1: adaptive precoding using the precoder of rank Mt of AMS’s choice |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 63 | Explicit Index for HARQ Feedback Allocation |
| Initial_ACID | 34 | Initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers.HARQ channel identifier N_ACIDs: Number of ACIDs for implicit cycling of HARQ channel identifier N_ACID=Floor{ PA_Max_ReTx_Delay/ (Allocation Period*Frame_length) }+1 N_ACID=Min(8, Floor{UL_MAX_ReTx_Delay/(Allocation Period∙Frame_length)}+1) |
| Reserved | 24 | Reserved bits |
| } | - | - |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Persistent A-MAP IE and the CRC is masked by Station ID.

The Resource Index field in the UL Persistent A-MAP IE is interpreted as in the DL Basic Assignment A-MAP IE with ‘DL’ specific terminology replaced by ‘UL’ equivalents.

The UL maximum HARQ retransmission delay for persistent allocation, UL_Max_ReTx_Delay can be computed from UL_N_MAX_ReTx, the maximum number of retransmission defined in 16.2.14.2.1.2 and UL_ReTx_Interval, the time interval between consecutive retransmission as follows.

UL_MAX_ReTx_Delay = UL_N_MAX_ReTx∙UL_ReTx_Interval

where UL_ReTx_Interval = (1+w+v)∙Frame_length where w is UL HARQ feedback offset, v is UL HARQ transmission offset defined in 16.2.14.2.2 and Frame_length is the length of the frame.

---------------------------------------------------End Proposed Text-------------------------------------------------------------

[Remedy#2: Modify the text, line 64 in page 372, section 16.3.6.3.2.2]

--------------------------------------------------Start Proposed Text-------------------------------------------------------------

For deallocation of persistent allocation, the HF-A-MAP resource index is specified in the HFA of UL Persistent Allocation A-MAP IE.

…

For resource allocation using the UL basic assignment A-MAP IE, UL persistent allocation A-MAP IE, and CDMA Allocation A-MAP IE, the HF-A-MAP resource index is (M(j)+n) mod NHF-A-MAP, where j is HF-A-MAP Index Parameter in the Non-user specific A-MAP IE, n is 3 bit HFA value in UL basic each assignment A-MAP IE, NHF-A-MAP is the total number of HF-A-MAP channels configured per HF-A-MAP region. M(j) is STID for the UL basic assignment A-MAP IE and RA-ID for CDMA Allocation A-MAP IE when j=0 and M(j) is lowest LRU index of the corresponding UL transmission when j=1. For UL persistent allocation A-MAP IE, M(j) is always STID regardless of value j.

---------------------------------------------------End Proposed Text-------------------------------------------------------------

[Remedy#3: Modify the text, line 7 in page 508, section 16.3.8.3.3.2]

--------------------------------------------------Start Proposed Text-------------------------------------------------------------

For deallocation of persistent allocation, index k is specified in HFA of the DL Persistent Allocation A-MAP IE.

…

For resource allocation using the DL Basic Assignment A-MAP IE, DL persistent allocation A-MAP IE, Feedback Allocation A-MAP IE, and Feedback Polling A-MAP IE, the index k is (M(j)+n) mod LHFB, where j is HFBCH Index Parameter in the Non-user Specific A-MAP IE, n is 3 bit HFA value in DL Basic each Assignment A-MAP IE, LHFB is the total number of HFBCH configured per HARQ feedback region, M(j) is STID when j = 0 and M(j) is lowest LRU index of corresponding DL transmission when j = 1. For DL persistent allocation A-MAP IE, M(j) is always STID regardless of value j.

---------------------------------------------------End Proposed Text-------------------------------------------------------------

[Remedy#4: Modify the text, line 12 in page 181, section 16.2.7.1]

--------------------------------------------------Start Proposed Text-------------------------------------------------------------

16.2.7.1 Allocation Mechanism

For persistent allocation in the DL/UL, the ABS shall transmit the DL/UL Persistent Allocation A-MAP IE. Allocation of the persistently assigned resource begins in the DL/UL AAI subframe that is referenced by the DL/UL Persistent Allocation A-MAP IE and repeats after an allocation period that is specified in the DL/UL Persistent Allocation A-MAP IE. The configuration parameters of the persistently allocated resource are maintained as per the DL/UL Persistent Allocation A-MAP IE. The values of ACID field and N_ACID field in the DL/UL Persistent Allocation A-MAP IE are used together to specify an implicit cycling of HARQ channel identifiers. The allocation period and number of ACIDs required for persistent operation are configured in the DL/UL Persistent Allocation A-MAP IE.

The values of ACID and N_ACID configured in the DL/UL Persistent Allocation A-MAP IE are used together to specify an implicit cycling of HARQ channel identifiers. At the initial transmission with the DL/UL Persistent Allocation A-MAP IE, the ACID of the HARQ burst is set to the value specified in the ACID field of the DL/UL Persistent Allocation A-MAP IE. From the next new transmission, the ACID of the HARQ burst is incremented by 1, and cycled within the range of [Initial_ACID, Mod (Initial_ACID+N_ACID, 16)], where the Initial_ACID is the starting ACID value at the initial transmission. If the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, the retransmission process for the previous HARQ burst is terminated and the new HARQ burst overrides it.

---------------------------------------------------End Proposed Text-------------------------------------------------------------