| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on MCRC masking operations (16.2.3.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Kelvin Chou MediaTek Inc. Daniel Cohn Alvarion | kelvin.chou@mediatek.com daniel.cohn@alvarion.com |
| Re: | Letter Ballot #30b on the Draft Amendment (IEEE P802.16m/D3) |  |
| Abstract | IEEE 802.16m/D3 specifies a masking operation to construct the MCRC which is not defined in the standard |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on MCRC masking operations (16.2.3.1)

Kelvin Chou

MediaTek Inc.

Daniel Cohn

Alvarion

Motivation

Several A-MAP IE messages include the MCRC field, which is defined as the message 16-bit CRC masked by either the STID or by the RA-ID and masking indicator (depending on the IE type). The standard does not define the masking operation, although it seems clear that intention was to refer to a logical exclusive OR operation which would allow the destination MS to recover original CRC after applying the masking operation again. This is especially confusing since masking is usually referred to a logical AND operation.

Suggested Modifications

In P802.16m/D3, Page 41, Line 5, modify the following sentence:

-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

In response to the additional UL bandwidth request, ABS should provide UL bandwidth by CDMA Allocation A-MAP IE still masked with the RA-ID and masking indicator for the ranging code (refer to 15.2.15.3.1), using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 209, Line 16, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

AMS decodes the IE and checks the MCRC field by performing the bitwise exclusive OR operation with its specific 12-bit RA-ID and 4-bit Masking Indicator.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 209, Line 63, Table 743, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

CRC masked by RA-ID and Masking Indicator, using the bitwise exclusive OR operation

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 374, Line 29, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

A 16-bit CRC is generated based on the contents of the A-MAP IE excluding the station ID. The CRC is then masked by the Station ID using the bitwise exclusive OR operation

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 397, Line 34, modify the following sentence:
------------------------------------------------------------------------ Text Start -------------------------------------------------------------------------

*A 16 bit CRC is generated based on the contents of the DL Basic Assignment A-MAP IE. The CRC is masked by the Station ID, using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 402, Line 60, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID, using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 427, Line 9, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

*A 16 bit CRC is generated based on the contents of the Feedback Allocation A-MAP IE. The CRC is masked by the Station ID, using the bitwise exclusive OR operation.

------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 429, Line 29, modify the following sentence:
------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

*A 16-bit CRC is generated based on the contents of the UL Sounding Channel Command A-MAP IE. The CRC is masked by the Station ID, using the bitwise exclusive OR operation.

------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 432, Line 22, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

*A 16 bit CRC is generated based on the contents of the DL Persistent Allocation A-MAP IE. The CRC is masked by the Station ID, using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 434, Line 56, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

*A 16 bit CRC is generated based on the contents of the UL Persistent Allocation A-MAP IE. The CRC is masked by the Station ID, using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 435, Line 60, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

A 16-bit CRC shall be generated based on the contents of each segment. The CRC is masked with the Group ID of the corresponding group, using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 436, Line 62, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

A 16-bit CRC shall be generated based on the contents of each segment. The CRC is masked with the Group ID of the corresponding group, using the bitwise exclusive OR operation.

------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 441, Line 26, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

*A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID, using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------

In P802.16m/D3, Page 443, Line 11, modify the following sentence:
-------------------------------------------------------------------------- Text Start -------------------------------------------------------------------------

*A 16 bit CRC is generated based on the contents of the BR-ACK A-MAP IE. The CRC is masked by the reserved Station ID for BR-ACK A-MAP, using the bitwise exclusive OR operation.

-------------------------------------------------------------------------- Text End -------------------------------------------------------------------------