| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for E-MBS Configuration Indicators (P802.16m/D3-16.9.3.1) |  |
| Date Submitted | 2010-01-07 |  |
| Source(s) | Kaiying Lv, Nan Li, Bo Sun ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: lv.kaiying@zte.com.cn |
| Re: | Category: LB30b / Area: Chapter 16.9.3.1.( E-MBS Configuration Indicators) |  |
| Abstract | This contribution proposes modifications on E-MBS Configuration Indicators |  |
| Purpose | To discuss and adopt the proposed text in TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed text for E-MBS Configuration Indicators

- Kaiying Lv，Nan Li，Bo Sun

- ZTE Corporation

- Proposed Text Changes

-------------------------------------------------------Start of the Text --------------------------------------------------------

[Make the following changes to Section16.9.3.1 in page643 beginning Line 3 in P802.16m/D3. ]

16.9.3.1 E-MBS Configuration Indicators

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-E-MBS-CFG_Message_Format() { |  |  |
| Num_E-MBS_Zones | 3 | Number of E-MBS Zones with which the ABS is associated. |
| E-MBS subframe indicator | 3 | Number of AAI subframes reserved for E-MBS in the DL zone within a frame 0b000: Last 1 subframe of the DL zone 0b001: Last 2 subframe s of the DL zone 0b010: Last 3 subframe s of the DL zone 0b011: Last 4 subframe s of the DL zone 0b100: Last 5 subframes of the DL zone 0b101: Last 6 subframes of the DL zone 0b110: Last 7 subframes of the DL zone 0b111:Last 8 subframes of the DL zone |
| Total E-MBS Resource | Variable | Total number of subbands allocated to EMBS The size depends on system bandwidth as below: 2 bits for 5 MHz 4 bits for 10 MHz 5 bits for 20 MHz |
| for (i = 0; i< Num_E-MBS_Zones; i++) { |  |  |
| E-MBS_Zone_ID | 7 | The EMBS_Zone_ID to which this EMBS MAP applies. |
| Resource indicator | Variable | Frequency Resource indication Resource allocation Index for EMBS zone ID The size depends on total number of subbands allocated to EMBS (N), that is Log2（N*（N+1）/2） |
| MSI Length | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframe, 40ms 0b01: 4 superframe, 80ms 0b10: 8 superframe, 160ms 0b11: 16 superframe, 320ms |
| E-MBS MAP Resource Index | [TBD] 11 | Resource index includes location and allocation size. |
| E-MBS MAP ISizeOffset | 5 |  |
| Service Continuity Indicator | 1 | Service Continuity indication 0: no inter-zone service continuity in neighbor ABS 1: inter-zone service continuity |
| If (Service Continuity Indicator = =1) |  |  |
| Neighboring E-MBS Zone ID | 7 |  |
| Physical Carrier Index |  | Physical carrier Index of the corresponding neighboring E-MBS Zone |
| MSTID + FID mapping indication | 1 | Indicate preallocation or continuity of MSTID+FID mappings 0: preallocation 1: continuity |
| If (MSTID + FID mapping indication = = 0) |  |  |
| List of MSTID+FID Mappings |  | Current MSTID+FID(1); New MSTID+FID(1); Current MSTID+FID(2); New MSTID+FID(2); …. |
| If (MSTID + FID mapping indication = = 1) |  |  |
| List of MSTID+FID Mappings |  | Current MSTID+FID(1); Current MSTID+FID(2); …. |
| } |  |  |
| } |  |  |

Total E-MBS Resource:  Indicates the total number of subbands allocated to EMBS zones to which the BS belongs. The size of this field depends on system bandwidth.

Resource indicator:      Indicates frequency resources reserved for the corresponding E-MBS zone. It is a resource allocation Index with variable size which depends on total E-MBS resources. When “Total E-MBS Resource” is N, the size of this field is log2(N∗(N+1)2)

The resource indicator is constructed using the following rules:

- the SLRUs reserved for E-MBS in an AAI-subframe in each frequency partition shall be mapped into one contiguous logical resource space in unit of subband.

Let

N denotes the total number of subbands reserved for E-MBS.

NFPi denotes the number of subband for E-MBS in frequency partition i.

SBFPi(j) denotes a subband in frequency partition i.

CSB(k) denotes a subband in contiguous logical resource space.

k(i,j) = j，if i=0；

where, NFP0 is the number of subband for E-MBS in frequency partition 0.

When FPCT=3,

k(i,j) = i + 3*j,   i>0

- Resource allocation Index is the index of a Look-up table. Each entry with index m of the Look-up table represents a subset of contiguous CSBs. The subset of contiguous CSBs is denoted by (x,y), where x +1 represents the number of subbands allocated to an E-MBS zone, and y represents the starting CSB allocated to an E-MBS zone.

m (x，y）= k=0x(N−k+1)+y

where,  0 < m < [N*（N+1）/2],   0 < x < N,  0 < y < N-x

---------------------------------------------------------End of the Text---------------------------------------------------------

Reference

[1] IEEE P802.16m/D3