| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the Dedicated Preamble Code for Ranging Channels in IEEE P802.16m/D3 (16.3.9) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, and HanGyu Cho LG Electronics | Voice: +82-31-450-7902 E-mail: hyunwoolee.lee@lge.com, jinsam.kwak@lge.com, hg.cho@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30b P802.16m/D3 : 16.3.9.2.4.1, 16.3.9.2.4.2 and 16.3.9.2.4.3 |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE P802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of the Dedicated Preamble Code for Ranging Channels in IEEE P802.16m/D3 (16.3.9)

HyunWoo Lee, Jin Sam Kwak, and HanGyu Cho

LG Electronics

# 1. Introduction

The generation method of dedicated preamble code for ranging channels is not fully defined yet. This contribution proposes the modified text of ranging channel section for dedicated preamble code in the IEEE P802.16m/D3 [1].

# Reference

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” December 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 16.3.9.2.4 of P802.16m/D3

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

-----------------	Remedy #1 : 	16.3.9.2.4.1 Ranging channel for non-synchronized AMSs	------------------

---------------			Page 531 / Line:19~33						------------------

p	is the index for pth ranging preamble code which is made as the spth cyclic shifted sequence from the root index rp of Zadoff-Chu sequence.

(259)

(260)

where rns0 is broadcasted in the S-SFH and Mns is the number of cyclic shifted codes per ZC root index according to Table 888. NTOTALcont is the total number of initial (0~NIN -1) and handover preamble codes (NIN ~NIN +NHO -1) per sector for contention-based approach. Ndedi is the total number of dedicated handover preamble code (Ncont~ Ncont+Ndedi-1) per sector.

-----------------	Remedy #2 : 	16.3.9.2.4.2 Ranging channel for synchronized AMSs	--------------------------

---------------			Page 533 / Line:53 ~ Page 534 / Line:5			---------------------------

p	is the index for pth ranging preamble code within a basic unit which is made as the sp th cyclic shifted sequence from the root index rp of Zadoff-Chu sequence.

sp=mod(p,Ms),p=0,1,...,NTOTAL−1

The start root index, rs0, is broadcasted. Ms is the number of cyclic shift per ZC root index and defined by Ms=1G. NTOTALcont is the number of periodic ranging preamble codes (0~NPE -1) per sector, which is defined by Table 891 for contention-based approach. For femtocell, NTOTALcont is the total number of initial (0~NIN -1), handover (NIN ~NIN +NHO -1) and periodic ranging preamble codes (NIN +NHO ~ NIN +NHO+NPE -1) per sector which is defined by Table 892 for contention-based approach. Ndedi is the total number of dedicated handover preamble code (Ncont~ Ncont+Ndedi-1) per sector.

-----------------	Remedy #3 : 	16.3.9.2.4.1 Ranging channel for FDM-based PUSC Zone support	---------

---------------			Page 536 / Line:48~61							---------

The number of available codes is 256, numbered 0, ... , 255. Each ABS uses a subgroup of these codes, where the start index of subgroup is defined by a number S, S = 16×kns+1 where kns (= 0, 1, 2, …, or 15) is a cell specific value broadcasted by S-SFH. The group of codes shall be between S and ((S + O + N + M + D) mod 256).

— The first N codes produced are for contention-based initial ranging. Clock the PRBS generator 144 × (S mod 256) times to 144 × ((S + N) mod 256) – 1 times.

— The next O codes produced are for contention-based handover ranging. Clock the PRBS generator 144 × ((S + N) mod 256) times to 144 × ((S + N + O) mod 256) – 1 times.

— The next M codes produced are for contention-based periodic ranging. Clock the PRBS generator 144 × ((S + N + O) mod 256) times to 144 × ((S + N + O + M) mod 256) – 1 times.

— The next D codes produced are for dedicated handover ranging. Clock the PRBS generator 144 × ((S + N + O + M) mod 256) times to 144 × ((S + N + O + M + D) mod 256) – 1 times.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------