| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging procedure in coverage loss declaration (section 16.2.26) |  |
| Date Submitted | 2010-01-12 |  |
| Source(s) | Chunmei Liu, Masoud Olfat Clearwire Yeongmoon Son Samsung Electronics | Chunmei.liu@clearwire.com Masoud Olfat@clearwire.com Ym1004.son@samsung.com |
| Re: | Proposed text modifications on coverage loss Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.26” |  |
| Abstract | Proposed text modifications on coverage loss in the draft P802.16m/D3 amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Ranging procedure in coverage loss declaration

Chunmei Liu and Masoud Olfat

Clearwire

Yeongmoon Son

Samsung Electronics

# Introduction

In normal network deployments, after declaring a coverage loss for an AMS, a typical action of the ABS is to stop services to the AMS. Hence unnecessary loss declarations must be avoided as they directly deteriorate user experience and operator’s services.

In 16m/D3, the ABS declares that the AMS is in a coverage loss mainly by failing to receive UL PDUs upon successive UL grants. The ranging procedure after failure of UL PDUs is optional instead of mandatory before the ABS declares a loss.

However in actual network deployments, when the ABS does not receive UL PDUs as expected, the reason could be that the transmission parameters used at the AMS is not appropriate, which can be simply corrected by a ranging procedure. For example, upon failure UL PDUs, with certain probability esp. at the edge of a loss, the UL link could be restored by simply increasing UL transmission power. In these cases a loss declaration immediately after failure of UL PDUs and without a ranging procedure is unnecessary.

Please note that the existing periodic ranging procedure does adjust transmission parameters periodically. However, it does not guarantee that the parameters are always appropriate between two periodic ranging procedures.

Hence, the ranging procedure before declaring a loss shall be mandatory instead of optional. That is, instead of declaring a loss immediately after failure of receiving expected UL bursts, the ABS shall request a ranging procedure.

# Proposed Amendment text

Black text: existing text in AWD (P802.16m/D3)

Red Strikethrough text: deleted text

Blue text: new text

Proposed text #1

[Modify the text from page 287, line 55 to page 288, line 10 as follows]

======================== Start of Proposed Text =====================

(16.2.26.1 Coverage loss detection at ABS and ABS’s behavior)

If the ABS does not receive padding PDU or MPDU on a predetermined number of successive UL grants (e.g., 10), under certain conditions the ABS may shall send an unsolicited AAI-RNG-RSP message to request the AMS to perform periodic ranging, as described below. The conditions are TBD.

The ABS may shall send unsolicited AAI-RNG-RSP message to request the AMS to perform ranging using periodic ranging codes by setting the Ranging Request bit to one. Upon receiving an AAI-RNG-CFM message with AMS’s STID, which indicates a successful periodic ranging initiated by this unsolicited AAI-RNG-RSP, the ABS shall respond with AAI-MSG-ACK and restart the active_ABS_timer. If the ABS does not receive the AAI-RNG-CFM message upon the expiration of some timer (the value of this timer is TBD) upon the expiration of the retry timer for the AAI-RNG-RSP message, the ABS resends the AAI-RNG-RSP message until its retry limit is reached. Upon exhausted AAI-RNG-RSP retries, the ABS shall start the Resource Retain Time.

If the ABS does not receive any padding PDU or MPDU on a predetermined number of successive UL grants (e.g., 10), and if the ranging procedure is not requested, the ABS shall regard the AMS in coverage loss, stop DL/UL scheduling for that AMS, and start the Resource Retain Time.

Once Resource Retain Time is started, the ABS shall not restart the active-ABS-timer.

======================== End of Proposed Text =====================

Proposed text #2

[Modify the text from page 288, line 36 to line 41 as follows]

======================== Start of Proposed Text =====================

(16.2.26.2 Coverage loss detection at AMS and AMS’s behavior)

The AMS can detect a coverage loss when it loses PHY synchronization or DL synchronization or UL synchronization, i.e., if the AMS cannot decode a predetermined number of contiguous (e.g., 5) SFHs, the AMS shall regard it as Link Loss from the ABS.

Under certain conditions the ABS may send an unsolicited AAI-RNG-RSP message to request the AMS to perform periodic ranging, as described below. The conditions are TBD.

In case the ABS performs the ranging processing, tThe AMS upon receiving an unsolicited AAI-RNG-RSP message with Ranging Request bit to be one, which indicates that the ABS requests ranging procedure to detect coverage loss,  the AMS shall perform periodic ranging. Upon successful ranging initiated by an unsolicited AAI-RNG-RSP message with Ranging Request bit to be one, indicated by receiving an AAI-RNG-ACK message with a success status and corresponding periodic ranging code, the AMS shall request bandwidth and send back a AAI-RNG-CFM message that includes its STID and MCEH with polling bit set to 1, which requests the ABS to respond with AAI-MSG-ACK. If the AMS does not receive the AAI-MSG-ACK message upon the expiration of the retry timer for the AAI-RNG-CFM message, the AMS resends the AAI-RNG-CFM message until its retry limit is reached. Upon exhausted HARQ retransmissions of the AAI-RNG-CFM message retries, the AMS considers that the ABS considers that the AMS is not with the ABS anymore, and the AMS shall perform coverage loss recovery procedure as indicated in Section 16.2.26.3.

======================== End of Proposed Text =====================