| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for E-MBS dynamic multicast operation in Section 16.9.2 |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yongjoo Tcha, Kyung-ah Kim KT | Email : yjtcha@kt.com kka1@kt.com |
| Re: | Contribution in support of a comment on P802.16m/D3 currently under LB#30b |  |
| Abstract | This contribution defines the text and format for E-MBS MAP control information elements |  |
| Purpose | For review and adoption by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for E-MBS dynamic multicast operation in Section 16.9.2

Yongjoo Tcha, Kyung-ah Kim

KT

References

[1] C80216m-09_2337r3, “Dynamic Multicast Service Support for the IEEE 802.16m E- MBS”

---------------------------------------------------- Proposed Text ------------------------------------------------

On the IEEE P802.16m/D3, page 643, line 28, add the following subsections

-----------------------------------------------------------Start of the Text---------------------------------------------------------

16.9. Support for Enhanced Multicast Broadcast Service

16.9.2.3 E-MBS Operation in Idle State

When an AMS in Idle mode moves to an ABS which does not belongs to AMS' previous E-MBS Zone, the AMS is expected to update the E-MBS service flow management encodings at that ABS to provide continuous reception of E-MBS content. The AMS may obtain the E-MBS information in the target E-MBS zone through broadcast messages in the EMBS-Zone of the serving ABS. If the idle AMS has not received such information from the serving E-MBS Zone, the AMS shall use location update procedure to acquire updated E-MBS service flow management encodings. In order to perform the MBS location update process, the AMS shall transmit AAI_RNG-REQ message with Ranging Purpose Indication Bit # 4 set to 1. In response to the request for MBS location update, the ABS shall transmit AAI_RNG-RSP message which may include the E-MBS zone identifier, MSTID, FID, and E-MBS Zone Identifier Assignment parameter and etc. to provide update service flow management encodings for any affected E-MBS flow(s).

The AMS may also conduct re-entry from Idle mode if required by quality of service of corresponding EMBS flow.

In order to improve the efficiency of radio resource utilization, dynamic multicast service may be supported, in which the transmission of the multicast service data should be decided based on the number of the AMSs within the ABS. Dynamic multicast service may be supported regardless of the AMS operating state – Connected State and Idle State.

16.9.2.4 Dynamic Multicast Operation

In order to improve the efficiency of radio resource utilization, dynamic multicast service may be supported, in which the transmission of the multicast service data should be decided based on the number of the AMSs within the ABS. Dynamic multicast service may be supported regardless of the AMS operating state – Connected State and Idle State.

The ABS may not transmit the E-MBS data when there is no AMS participating in the E-MBS connection in the coverage.  In order to support mobility and interference mitigation between ABSs, the transmission of E-MBS data should be turned on or off according to the number of the AMSs within the ABS and the neighboring ABSs.

The messages of the number of the AMSs within the ABS may be exchanged between neighboring ABSs through the backhaul network. The messages and procedures are out of scope of this document.

-----------------------------------------------------------End of the Text---------------------------------------------------------