| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Adding MAC layer CRC to improve reliability (16.2.2, 16.2.3.41.1, 16.2.3.41.2, 16.2.3.41.4, 16.2.3.41.5) |  |
| Date Submitted | 2010-01-07 |  |
| Source(s) | Muthaiah Venkatachalam, Aran Bergman, Frishman, Yaniv | Intel Corporation |
| Re: | Adding MAC layer CRC to improve reliability |  |
| Abstract | Adding MAC layer CRC to improve reliability |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Adding MAC layer CRC to improve reliability

# Introduction

According to 802-2001 [1], section 7.3, the error misdetection probability for a 802 protocol (at least for data, I guess), shall be less than 5x10^-14 per octet or MSDU length. It would seem that the 16-bit CRC at the burst level is not sufficient to meet this requirement.

To reduce the error misdetection probability, need to add 2 or 4 byte CRC. Whether to add the CRC or not - based on FID configuration (DSA).

Reference:

[1] IEEE 802.2001: IEEE Standard for Local and Metropolitan Area Networks: Overview and Architecture, 8 March 2002

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

16.2.2 MAC PDU formats

MAC PDUs shall be of the form illustrated in Figure 385. Each PDU shall begin with a MAC header. The header may be followed by one or more extended headers. The MAC PDU may also contain payload. The MAC PDU may also contain a 4B CRC if it is agreed upon during the connection establishment that CRC shall be included. The CRC shall cover the generic MAC header and the payload of the MAC PDU. The 32 bit CRC shall be generated according to section 6.3.3.5.2

Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS can be multiplexed into a single MAC PDU. The multiplexed unicast connections shall be associated with the same security association.

[Replace Figure 385 as follows]

Figure 385 – MAC PDU format

16.2.3.41.1 AAI_DSA-REQ

[Add the following parameter to the message]

| Name | Value | Usage |
| --- | --- | --- |
| CRC Indicator (1 bit) | 0 = No CRC 1 = 4Byte CRC | Indicates what CRC (if any) should be added to all the PDUs of this FID |

16.2.3.41.2 AAI_DSA-RSP

[Add the following parameter to the message]

| Name | Value | Usage |
| --- | --- | --- |
| CRC Indicator (1 bit) | 0 = No CRC 1 = 4Byte CRC | Indicates what CRC (if any) should be added to all the PDUs of this FID |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]