| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN! |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty, Avishay Shraga, Pouya Taaghol | Intel Corporation |
| Re: | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN! |  |
| Abstract | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN! |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN!

# Introduction

Operators have WiMAX 1.0/1.5 networks in deployment today. This uses 802.16e air interface. 802.16m air interface promises much higher throughputs compared to 802.16e. Some more improvements are achieved in 16m by changing UMAC functionality that affects the ANG-GW and in particular R6 interface. Expecting the operators and vendors to upgrade the ASN-GW in order to start deploying 16m BSs will slow down the 16m adoption. The goal is to standardize a mechanism that allows operators to upgrade to 802.16m from 802.16e in phases where the first phase is using 16m BS without any changes to the existing network infrastructure and in particular no change on R6. No changes to ASN-GW and other network elements

In summary, we enable a model where 16m BSs can be simply plugged into the existing WiMAX ASN-GWs.

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Change-1:  Add “legacy ASN” bit to SFH]

16.3.6.5.1.2 	S-SFH IE

[Add the following parameter to table 802, S-SFH SP2 IE page 379]

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Legacy ASN indicator | 1 | Indicates whether the ABS is connected to legacy ASN or advanced ASN. Value of “0” means ABS is connected to advanced ASN. Value of “1” means ABS is connected to legacy ASN. |

[Change-2: Usage of PKMv2]

16.2.5.1 	Security Architecture

802.16m uses PKMv3 as the main security mode in the advanced air interface.

However, in the legacy ASN mode,

- AMS and ABS shall use single mode of PKMv2 Security frame work for authentication and key management (EAP Authentication, CMAC, PKMv2 messages).

- All other MAC control messages shall be protected according to PKMv3 definitions using the PKMv2 SAs.

- Data plane encryption shall be done according to 16m definitions using the PKMv2 derived keys.

- Concept of MSID* and Temp STID shall not be used.

  - AMS to provide real MAC ID in AAI_RNG-REQ

  - AMS to be given actual STID in AAI_RNG-RSP

16.2.15.3.1 Contention-based initial ranging and automatic adjustments

[Modify the last 3 bullets of this section as follows:]

•Upon receiving a Success status notification, the AMS shall wait for the ABS to provide UL BW allocation. When receiving an UL BW allocation, the AMS shall send the AAI_RNG-REQ message. If the granted BW allocation cannot accommodate the entire AAI_RNG-REQ message, the AMS may fragment the AAI_RNG-REQ message to fit the provided BW allocation, and requests additional UL bandwidth through PBREH as defined in 15.2.2.2.9 for the remaining fragments. In response to the PBREH, the ABS shall provide UL BW allocation through CDMA Allocation A-MAP IE, which is identified by the same RA-ID used for the previous BW allocation. The RA-ID is used until AAI_RNG-REQ transmission is completed, but if AMS does not receive UL bandwidth allocation in T3(TBD) or the AAI_RNG-REQ transmission is not completed in 64 frames, it sends ranging code to perform ranging procedure again. In the legacy ASN mode, where the ABS is connected to the legacy ASN, the AMS provides its real MAC ID in the AAI_RNG-REQ message.

•Initial ranging process is over after receiving AAI_RNG-RSP message, which includes a TSTID (temporary station ID) to be used until STID is received at successful registration. In the legacy ASN mode, where the ABS is connected to the legacy ASN, the actual STID of the AMS is provided in the AAI_RNG-RSP message.

•The timer required for AMS to wait for AAI_RNG-RSP or CDMA Allocation A-MAP IE, is defined by T3 (TBD).

16.2.3.1 AAI_RNG-REQ

[Modify the table 674 as follows:]

Table 674—Parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| AMSID*/ MAC ID | It's the hash value of AMSID in order to protect AMS privacy, which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. In the legacy ASN mode, where the ABS is connected to the legacy ASN, the AMS provides its real MAC ID instead. | It shall be included when the AMS is attempting network entry without its STID/DID which the ABS/Paging Controller assigns. |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |

16.2.3.2 AAI_RNG-RSP

[Modify the table 675 as follows:]

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Status | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success | It shall be included in the AAI_RNG-RSP message |
| Temporary STID | Used for AMS identification until STID is assigned to the AMS during registration procedure. In the legacy ASN mode, where the ABS is connected to the legacy ASN, the actual STID of the AMS is provided instead. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID/DID yet. |
| AMSID* | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. In the legacy ASN mode, where the ABS is connected to the legacy ASN, the actual MAC ID of the AMS is used instead. |  |

16.2.3.8 AAI_REG-RSP

An AAI_REG-RSP message is transmitted by ABS in response to AAI_ REG-REQ message during initialization.

The AAI_REG-RSP message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

The following parameter is included in the AAI_REG-RSP message:

—STID: It is used for AMS identification in place of the temporary STID which has been transferred by AAI-RNG-RSP message. In the legacy ASN mode, where the ABS is connected to the legacy ASN this parameter shall not be provided in the AAI_REG-RSP message.

[Change-3:  Add legacy ASN indication in HO CMD as a parameter.]

16.2.3.11 AAI_HO-CMD

[Add the following parameter table 683:]

| Name | Value | Usage |
| --- | --- | --- |
| Legacy ASN indicator | 1 bit Value of “0” means ABS is connected to advanced ASN. Value of “1” means ABS is connected to legacy ASN. | Indicates whether the target ABS is connected to legacy ASN or advanced ASN. |

16.2.6.3.4 HO Execution

[Add the following to the end of this section]

If the target ABS is attached to the legacy ASN, this shall be indicated to the AMS in the HO-CMD with the “Legacy ASN indicator” parameter. A value of 1 for this parameter signifies to the AMS that the target ABS is attached to the legacy ASN.

[Change-4: Add legacy ASN indication in AAI_PAG-ADV as a parameter.]

16.2.17.2.1 Broadcast paging message

[Add the following to the end of this section]

If the ABS is attached to the legacy ASN, this shall be indicated to the AMS in the AAI_PAG-ADV with the “Legacy ASN indicator” parameter. A value of 1 for this parameter signifies to the AMS that the ABS is attached to the legacy ASN. Under this scenario, the AMS index as used in 802.16-2009 shall be used for paging the AMS.

[Add the following to table 756]

| Name | Value | Usage |
| --- | --- | --- |
| Legacy ASN indicator | 1 bit Value of “0” means ABS is connected to advanced ASN. Value of “1” means ABS is connected to legacy ASN. | Indicates whether the ABS is connected to legacy ASN or advanced ASN. If attached to the legacy ASN, AMS index as used in 802.16-2009 shall be used for paging the AMS (instead of DID) |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]