| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN |  |
| Date Submitted | 2009-01-13 |  |
| Source(s) | Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty, Avishay Shraga, Pouya Taaghol, Jose Puthenkulam Masoud Olfat, David McGinnis, Chunmei Liu Shailesh Awasthy, Vinod Bhatia Oleg Marinchenco, Daniel Cohn, Vladimir Yanover Kiran Kuchi, Klutto Milleth Yih-Shen Chen, I-Kang Fu and Kelvin Chou Kalyan Pal, Raja Srinivas, BP Tiwari, Neeraj Sonker, Deepak Mittal Phillip Barber, Ronald Mao Ken Falkenstein, Asan Khan, Dave Urban Ming Hung Thao Rakesh Taori, Jungshin Park, Hokyu Choi, Hyunjeong Kang, Jung Je Son Mary Chion, Sean Cai, Huiying Fang, Yang Liu | Intel Corporation, Clearwire, Reliance Communications, Alvarion, CeWIT, MediaTek, Tata Communications, Huawei, Comcast, ITRI, Samsung Electronics, ZTE |
| Re: | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN! |  |
| Abstract | Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN! |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Upgrading the air interface from 802.16e to 802.16m without impacting the deployed ASN-GWs and CSN

# Introduction

Operators have WiMAX 1.0/1.5 networks in deployment today. This uses 802.16e air interface. 802.16m air interface promises much higher throughputs compared to 802.16e. Some more improvements are achieved in 16m by changing UMAC functionality that affects the ASN-GW and in particular R6 interface. Expecting the operators and vendors to upgrade the ASN-GW in order to start deploying 16m BSs will slow down the 16m adoption. The goal is to standardize a mechanism that allows operators to upgrade to 802.16m from 802.16e in phases where the first phase is using 16m BS without any changes to the existing network infrastructure and in particular no change on R6. No changes to ASN-GW and other network elements

In summary, we enable a model where 16m BSs can be simply plugged into the existing WiMAX ASN-GWs.

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Change-1 Add the following overview section and text to 802.16m draft]

16.12 Migrating to Advanced Air Interface without impacting the deployed legacy network

The migration to WirelessMAN OFDMA Advanced Air Interface may be done without impacting the deployed legacy network elements. The ABS should be able to connect to legacy access and core network elements. The ABS shall communicate to the AMSs that it is attached to the legacy network and the AMSs shall function in accordance to legacy network operations.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]