| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multiple BR-ACK IEs in a Frame |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | LB30b |  |
| Abstract | This contribution proposes solutions for dealing with the situation where an ABS sends multiple BR-ACK IEs in a frame. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multiple BR-ACK IEs in a Frame

Stavros Tzavidas

Motorola

# Introduction

The current text implies that an AMS that has performed BW request considers it has received an implicit NACK after it sees a BR-ACK IE which does not include the code it used. This can be problematic in case the ABS wishes to split the acknowldegments into multiple BR ACK IEs.

# Text Proposal

Modify paragraph 16.2.11.1.1 Contention-based random access bandwidth request as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

16.2.11.1.1 Contention-based random access bandwidth request

[text omitted]

If the BR-ACK Bitmap indicates no BR preamble sequence is detected at the BR opportunity selected by the AMS, or the AMS’s BR preamble sequence is not included at the selected BR opportunity in the BR-ACK A-MAP IE, the AMS shall consider that it has received a Negative-ACK. The AMS shall wait until the last DL subframe of a frame before deciding it has received an implicit Negative-ACK.

Upon successfully decoding the quick access message, the ABS may provide an UL grant to the AMS, using the STID and the BR index provided in the quick access message.

-------------------------------------------------- End of the text proposal --------------------------------------------------