| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text on Group Configuration MAC Control Message for Group Resource Allocation (16.2.3.44) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan Intel Corporation Hyunkyu Yu, Seho Kim Samsung Electronics Kevin Power Fujitsu Youngsoo Yuk, Jeongki Kim LG Electronics Yanfeng Guan, Xianming Chen, Huiying Fang ZTE Jisoo Park ETRI | Shweta.shrivastava@intel.com Hk.yu@samsung.com Kevin.Power@uk.fujitsu.com youngsoo.yuk@lge.com guan.yanfeng@zte.com.cn jsp@etri.re.kr |
| Re: | Comments on IEEE 802.16-09/D3 for IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | Proposed text on Group Configuration MAC Control Message for Group Resource Allocation (16.2.3.44) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text on Group Configuration MAC Control Message for Group Resource Allocation (16.2.3.44)

Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan

Intel Corporation

Hyunkyu Yu, Seho Kim

Samsung Electronics

Kevin Power

Fujitsu

Youngsoo Yuk, Jeongki Kim

LG Electronics

Yanfeng Guan, Xianming Chen, Huiying Fang

ZTE

Jisoo Park

ETRI

- Introduction

This contribution proposes text changes for the section 16.2.3.44 Group Configuration MAC Control Message (AAI_GRP-CFG) of the IEEE 802.16m draft D3 [1]. The current format of the message does not have any indication of whether the AMS has been added to (or deleted from) a DL GRA group or a UL GRA group. Since the same Flow ID may be used for both DL and UL, the Flow ID cannot distinguish between DL and UL. Hence this contribution proposes to add an additional 1-bit field in the message called DL/UL indicator.

- References

[1] Draft Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface IEEE 802.16m Draft, IEEE P802.16m/D3, December 2009.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

Text proposal for inclusion in the 802.16m draft D2

Adopt proposed text changes in section 16.2.3.44 in 802.16m draft D3 [1].

The proposed text is shown in underlined blue font and existing text in the draft is shown in black.

____________________________Begin proposed text with editorial markup____________________________

16.2.3.44 Group Configuration MAC Control Message (AAI_GRP-CFG)

Group Configuration MAC Control message is used for group management when using Group Resource Allocation. This message is used to inform an AMS that it has been added to or deleted from a GRA group. The format of the message is given in Table 722.

Table 722–Group Configuration MAC Control Message Format

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| MAC Control Message type | 8 |  |
| Deletion Flag | 1 | Flag to signal whether this message includes addition or deletion information. 0: Flow is added to a group 1: Flow is explicitly deleted from a group |
| Flow ID | 4 | Identifies the flow that is added to group for GRA |
| DL/UL Indicator | 1 | Signals whether the group corresponds to DL allocations or UL allocations. 0b0: DL allocation 0b1: UL allocation |
| If (Deletion Flag == 0) { |  |  |
| Group ID | 5 12 | ID of the group to which the flow is added |
| Long TTI Indicator | 1 | Defines number of AAI subframes spanned by the allocated resource. 0: 1 AAI subframe (default) 1: 4 DL/UL AAI subframes for FDD or all DL/UL AAI subframe for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, and DL/UL Indicator=0b1, then Long TTI Indicator = 0b1. |
| : |  |  |

____________________________End proposed text with editorial markup____________________________