| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ordering of BR opportunities within a BR-ACK A-MAP IE |  |
| Date Submitted | 2009-01-11 |  |
| Source(s) | Stavros Tzavidas Motorola Daniel Cohn Alvarion | stavros.tzavidas@motorola.com daniel.cohn@alvarion.com |
| Re: | LB30b |  |
| Abstract | This contribution proposes solutions for ordering BR opportunities within a BR ACK A-MAP IE.. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ordering of BR opportunities within a BR-ACK A-MAP IE

Stavros Tzavidas

Motorola

# Introduction

The current text implies that an AMS that has performed BW request considers it has received an implicit NACK after it sees a BR-ACK IE which does not include the code it used. This can be problematic in case the ABS wishes to split the acknowledgments into multiple BR ACK IEs.

# Text Proposal

Modify paragraph 16.3.6.5.2.4.12 “BR ACK A-MAP IE” in page 441, lines 38 to 44, as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

16.3.6.5.2.4.12 BR ACK A-MAP IE

The BR Acknowledgement A-MAP IE indicates the decoding status of the BR opportunities in the previous UL frame. All the successfully received preamble sequences, if present, shall be included in an ascending order. BR opportunities are encoded in ascending order based on the number of the uplink subframe in which they are contained. BR opportunities within the same uplink subframe are ordered based on increasing subchannel index in which they are contained. It In addition, the BR-ACK A-MAP IE also includes the allocation information for the fixed sized BR header. The UL resource and HFA shall be allocated to the preamble sequences whose grant indicator is ‘1’. The allocations shall be ordered based on the index of preamble sequences arranged in an ascending order. The maximum number of the HARQ retransmissions is TBD.

-------------------------------------------------- End of the text proposal --------------------------------------------------