| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for Multiplexing Several Protocols Over a Single FID of an AMS (11.13.18, 5.2.1, 5.2.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Aran Bergman, Muthaiah Venkatachalam Masoud Olfat, David McGinnis, Chunmei Liu | Aran.bergman@intel.com Intel, Clearwire |
| Re: |  |  |
| Abstract | This contribution proposes support for Multiplexing Several Protocols Over a Single FID of an AMS |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support for Multiplexing Several Protocols Over a Single FID of an AMS

# Introduction

In 16e, a WiMAX connection is shared between all packets that carry the same protocol and have same QoS requirements (and should use the same QoS parameters). That is, if a MS has to support IPv6, IPv4 and Ethernet packets, it would have to use 3 different connections, although all of them may require BE scheduling type.

In 16m, we would like a FID to be able to multiplex several protocols (e.g., RoHC, IPv4, IPv6, Ethernet) on the same flow. Different protocols on the FID may need to be treated with different levels of QoS.

# Proposed Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

11.13.18 CS-specific service flow encodings

11.13.18.1 CS Specification parameter

This parameter specifies the CS that the connection being set up shall use.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].28 | 1 | 0: GPCS (Generic Packet Convergence Sublayer) 1: Packet, IPv4 2: Packet, IPv6 3: Packet, IEEE 802.3/Etherneta 4: Reserved 5: Packet, IPv4 over IEEE 802.3/Etherneta 6: Packet, IPv6 over IEEE 802.3/Etherneta 7: Reserved 8: Reserved 9: ATM 10: Reserved 11: Reserved 12: Reserved 13: Reserved 14: Packet, IPb 15: Multiprotocol flow 1516–255 Reserved | DSA-REQ |

aClassifiers for IEEE 802.1Q VLAN tags may be applied to service flows of this CS type.

bSDUs for service flows of this CS type may carry either IPv4 or IPv6 in the header-compressed payload.

5.2.1 MAC SDU format

Once classified and associated with a specific MAC connection, higher layer PDUs shall be encapsulated in the MAC SDU format as illustrated in Figure 8. The 8-bit PHSI (payload header suppression index) field shall be present when a PHS rule has been defined for the associated connection. PHS is described in 5.2.3. The 8-bit Protocol ID field shall be present when a Multiprotocol flow is defined for the associated connection. This is described in section 5.2.6.

[Replace figure 8]

MAC SDU

Packet PDU

PHSI (optional)

Protocol ID (optional)

Figure 8 – MAC SDU format

[Add a new subclause]

5.2.6 Support for multiple protocols on the same flow

In order to transport several types of protocols, over the same MAC connection the Multiprotocol flow can be used. The receiver must identify the protocol to correctly forward the SDU. For instance – if the information carried by the SDU is a RoHC packet, it should be forwarded to the RoHC decompressor. The receiver does this according to the Protocol ID field which is the first byte of a Multiprotocol flow connection as depicted in Figures XX and XX.

Protocol ID

Packet

Figure XX – Multiprotocol flow PDU format without PHS

Protocol ID

Packet

PHSI

Figure XX – Multiprotocol flow PDU format with PHS

Once the protocol type of an incoming packet is determined, the appropriate classification rules are applied to the packet and the correct service flow is identified. It is then optionally forwarded to the header suppression mechanism (PHS or RoHC) and then mapped MAC SAP using the format described in this section. The Protocol ID content is set by the transmitter by the protocol identified before the classification rules were applied and according to the Table YY.

The method by which the protocol of a packet introduced to the CS layer is identified is beyond the scope of this standard.

Table YY – Possible values for Protocol ID field

| Protocol ID | Meaning |
| --- | --- |
| 1 | Raw IP |
| 2 | IPv4 |
| 3 | RoHC |
| 4 | IPv6 |
| 5 | ECRTP |
| 6..256 | Reserved |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]