| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Ranging ACK timing |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | LB30b |  |
| Abstract | This contribution discusses the timing of the ranging ACK message. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Ranging ACK timing

Stavros Tzavidas

Motorola

# Introduction

The current text implies that an AMS that has performed BW request considers it has received an implicit NACK after it sees a BR-ACK IE which does not include the code it used. This can be problematic in case the ABS wishes to split the acknowldegments into multiple BR ACK IEs.

# Text Proposal

Modify paragraph 16.2.15.3 as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

16.2.15.3.1 Contention-based initial ranging and automatic adjustments

An AMS that wishes to perform initial ranging shall take the following steps:

•The AMS, after acquiring downlink synchronization and uplink transmission parameters, shall select one Ranging Slot based on the random backoff. The random backoff shall use a binary truncated exponent algorithm. After selecting the Ranging Slot, the AMS shall choose a ranging sequence (from the Initial Ranging domain) using a uniform random process. The AMS shall send the selected ranging sequence to the ABS in the selected Ranging Slot.

•The ABS should respond with an AAI_RNG-ACK message in a predefined, subsequent DL AAI subframe as define in 16.2.3.19. The AAI_RNG-ACK may appear several frames after the ranging slot where the AMS performed ranging. The AAI_RNG-ACK message provides responses to all the successfully received and decoded IR requests in initial ranging slots in a previous UL AAI subframe. If the AMS finds in the RNG-ACK bitmap that no initial ranging sequence has been successfully decoded in the ranging slot selected by the AMS, or it does not find a response in the AAI_RNG-ACK message to its IR access attempt, the AMS considers its initial ranging request is failed and restarts the initial ranging procedure..

-------------------------------------------------- End of the text proposal --------------------------------------------------