| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Service flow parameter update for ARQ connection (Section 16.2.12) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics | yb.chang@samsung.com, anilag@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b / Topic: Service Flow/Convergence Sublayer Parameters (Section 16.2.12.8) |  |
| Abstract | This contribution proposes the service flow parameter update for ARQ connection |  |
| Purpose | To be discussed and adopted to P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Service flow parameter update for ARQ connection

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

Current ARQ parameters and timers are not applied in table 748. It should be updated for ARQ operation.

1. Reference section should be correct.

2. ARQ_RETRY_TIMEOUT is not used in 16m ARQ operation.

3. ARQ_DELIVER_IN_ORDER is not used in 16m ARQ operation. ARQ in-order delivery is mandatory to prevent redundant TCP retransmission.

4. ARQ_BLOCK_SIZE is not necessary, ARQ block is variable size.

5. Some timers are introduced in 16m ARQ section, these timers should be included.

6. FSN is not used in 16m Header.

# Proposed text

[Modify the parameters in the table 748 as follows:]

| Fields | Size(bits) | Description |
| --- | --- | --- |
| … |  |  |
| ARQ Enable | 1 | Refer 11.13.187.1 |
| ARQ_WINDOW_SIZE | 16 | Refer 11.13.187.2 |
| ARQ_RETRY_TIMEOUT – Transmitter Delay | 16 | Refer 11.13.18.3 |
| ARQ_RETRY_TIMEOUT - Receiver Delay | 16 | Refer 11.13.18.3 |
| ARQ_BLOCK_LIFETIME | 16 | Refer 11.13.187.4 |
| ARQ_SYNC_LOSS_TIMEOUT | 16 | Refer 11.13.187.5 |
| ARQ_DELIVER_IN_ORDER | 1 | Refer 11.13.18.6 |
| ARQ_RX_PURGE_TIMEOUT | 16 | Refer 11.13.187.7 |
| ARQ_BLOCK_SIZE | 8 | Refer 11.13.18.8 |
| RECEIVER_ARQ_ACK_PROCESSING TIME | 8 | Refer 11.13.187.9 |
| ARQ_SUB_BLOCK_SIZE | 8 |  |
| ARQ_ERROR_DETECTION_TIMEOUT | 16 |  |
| ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | 16 |  |
| CS Specification | variable | Refer to 11.13.19 |
| Unsolicited Grant Interval | 6 | Refer to 11.13.20 |
| Unsolicited Polling Interval | 6 | Refer to 11.13.21 |
| FSN size | 1 | Refer to 11.13.22 |
| … |  |  |