| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ feedback IE modification to avoid ARQ feedback delay (Section 16.2.13) |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics Aran Bergman Intel Corporation | yb.chang@samsung.com, anilag@samsung.com aran.bergman@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b / Topic: ARQ (Section 16.2.13.1.1) |  |
| Abstract | This contribution proposes the ARQ feedback IE modification and text proposal of ARQ feedback processing. |  |
| Purpose | To be discussed and adopted to P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ feedback IE modification to avoid ARQ feedback delay

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

Aran Bergman

Intel Corporation

# Problem Description

In the ARQ operation, Error detection is possible after ARQ_ERROR_DETECTION_TIMEOUT. Due to the ARQ_ERROR_DETECTION_TIMEOUT, ARQ feedback delay happen. The below is the example:

1. Scenario

When transmitter sends ARQ block #1, this ARQ block can be failed during all HARQ retransmission. However, if other ARQ blocks(#2,3) is received correctly and ARQ feedback polling is requested by transmitter side, receiver can not send ARQ feedback due to the delay of ARQ_ERROR_DETECTION_TIMER of ARQ block #1.

2. ARQ ACK delaying

- Normally, transmitter polls ARQ feedback to flush the ARQ buffer. If receiver can not send ARQ feedback after receiving ARQ feedback polling from transmitter side, which makes delays freeing of memory occupied by the ARQ blocks in the transmitter.

- Delaying of ARQ ACK directly affects average ARQ throughput. Longer ARQ RTT reduces average ARQ Throughput.

2. Redundant retransmission of ARQ feedback Polling

- After transmitting the ARQ feedback poll, if ARQ feedback is not received before the ARQ FEEDBACK POLL RETRY TIMEOUT expires, the transmitter may retry the ‘ARQ feedback poll’

# Proposed Remedy

In this contribution, we propose the indication bit in ARQ feedback IE which represents that receiver is waiting for ARQ_ERROR_DETECTION_TIMEOUT expiry of a specific ARQ block. Using this bit, transmitter does not initiate ARQ retransmission even if ARQ block reported as a NACK in ARQ feedback IE.

# Proposed text

[Modify the table 748 as follows:]

Table 749—ARQ feedback IE format for ARQ block

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST) { | variable |  |
| Last | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| Flow ID | 4 | The ID of the flow being referenced |
| FLAG1 |  | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG1 = 0, ARQ block up to and including SN has been received successfully. FLAG1 = 1, ARQ block less than SN has been received successfully. |
| if(FLAG1==1){ |  |  |
| NSI | 1 | NACK Suspended Indicator 0 = Bit marked ‘0’ in the following Selective ACK MAP represents a ARQ block NACK 1 = Bit marked ‘0’ in the following Selective ACK MAP represents that ARQ block NACK decision is suspended (i.e. ARQ_ERROR_DETECTION_TIMER is running for corresponding ARQ block). |
| EXT | 1 | Extension for ARQ sub-block indication 0= No ARQ sub-block ACK/NACK indication 1= ARQ sub-block ACK/NACK indication follows |
| Selective ACK MAP1 | 65 |  |
| EXT | 1 | Extension for ARQ sub-block indication 0= No ARQ sub-block ACK/NACK indication 1= ARQ sub-block ACK/NACK indication follows |
| FLAG1 | 1 | 0 = No more selective ACK MAP1 and FLAG1 1 = Another set of selective ACK MAP1 and FLAG1 follows |
| while(FLAG1==1){ |  |  |
| Selective ACK MAP1 | 7 | Each bit represents ACK or NAK of corresponding ARQ block. '0' is NAK and '1' is ACK. Selective ACK MAP1 represents ACK or NAK information of ARQ blocks after first selective ACK MAP1. |
| FLAG1 | 1 | 0 = No more selective ACK MAP1 and FLAG11 = Another set of selective ACK MAP1 and FLAG1 follows |
| } |  |  |
| if(EXT==1){ |  |  |
| SEM | variable | Sub-block-Existence Map The numbers of SEM bits are same as NACKed ARQ blocks in the Selective ACK MAP1.Each bit represents the existence of partially nacked ARQ blocks or completely nacked ARQ blocks. 1 = Partially nacked ARQ block (the ARQ block has some sub-blocks which have received correctly) 0 = Completely nacked ARQ blocks( the ARQ block has no sub-blocks which have received correctly) |
| For(i=0;i<Num_SN<i++){ |  | Num_SN is the number of bits equal to 1 in the SEM field |
| do{ |  | If NSI bit set to ‘0’, aAll ARQ sub-blocks that are not listed the Start_SSN and Num_SSN in ‘do loop’ are considered as NACK. If NSI bit set to ‘1’, All ARQ sub-blocks that are not listed the Start_SSN and Num_SSN in ‘do loop’ are considered that ARQ sub-block NACK decision is suspended. |
| START_SSN | 11 | Start of ARQ sub-block SN which was received correctly. |
| Num_SSN | 11 | Number of consecutive ARQ sub-blocks which were received correctly from START_SSN onwards |
| End | 1 | 0 = One more set of START_SSN, Length and End fields 1 = There are no more START_SSN, NUM_SSN and End fields |
| }while(End==0) |  |  |
| } |  |  |
| Padding | variable | For Byte alignment |
| } |  |  |
| } |  |  |
| } |  |  |

[Modify the text from line 59-65 on page 225 as follows:]

If the SN is valid and the ARQ feedback IE includes an selective ACK MAP, it shall consider all ARQ blocks in the interval ARQ_TX_WINDOW_START to SN (not inclusive) as positively acknowledged, and set the ARQ_TX_WINDOW_START to SN. The selective ACK MAP is processed from left to right (MSB to LSB) and the left most bit indicates SN. Each bit in the selective ACK MAP that is set to '1'indicates that the corresponding ARQ block has been received correctly. Each bit that is set to "0" in the selective ACK MAP indicates that the corresponding ARQ block has been received incorrectly. If ‘NSI’ is set to ‘1’, all bits set to “0” in the selective ACK MAP, transmitter ignore all bits set to “0” in the selective ACK MAP and transmitter only considers bits set to “1” in the selective ACK MAP. If ARQ feedback IE includes selective ACK MAP and “EXT” field set to 1, the transmitter shall process ARQ feedback information of ARQ sub-blocks. “Num_SSN” represents the number of ARQ sub-blocks which has been received correctly from “START_SSN” onwards. If ‘NSI’ is set to ‘1’, transmitter only considers correctly received ARQ sub-blocks.