| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ state machine modification for ARQ sub-block operation (Section 16.2.13) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics | yb.chang@samsung.com, anilag@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b / Topic: ARQ (Section 16.2.13.5.2.1) |  |
| Abstract | This contribution proposes the ARQ state machine change for clarifying ARQ sub-block operation |  |
| Purpose | To be discussed and adopted to P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ state machine modification for ARQ sub-block operation

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

# Problem description

In the current spec [1], ARQ state machine in a transmitter is defined in figure 427:

- Current ARQ state machine is operated based on ARQ block.

- In ARQ retransmission with re-arrangement, ARQ sub-block can be generated. After generating ARQ sub-blocks, each ARQ sub-block state from the same ARQ block can be different.

- If ARQ sub-block is retransmitted, it can be “outstanding”. If ARQ sub-block received as a NACK can be “waiting for retransmission”. If ARQ sub-block received as a ACK can be “done” state.

- To reflect various states of ARQ sub-blocks, ARQ state machine should be modified.

# Proposed remedy

ARQ state machine for ARQ sub-block operation can be defined as separate ARQ state in the ARQ state machine. In this contribution, it is called “Rearrangement” state. In the “Rearrangement” state, ARQ block is fragmented as ARQ sub-blocks and all ARQ retransmission and waiting for ARQ feedback is performed in this state. We propose the following ARQ state machine modification:

# Proposed text

[Modify the text from line 40-41 on page 224 as follows:]

16.2.13.5.2.1 Transmitter state machine

Each ARQ enabled connection shall have an independent ARQ state machine. The ARQ state transitions shall be ARQ block based.(i.e., not based on the ARQ sub-block).

An ARQ block may be in one of the following five six states-not-sent, outstanding, waiting-for-retransmission, rearrangement, discard, and done state.

[Replace figure 427 page 122 as following figure:]

[Modify the text from line 17-21 on page 225 as follows:]

While an ARQ block is in "waiting-for-retransmission" state, transmitter prepares for ARQ block retransmission.

If ARQ block is re-transmitted without rearrangement, the ARQ block state transits to "outstanding" state. If ARQ block is rearranged for retransmission, the ARQ block state transit to “rearrangement” state. If a positive acknowledgement (ACK) arrives, the ARQ block state transits to the "done" state. If the ARQ_BLOCK_LIFETIME period expires, the ARQ block state transits to "discard" state.

While an ARQ block is in "rearrangement" state, transmitter prepares for ARQ block retransmission with rearrangement. ARQ block is fragmented into ARQ sub-blocks and a sub-block SN(SSN) is sequentially assigned. These ARQ sub-blocks and SSN are maintained in “rearrangement” state. The transmitter re-transmits ARQ sub-blocks and wait for ARQ feedback of corresponding ARQ sub-blocks. If positive acknowledgement (ACK) arrives regarding ARQ sub-block, the transmitter can flush the ARQ sub-block and waits for positive acknowledgement regarding the remaining ARQ sub-blocks of ARQ block having a same SN. If ARQ sub-block is negatively acknowledged (NACK or Local NACK), ARQ sub-block is retransmitted. When ARQ block is staying in the “rearrangement” state, state transition is performed based on ARQ block.

If all positive acknowledgements arrive regarding all ARQ sub-blocks of a single ARQ block, the corresponding ARQ block state transits to the "done" state. If the ARQ_BLOCK_LIFETIME period of ARQ block expires, the corresponding ARQ block state transits to "discard" state.

[Modify the text from line 31-34 on page 225 as follows:]

For a given connection the transmitter shall first handle (transmit or discard) blocks in "waiting-for-retransmission" state and “rearrangement” state and only then blocks in "non-sent" state. Blocks in "outstanding" or "discarded" state shall not be transmitted. When ARQ blocks or sub-blocks are retransmitted, the ARQ block or sub-block associated with the lowest BSN or SSN shall be retransmitted first.

# Reference

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”