| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Radio Calibration Allocation |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Hassan Yaghoobi, JK Fwu, Dov Andelman Yi Hsuan, Shantidev Mohanty Intel Corporation [Address Line 1 (optional)] [Address Line 2 (optional)] | Voice: [Telephone Number (optional)]] E-mail: hassan.yaghoobi@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Letter Ballot 30b on P802.16m/D3 |  |
| Abstract | In order to optimize the operation of radio devices, 802.16m standard needs to specify Rx and Tx calibration allocations. Currently, there is no provision of such calibration allocation. The purpose of this comment is to propose required modifications to the draft specification in this regards. |  |
| Purpose | Adoption for inclusion in P802.16m/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Radio Calibration Allocation

Hassan Yaghoobi, JK Fwu, Dov Andelman, Yi Hsuan, Shantidev Mohanty

Intel Corporation

# Introduction

# Without provisioning auto calibration (on line calibration), excessive time will be required for factory calibration (production line calibration) of 802.16m compliant devices. This adds substantial cost and limits the volume that can be produced on a test line.

# Problem Statement

IEEE 802.16m D2 standard draft has not specified any provisions to enable auto calibration. Auto calibration is required during the startup and also periodically when parameters such as frequency change (Inter-FA HO) and temperature are changing. Standard needs to provide specifications in form of proper allocations to MS for Rx and Tx calibration of the Radios.

Rx and Tx calibration require start up and periodic allocations in DL and UL that meets specific requirements.

For Rx calibration, the following requirements are considered for the development of standard specification.

- A quiet DL Calibration Allocation (DCA) of at least 18 adjacent subcarriers over one or two OFDM symbols.

- In the case of FDD, there will be no UL allocation for calibrating MSs during the OFDM symbol time containing the DCA in DL.

For Tx calibration, the following requirements are considered for the development of standard specification.

- A quiet UL Calibration Allocation (UCA) of at least 18 adjacent subcarriers over one or two OFDM symbols. During the first OFDM symbol the MS may transmit some narrow band calibration signal that BS has to ignore.

- In the case of FDD, there will be no DL allocation for calibrating MSs during the OFDM symbol time containing the UCA in UL.

# Proposed Remedy

<< ---------------------------------------------- Text proposal 1, pp. 593, line 17 ------------------------------------------------  >>

<< Add the following new section>>

16.3.13 AMS Auto Calibration

Uplink Calibration Allocation (UCA) units can be allocated periodically with period of 2 sec to facilitate auto Tx-calibration. The UCAs are shared by multiple AMSs to transmit calibration signal within the allocated CAUs in order to enable auto-Tx-calibration.  The physical structure of UCA is the same as sounding PHY structure (18 subcarriers by 1 OFDMA symbol). The calibration signal is an OFDMA signal with maximum power the same as maximum transmit power for the normal 802.16m operation. Additional details of calibration signal are vendor specific.

To enable auto Rx-calibration, ABSs can allocate some periodic small Downlink Calibration Allocation (DCA) burst using NLRU or SLRU as quiet period for the AMSs to perform auto Rx-calibration. This allocation is a quiet allocation with no downlink transmission and may be assigned with a pre set period of 2 sec. The allocation is signaled through broadcast assignment A-MAP IE.

<< ---------------------------------------------- Text proposal 2, pp. 443, Table 829 ------------------------------------------------  >>

16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE

<< Modify Table 829 as follows>>

Table 829 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast-A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast A-MAP IE |
| Broadcast mode | 1 | 0: broadcast messages included 1: AMS Rx calibration allocation |
| If (Broadcast mode == 0b00){ |  |  |
| MCS (TBD) |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Reserved | TBD | Reserved bits |
| } else { |  |  |
| Resource Index | 11 | Quiet resource allocation for AMS Rx calibration |
| Period | TBD | Periodicity of the quiet resource allocation |
| } |  |  |
| } |  |  |

<< ---------------------------------------------- Text proposal 3, pp. 428, Table 822 ------------------------------------------------  >>

<< Modify Table 822 as follows>>

Table 822– UL Sounding Command A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| UL Sounding Command IE format () { |  |  |
| A-MAP IE Type | 4 |  |
| Sounding/Calibration Mode Switch | 1 | 0: sounding allocation 1: calibration allocation |
| Sounding subframe | 32 | Indicates the sounding subframe |
| If (Sounding/Calibration Mode Switch ==0){ |  |  |
| Sounding subband bitmap | variable max. 1224 | FFT size dependant |
| If (Multiplexing type == 0) { |  |  |
| Decimation offset d | 5 | Unique decimation offset |
| } else { |  |  |
| Cyclic time shift m | 5 | Unique cyclic shift |
| } |  |  |
| Periodicity (p) | 3 | 0b000 = Single command, not periodic, or terminate the periodicity. Otherwise, repeat sounding once per 2(p-1) frames, where p is decimal value of the periodicity field |
| Antenna switching | 1 | 0b0: Antenna switching 0b1: No antenna switching |
| If (Antenna switching == 1) { |  |  |
| Transmit antenna bitmap | 4 | Indicates an active set of transmit antennas |
| } |  |  |
| Padding | variable | Padding |
| } |  |  |
| } |  |  |

# References

N/A