| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Paging Operation in Multi-carrier Operation in 802.16m (16.2.8.2.10.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Muthaiah Venkatachalam, Etemad Kamran, Shantidev Mohanty Intel Corporation Masoud Olfat Clearwire | E-mail: shantidev.mohanty@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.8.2.10.2” |  |
| Abstract | This contribution proposes amendment text describing paging operation in multi-carrier system for ABS Type in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Paging Operation in Multi-carrier Operation

- in 802.16m (16.2.8.2.10.2)

Muthaiah Venkatachalam, Etemad Kamran, Shantidev Mohanty

- Intel Corporation

Masoud Olfat

Clearwire

- Introduction

This contribution proposes amendment text describing the paging operation when a 16m ABS supports multiple carriers.

.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D3

========================== Start of Proposed Text ==========================

[Note to Editor: Add the following text after line 36 in Page 195 in “16.2.8.2.10.2 Idle mode”]

- 16.2.8.2.10.2 Idle mode

In a multi-carrier system the PGID-Info message is transmitted in all the carriers. The AAI_PAG-ADV message for an AMS shall be transmitted in only one of the carriers. The AAI_PAG-ADV message is transmitted in one or more of the frames starting from the second sub-frame in the super-frame.

The carriers are indexed in the increasing order of their center frequency, i.e., the carrier with lowest center frequency has the smallest index and the carrier with highest center frequency has the largest index. The value of HMC is the number of carriers in a BS that are used for transmitting paging message for an idle mode MS. This can be all the carriers or a subset of carriers used by a multi-carrier BS.

The contents of PAG-Info message are shown in Table xxx.

| Field | Size | Notes |
| --- | --- | --- |
| Paging carrier indicator | 1 | Indicates if the carrier transmitting this MC-PAG-Info message is a carrier that transmits paging message for idle mode MSs or not: 0: this carrier does not transmit paging message for idle mode MS. 1: this carrier transmits paging message for idle mode MSs. |
| If (Paging carrier indicator ==1) { |  |  |
| Number of carriers used for paging message transmission (HMC) | N | Either all the carriers or a subset of carriers of a BS. N = roof (log2(HMCmax)) where HMCmax is the maximum number of carriers in a BS |
| } |  |  |

========================== End of Proposed Text ===========================