| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for LBS Capabilities in AAI_REG-RSP message(16.2.3) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Beijing Samsung Telecom R&D Center Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | P802.16m/D3 |  |
| Abstract | The contribution proposes to clean up LBS capabilities on “TBD” item in AAI_REG-RSP message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for LBS Capabilities in AAI_REG-RSP message

Lei Zhou, Xufeng Zheng,

Beijing Samsung Telecom R&D Center

Sangheon Kim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

Negotiation capabilities on LBS have been included in 802.16m/D3. So "TBD" value in AAI_REG-RSP is unnecessary. We need remove LBS capability on “TBD” item in AAI_REG-RSP.

- The Proposed Text in AWD

-----------------------------------------  Proposed Text Start  -------------------------------------------------

[adopt the following text modification started from line#1 Page 58 in P802.16m/D3]

| - Parameters for AAI_ REG-RSP message |  |  |  |
| --- | --- | --- | --- |
| Name |  |  | Value |
| MS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values; default =2^10 |
|  |  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | the maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEOUT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | Capabilities for interference mitigation support | DL PMI coordination capability | 0: ABS is DL PMI coordination capable 1: ABS is not DL PMI coordination capable |
|  |  | DL collaborative multi-BS MIMO capability | 0: ABS is DL collaborative multi-BS MIMO capable 1: ABS is not DL collaborative multi-BS MIMO capable |
|  |  | DL closed-loop multi-BS macro diversity capability | 0: ABS is DL closed-loop multi-BS macro diversity capable 1: ABS is not DL closed-loop multi-BS macro diversity capable |
|  |  | UL PMI combination capability | 0: ABS is UL PMI combination capable 1: ABS is not UL PMI combination capable |
|  | LBS Capabilities | Capability for support A-GPS method | 0b0: no support 0b1: support |
|  |  | TBD | TBD |
|  | Multicarrier capabilities |  | 0b00: No MC modes 0b01: Multicarrier aggregation 0b10: Multicarrier switching 0b11: Both multicarrier aggregation and switching |
| Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  | indicates which classification/PHS options and SDU encapsulation the AMS supports |
|  | Maximum number of classification rules |  | Maximum number of admitted classification rules that the AMS supports. |
|  | ROHC support |  | Indicates the level of ROHC support. |
|  | PHS support |  | Indicates the level of PHS support. |
|  | Resource_Retain_Time |  | Number of frames that the serving ABS shall retain the AMS resource. |
|  | IP-Service-Type IE |  | Indicates which IP service will be used, the value is one of following: IPv4-Service IPv6-Service IPv4/IPv6-Dual-Mode-Service |
| Host configuration capabilities and its parameters | IPv4-Host-Address IE |  | The allocated IPv4 Host Address for the AMS. Size: 4 octet |
|  | IPv6-Home-Network-Prefix IE |  | The allocated IPv6 Home Network Prefix for the AMS. Size: 8 octet |
|  | Additional-Host-Configurations IE |  | Includes additional host configuration options in DHCP Options format as detailed in [3], [4] |

----------------------------------------- Proposed Text End  ------------------------------------